| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G704 |
| --- | --- |

| Title: | Last position coding for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this proposal,  the CABAC coding of  last significant coefficient position is modified. A progressive codeword structure with reduced number of bins and shorter truncated unary codes is proposed. Additionally, the number of context models for thelast position is reduced by two. Experimental results reportedly show a BD-rate of -0.04%, -0.01% and -0.03% in high efficiency intra-only, random access and low-delay test conditions, respectively.

# Introduction

In the last meeting, [1] was adopted to HM4.0. The proposal reduces the contexts used in last position coding for CABAC by introducing fixed length codes with bypass mode. Bypass mode means that there is no context modelling procedure and every symbol is coded with equal probability state. Therefore, increasing the number of bins coded in bypass mode while reducins the bins in regular mode helps the speed-up and parallelization of the codec.

In [1], the maximum possible magnitude of last position component, max_length, is divided equally in two halves. The first half is coded with a truncated unary code and the second half is coded with fixed length codes (the number of bins being equal to log2(max_length/2)). In the worst case scenario, the number of the bins that use context modeling is equal to max_length/2. Table 1 shows the binarization for TU 32x32 in HM4.0.

| Magnitude of last position component | Truncated unary (context model) | Fixed binary (bypass) |
| --- | --- | --- |
| 0 | 1 | - |
| 1 | 01 | - |
| 2 | 001 | - |
| 3 | 0001 | - |
| 4 | 00001 | - |
| 5 | 000001 | - |
| 6 | 0000001 | - |
| 7 | 00000001 | - |
| 8 | 000000001 | - |
| 9 | 0000000001 | - |
| 10 | 00000000001 | - |
| 11 | 000000000001 | - |
| 12 | 0000000000001 | - |
| 13 | 00000000000001 | - |
| 14 | 000000000000001 | - |
| 15 | 0000000000000001 | - |
| 16-31 | 0000000000000000 | XXXX |

Table 1. Binarization for TU 32x32 in HM4.0, where X means 1 or 0.

# Proposed Solution

A  progressive last position coding scheme is proposed for all TU sizes. Compared to HM4.0, the proposed coding scheme uses less bins in the worset case scenario, shorter truncated unary codes and less contexts. The codeword in the proposed scheme still starts with a truncated unary code and followed by a fixed length code. Each magnitude of last position uses the same binerization for all TU sizes, except when the last position equals to TU size minus 1. The exception is due to the truncated unary code.

Assume that T is the TU size and the number of zeros in truncated unary code is N={0,…,2log2(T)-1}. The following b bits fixed length binary code  has a value, fvalue=0,...,2b−1 , where b= max(0,  N/2-1).  The magnitude of last position, lastpos, can be derived from N and fvalue as:

lastpos=N,if N<22N2−1×(2+mod(N,2))+fvalue,if N≥2

where mod(.) represents modular operation andfvalue represents the value of the fixed length code.

Table 2 shows the binarization for TU 32x32.

| Magnitude of last position component | Truncated unary (context model) | Fixed binary (bypass) | fvalue |
| --- | --- | --- | --- |
| 0 | 1 | - | 0 |
| 1 | 01 | - | 0 |
| 2 | 001 | - | 0 |
| 3 | 0001 | - | 0 |
| 4-5 | 00001 | X | 0-1 |
| 6-7 | 000001 | X | 0-1 |
| 8-11 | 0000001 | XX | 0-3 |
| 12-15 | 00000001 | XX | 0-3 |
| 16-23 | 000000001 | XXX | 0-7 |
| 24-31 | 000000000 | XXX | 0-7 |

Table 2. Proposed binarization for TU 32x32, where X means 1 or 0.

|  | HM4.0 | Proposed solution |
| --- | --- | --- |
| TU 4x4 | 3 | 3 |
| TU 8x8 | 4 | 5 |
| TU 16x16 | 8 | 7 |
| TU 32x32 | 16 | 9 |
| Total (Luma) | 31 | 24 |
| Total (Chroma) | 15 | 15 |

Table 3. Maximum length of the truncated unary code.

|  | HM4.0 | Proposed solution |
| --- | --- | --- |
| TU 4x4 | 3 | 3 |
| TU 8x8 | 6 | 6 |
| TU 16x16 | 11 | 9 |
| TU 32x32 | 20 | 12 |

Table 3. Maximum length of the bins on one last position component.

Table 3 and 4 show the comparison of the proposed scheme and HM4.0. The unary code can be as long as 16 bins in HM4.0 while it is up to 9 bins in the proposed scheme. In terms of number of the bins, it is also reduced from 40 to 24 in the worst case, i.e., when last position is located at the end of a 32x32 TU.

Since binarization process is changed, context modelling of the last position coding is adjusted for the truncated unary codes. Table 4 shows the context indexing for each bin in different TU sizes. Compared to [1], two contexts are reduced in proposed solution.

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 4 | 5 | 5 | 6 |  |  |  |  |
| TU 16x16 | 7 | 8 | 9 | 9 | 10 | 10 | 11 |  |  |
| TU 32x32 | 12 | 13 | 14 | 14 | 15 | 15 | 16 | 16 | 17 |

# Simulation Results

This method is implemented in HM4.0. Simulation results are presented for the high efficiency common test conditions [2]. Table 5 shows the results for this method. The BD-rate is 0.04℅, 0.01℅ and 0.03℅ for AI, RA and LB configurations, respectively.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.03% | 0.01% | 0.02% | 0.03% | 0.05% | -0.07% |  |  |  |
| Class B | -0.05% | -0.03% | -0.02% | -0.05% | 0.04% | -0.21% | -0.04% | -0.16% | -0.37% |
| Class C | -0.02% | -0.01% | -0.02% | -0.01% | -0.08% | -0.03% | 0.02% | -0.10% | -0.24% |
| Class D | -0.03% | -0.01% | -0.06% | 0.00% | 0.01% | -0.12% | 0.06% | 0.57% | -0.08% |
| Class E | -0.08% | -0.04% | -0.01% |  |  |  | -0.21% | -0.65% | 0.66% |
| All | -0.04% | -0.02% | -0.02% | -0.01% | 0.01% | -0.11% | -0.03% | -0.05% | -0.07% |
| Enc T[%] | 99% |  |  | 100% |  |  | 99% |  |  |
| Dec T[%] | 100% |  |  | 100% |  |  | 99% |  |  |

# Conclusions

Comparing to HM4.0, maximum number of bins in last position coding is reduced from 40 to 24. The max number of bins require context modeling reduced from 16 to 9. Also, total context model number is reduced by 2 while achieving a slight performance gain. Based on the results, we suggest including this modification into HM.

# References

[1] V. Seregin, I.-K Kim , “Binarisation modification for last position coding”, JCTVC-F375, 6th JCT-VC Meeting, Torino, IT, July, 2011.

[2] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900.

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).