| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G705 |
| --- | --- |

| Title: | CABAC simplification for explicit signaling mode of AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes to remove the CABAC context models for the motion vector predictor index, mvp_idx_lX (mvp_idx,l0, mvp_idx_l1, mvp_idx_lc), and use CABAC bypass mode instead. It is asserted that this change reduces the number of CABAC contexts in memory and eliminates the associated CABAC context update step while having negligible impact on compression efficiency. For HM-4.0, high efficiency common test conditions, the proposed change shows an average BD bitrate impact of –

(Without Class F sequences)

RA_HE Y:0.0% U:0.0% V:0.0%; LB_HE Y:0.0% U:0.1% V:0.1%; LP_HE Y:0.0% U:0.2% V:-0.1%.

(With Class F sequences)

RA_HE Y:0.0% U:0.0% V:0.0%; LB_HE Y:0.0% U:0.1% V:0.1%; LP_HE Y:0.0% U:0.1% V:-0.1%.

# Introduction

In the current HM, CABAC codes syntax elements using either the regular coding engine or the bypass coding engine. The regular coding engine maintains a context in memory and is updated for every bin coded. The bypass coding engine on the other hand assumes an approximately equal probability distribution when coding bins and does not need to maintain a context in memory or perform updates.

# Proposed Change

In HM-4.0 the motion vector predictor index, syntax element mvp_idx_lX (mvp_idx,l0, mvp_idx_l1, mvp_idx_lc), is used to identify the motion vector selected for the current inter prediction unit from a list of candidate motion vector predictors when the adaptive motion vector prediction (AMVP) mode is set to 1. At present, mvp_idx_lX is coded using context models with CABAC. This requires a probability update step as part of the bin decoding process.

It is proposed that the motion vector predictor index mvp_idx_lX use the CABAC bypass coding engine instead.  The proposed change results in negligible impact on compression efficiency while reducing complexity.

# Results

The proposed change was implemented in HM 4.0 [1]. All sequences (including class F) were then coded using the configuration files described in JCTVC-F900 [2]. Full results are available in the attached XLS sheets. The rate-distortion summary is as follows:

## Without Class F sequences

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.5% | -0.1% |
| Class E | 0.1% | 0.0% | 0.9% |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.2% | -0.5% |
| Class E | 0.0% | 0.4% | 0.0% |
| Overall | 0.0% | 0.2% | -0.1% |
|  | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

## With Class F Sequences

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.5% | -0.1% |
| Class E | 0.1% | 0.0% | 0.9% |
| Class F | -0.1% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.2% | -0.5% |
| Class E | 0.0% | 0.4% | 0.0% |
| Class F | -0.2% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

We assert that this is a negligible impact on compression performance.

# Conclusion

This document proposed the use of bypass coding engine for the syntax element mvp_idx_lX. The performance impact was measured by implementing the change in HM-4.0 and was found to be negligible.  It is proposed to adopt the technique into the HM.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] High efficiency test model (HM) version 4.0 URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

[2] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-F900/m21452, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Proposed Working Draft Text Modifications

Changes are highlighted in yellow.

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_type | Error: Reference source not found | 0 | 1..4 | 5..9 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 | Table 9-3 |  | 0..1 | 2..3 |
| transform_tree() | no_residual_data_flag | Error: Reference source not found |  | 0 | 1 |
|  | split_transform_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_cr | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
| residual_coding() | last_significant_coeff_x, last_significant_coeff_y | Error: Reference source not found | 0..30 | 31..61 | 62..92 |
|  | significant_coeff_flag |  | Error: Reference source not found | Error: Reference source not found | Error: Reference source not found |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..79 | 80..159 | 160..239 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..79 | 80..159 | 160..239 |

| Table 9-2 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| … | … | … | … | … | … |
| abs_mvd_minus2[ ] | P/B | EG1 | na | na | na, (uses Decode Bypass) |
| mvd_sign_flag[ ] | P/B | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| mvp_idx_lc | B | TU, cMax = 2 | 1 | Table 9-3 | 2 |
| mvp_idx_l0 | P | TU, cMax = 2 | 1 | Table 9-3 | 0 |
|  | B |  | 1 | Table 9-3 | 2 |
| mvp_idx_l1 | B | TU, cMax = 2 | 1 | Table 9-3 | 2 |
| mvp_idx_lc | B | TU, cMax = 2 | na | na | na, (uses Decode Bypass) |
| mvp_idx_l0 | P | TU, cMax = 2 | na | na | na, (uses Decode Bypass) |
|  | B |  |  |  |  |
| mvp_idx_l1 | B | TU, cMax = 2 | na | na | na, (uses Decode Bypass) |
| no_residual_data_flag | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 1 |
| split_transform_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 4 |
|  | B |  | 0 | Error: Reference source not found | 8 |
| cbf_luma | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 2 |
|  | B |  | 0 | Error: Reference source not found | 4 |
| cbf_cb | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 4 |
|  | B |  | 0 | Error: Reference source not found | 8 |
| cbf_cr | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 4 |
|  | B |  | 0 | Error: Reference source not found | 8 |
| last_significant_coeff_x, last_significant_coeff_y | I | prefix and suffix as specified in subclause Error: Reference source not found | 30 | Error: Reference source not found | 0 |
|  | P |  | 30 | Error: Reference source not found | 31 |
|  | B |  | 30 | Error: Reference source not found | 62 |
| significant_coeff_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 0 |
|  | B |  | 0 | Error: Reference source not found | 0 |
| coeff_abs_level_greater1_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 60 |
|  | B |  | 0 | Error: Reference source not found | 120 |
| coeff_abs_level_greater2_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 60 |
|  | B |  | 0 | Error: Reference source not found | 120 |
| coeff_abs_level_minus3 | all | prefix and suffix as specified in subclause Error: Reference source not found | prefix: na suffix: na | prefix: na suffix: na | prefix: na, (uses Decode Bypass) suffix: na, (uses Decode Bypass) |
| coeff_sign_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |

Table 9-3 – Values of variable m and n for mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 ctxIdx

| Initialisation variables | mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| m | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 |