| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G707 |
| --- | --- |

| Title: | Using CABAC bypass mode for coding intra prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes the use of CABAC bypass mode for coding the intra prediction mode syntax elements mpm_idx and rem_intra_luma_pred_mode. This change reduces the number of CABAC contexts in memory and eliminates the associated CABAC context update.  It is asserted that the change results in negligible impact on compression efficiency. For HM-4.0, high efficiency common test conditions, the proposed change shows an average BD bitrate impact of –

(Without Class F sequences)

AI_HE Y:0.0% U:0.0% V:0.0%; RA_HE Y:0.0% U:0.0% V:0.0%; LB_HE Y:0.0% U:0.0% V:-0.5% LP_HE Y:0.0% U:-0.2% V:-0.3%

(With Class F sequences)

AI_HE Y:0.0% U:0.0% V:0.0%; RA_HE Y:0.0% U:0.0% V:0.0%; LB_HE Y:0.0% U:-0.1% V:-0.5% LP_HE Y:0.1% U:-0.2% V:-0.3%

# Introduction

In the current HM, CABAC codes syntax elements using either the regular coding engine or the bypass coding engine. The regular coding engine maintains a context in memory and is updated for every bin coded. The bypass coding engine on the other hand assumes an approximately equal probability distribution when coding bins and does not need to maintain a context in memory or perform updates.

# Proposed Change

In HM-4.0 the intra prediction mode syntax elements mpm_idx and rem_intra_luma_pred_mode, is used to identify the intra prediction mode associated with a luma block. At present, mpm_idx and rem_intra_luma_pred_mode are coded using context models with CABAC. This requires a probability update step as part of the bin decoding process.

It is proposed that the syntax elements mpm_idx and rem_intra_pred_mode use the CABAC bypass coding engine instead.  The proposed change results in negligible impact on compression efficiency while reducing complexity.

# Results

The proposed change was implemented in HM 4.0 [1]. All sequences (including class F) were then coded using the configuration files described in JCTVC-F900 [2]. Full results are available in the attached XLS sheets. The rate-distortion summary is as follows:

## CABAC bypass mode used for mpm_idx only

| WITHOUT CLASS F SEQUENCES | WITH CLASS F SEQUENCES |
| --- | --- |
| All Intra HE. Y; U; V. Class A; 0.0%; -0.1%; -0.1%. Class B; 0.0%; 0.0%; 0.0%. Class C; 0.1%; 0.1%; 0.0%. Class D; 0.0%; 0.0%; -0.1%. Class E; 0.0%; -0.1%; -0.1%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; 0.0%. Enc Time[%]; 96%. Dec Time[%]; 97%. Random Access HE. Y; U; V. Class A; 0.0%; 0.1%; -0.1%. Class B; 0.0%; -0.1%; -0.1%. Class C; 0.0%; -0.1%; 0.1%. Class D; 0.0%; 0.0%; -0.2%. Class E. Overall; 0.0%; 0.0%; -0.1%. 0.0%; -0.1%; -0.1%. Enc Time[%]; 100%. Dec Time[%]; 99%. Low delay B HE. Y; U; V. Class A. Class B; 0.0%; -0.2%; -0.3%. Class C; 0.0%; 0.0%; -0.1%. Class D; 0.0%; 0.3%; -0.3%. Class E; 0.0%; 0.1%; 0.0%. Overall; 0.0%; 0.0%; -0.2%. 0.0%; 0.0%; -0.2%. Enc Time[%]; 100%. Dec Time[%]; 100%. Low delay P HE. Y; U; V. Class A. Class B; 0.0%; 0.0%; -0.2%. Class C; 0.0%; -0.1%; 0.1%. Class D; 0.0%; 0.1%; -0.1%. Class E; 0.0%; -0.1%; 0.2%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; -0.1%. Enc Time[%]; #NUM!. Dec Time[%]; #NUM!. | All Intra HE. Y; U; V. Class A; 0.0%; -0.1%; -0.1%. Class B; 0.0%; 0.0%; 0.0%. Class C; 0.1%; 0.1%; 0.0%. Class D; 0.0%; 0.0%; -0.1%. Class E; 0.0%; -0.1%; -0.1%. Class F; 0.1%; 0.1%; 0.1%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; 0.0%. Enc Time[%]; 96%. Dec Time[%]; 97%. Random Access HE. Y; U; V. Class A; 0.0%; 0.1%; -0.1%. Class B; 0.0%; -0.1%; -0.1%. Class C; 0.0%; -0.1%; 0.1%. Class D; 0.0%; 0.0%; -0.2%. Class E. Class F; 0.2%; 0.2%; 0.1%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; -0.1%. Enc Time[%]; 100%. Dec Time[%]; 100%. Low delay B HE. Y; U; V. Class A. Class B; 0.0%; -0.2%; -0.3%. Class C; 0.0%; 0.0%; -0.1%. Class D; 0.0%; 0.3%; -0.3%. Class E; 0.0%; 0.1%; 0.0%. Class F; 0.1%; -0.3%; 0.5%. Overall; 0.0%; 0.0%; -0.1%. 0.0%; 0.0%; 0.0%. Enc Time[%]; 100%. Dec Time[%]; 102%. Low delay P HE. Y; U; V. Class A. Class B; 0.0%; 0.0%; -0.2%. Class C; 0.0%; -0.1%; 0.1%. Class D; 0.0%; 0.1%; -0.1%. Class E; 0.0%; -0.1%; 0.2%. Class F; 0.0%; 0.1%; 0.0%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.1%; -0.1%. Enc Time[%]; #NUM!. Dec Time[%]; #NUM!. |

## CABAC bypass mode used for rem_intra_luma_pred_mode only

| WITHOUT CLASS F SEQUENCES | WITH CLASS F SEQUENCES |
| --- | --- |
| All Intra HE. Y; U; V. Class A; 0.0%; -0.1%; -0.1%. Class B; 0.0%; 0.0%; 0.0%. Class C; 0.1%; 0.1%; 0.0%. Class D; 0.0%; 0.0%; -0.1%. Class E; 0.0%; -0.1%; -0.1%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; 0.0%. Enc Time[%]; 96%. Dec Time[%]; 97%. Random Access HE. Y; U; V. Class A; 0.0%; 0.1%; -0.1%. Class B; 0.0%; -0.1%; -0.1%. Class C; 0.0%; -0.1%; 0.1%. Class D; 0.0%; 0.0%; -0.2%. Class E. Overall; 0.0%; 0.0%; -0.1%. 0.0%; -0.1%; -0.1%. Enc Time[%]; 100%. Dec Time[%]; 99%. Low delay B HE. Y; U; V. Class A. Class B; 0.0%; -0.2%; -0.3%. Class C; 0.0%; 0.0%; -0.1%. Class D; 0.0%; 0.3%; -0.3%. Class E; 0.0%; 0.1%; 0.0%. Overall; 0.0%; 0.0%; -0.2%. 0.0%; 0.0%; -0.2%. Enc Time[%]; 100%. Dec Time[%]; 100%. Low delay P HE. Y; U; V. Class A. Class B; 0.0%; 0.0%; -0.2%. Class C; 0.0%; -0.1%; 0.1%. Class D; 0.0%; 0.1%; -0.1%. Class E; 0.0%; -0.1%; 0.2%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; -0.1%. Enc Time[%]; #NUM!. Dec Time[%]; #NUM!. | All Intra HE. Y; U; V. Class A; 0.0%; -0.1%; -0.1%. Class B; 0.0%; 0.0%; 0.0%. Class C; 0.1%; 0.1%; 0.0%. Class D; 0.0%; 0.0%; -0.1%. Class E; 0.0%; -0.1%; -0.1%. Class F; 0.1%; 0.1%; 0.1%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; 0.0%. Enc Time[%]; 96%. Dec Time[%]; 97%. Random Access HE. Y; U; V. Class A; 0.0%; 0.1%; -0.1%. Class B; 0.0%; -0.1%; -0.1%. Class C; 0.0%; -0.1%; 0.1%. Class D; 0.0%; 0.0%; -0.2%. Class E. Class F; 0.2%; 0.2%; 0.1%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; -0.1%. Enc Time[%]; 100%. Dec Time[%]; 100%. Low delay B HE. Y; U; V. Class A. Class B; 0.0%; -0.2%; -0.3%. Class C; 0.0%; 0.0%; -0.1%. Class D; 0.0%; 0.3%; -0.3%. Class E; 0.0%; 0.1%; 0.0%. Class F; 0.1%; -0.3%; 0.5%. Overall; 0.0%; 0.0%; -0.1%. 0.0%; 0.0%; 0.0%. Enc Time[%]; 100%. Dec Time[%]; 102%. Low delay P HE. Y; U; V. Class A. Class B; 0.0%; 0.0%; -0.2%. Class C; 0.0%; -0.1%; 0.1%. Class D; 0.0%; 0.1%; -0.1%. Class E; 0.0%; -0.1%; 0.2%. Class F; 0.0%; 0.1%; 0.0%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.1%; -0.1%. Enc Time[%]; #NUM!. Dec Time[%]; #NUM!. |

## CABAC bypass mode used for both mpm_idx and rem_intra_luma_pred_mode

| WITHOUT CLASS F SEQUENCES | WITH CLASS F SEQUENCES |
| --- | --- |
| All Intra HE. Y; U; V. Class A; 0.0%; -0.2%; -0.1%. Class B; 0.1%; 0.0%; 0.1%. Class C; 0.0%; 0.1%; 0.0%. Class D; 0.0%; 0.0%; -0.1%. Class E; 0.0%; -0.1%; 0.0%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; 0.0%. Enc Time[%]; 99%. Dec Time[%]; 99%. Random Access HE. Y; U; V. Class A; 0.0%; 0.1%; 0.0%. Class B; 0.0%; -0.1%; 0.0%. Class C; 0.0%; -0.1%; -0.1%. Class D; 0.0%; 0.0%; 0.0%. Class E. Overall; 0.0%; 0.0%; 0.0%. 0.0%; -0.1%; -0.1%. Enc Time[%]; 100%. Dec Time[%]; 100%. Low delay B HE. Y; U; V. Class A. Class B; 0.0%; -0.1%; -0.6%. Class C; 0.0%; -0.1%; -0.2%. Class D; 0.0%; 0.3%; -0.2%. Class E; 0.1%; -0.1%; -1.2%. Overall; 0.0%; 0.0%; -0.5%. 0.0%; -0.1%; -0.6%. Enc Time[%]; 98%. Dec Time[%]; 99%. Low delay P HE. Y; U; V. Class A. Class B; 0.0%; 0.0%; -0.3%. Class C; 0.0%; -0.1%; -0.1%. Class D; 0.0%; -0.4%; 0.0%. Class E; 0.1%; -0.5%; -1.3%. Overall; 0.0%; -0.2%; -0.3%. 0.1%; -0.2%; -0.4%. Enc Time[%]; #NUM!. Dec Time[%]; #NUM!. | All Intra HE. Y; U; V. Class A; 0.0%; -0.2%; -0.1%. Class B; 0.1%; 0.0%; 0.1%. Class C; 0.0%; 0.1%; 0.0%. Class D; 0.0%; 0.0%; -0.1%. Class E; 0.0%; -0.1%; 0.0%. Class F; 0.1%; 0.1%; 0.1%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; 0.0%. Enc Time[%]; 99%. Dec Time[%]; 99%. Random Access HE. Y; U; V. Class A; 0.0%; 0.1%; 0.0%. Class B; 0.0%; -0.1%; 0.0%. Class C; 0.0%; -0.1%; -0.1%. Class D; 0.0%; 0.0%; 0.0%. Class E. Class F; 0.1%; 0.4%; 0.0%. Overall; 0.0%; 0.0%; 0.0%. 0.0%; 0.0%; 0.0%. Enc Time[%]; 100%. Dec Time[%]; 100%. Low delay B HE. Y; U; V. Class A. Class B; 0.0%; -0.1%; -0.6%. Class C; 0.0%; -0.1%; -0.2%. Class D; 0.0%; 0.3%; -0.2%. Class E; 0.1%; -0.1%; -1.2%. Class F; 0.0%; -0.3%; -0.2%. Overall; 0.0%; -0.1%; -0.5%. 0.0%; -0.1%; -0.5%. Enc Time[%]; 98%. Dec Time[%]; 101%. Low delay P HE. Y; U; V. Class A. Class B; 0.0%; 0.0%; -0.3%. Class C; 0.0%; -0.1%; -0.1%. Class D; 0.0%; -0.4%; 0.0%. Class E; 0.1%; -0.5%; -1.3%. Class F; 0.1%; 0.0%; -0.1%. Overall; 0.1%; -0.2%; -0.3%. 0.1%; -0.2%; -0.3%. Enc Time[%]; #NUM!. Dec Time[%]; #NUM!. |

We assert that this is a negligible impact on compression performance.

# Conclusion

This document proposed the use of bypass coding engine for the syntax element mpm_idx and rem_intra_luma_pred_mode. The performance impact was measured by implementing the change in HM-4.0 and was found to be negligible.

We propose to adopt the technique into the HM.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] High efficiency test model (HM) version 4.0 URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

[2] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-F900/m21452, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Proposed Working Draft Text Modifications

Changes are highlighted in yellow.

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | … | … | … | … | … |
| coding_tree() | … | … | … | … | … |
| coding_unit() | … | … | … | … | … |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | rem_intra_luma_pred_mode | Table 9-3 | 0 | 1 | 2 |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 | Error: Reference source not found |  | 0..1 | 2..3 |

| Table 9-2 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| … | … | … | … | … | … |
| pcm_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| prev_intra_luma_pred_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| mpm_idx | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| rem_intra_luma_pred_mode | all | as specified in subclause Error: Reference source not found | na | na | na, (uses Decode Bypass) |
| rem_intra_luma_pred_mode | I | as specified in subclause Error: Reference source not found | 0 | Table 9-3 | 0 |
|  | P |  | 0 | Table 9-3 | 1 |
|  | B |  | 0 | Table 9-3 | 2 |
| intra_chroma_pred_mode ( IntraPredMode < 4 ) | I | TU, cMax = 3 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| intra_chroma_pred_mode ( 4 <= IntraPredMode < 34 ) | I | TU, cMax = 4 | 1 | Error: Reference source not found | 0 |
|  | P |  | 1 | Error: Reference source not found | 2 |
|  | B |  | 1 | Error: Reference source not found | 4 |
| … | … | … | … | … | … |
| coeff_sign_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |

Table 9-3 – Values of variable m and n for rem_intra_luma_pred_mode ctxIdx

| Initialisation variables | rem_intra_luma_pred_mode ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| m | -3 | -2 | 1 |
| n | 65 | 61 | 55 |