| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G709 |
| --- | --- |

| Title: | On partition size information coding in B-slices using CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yamamoto Tomoyuki, Kiran Misra, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA | Tel: Email: | +1 360 817 8451 yamamoto.tomoyuki@sharp.co.jp misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp |  |  |

_____________________________

# Abstract

This document describes a technique for CABAC complexity reduction while coding partition size information in B slices. The document proposes the use of bypass coding engine of CABAC to code bins of syntax element pred_type used to distinguish between PartMode (2NxN, 2NxnU, 2NxnD) as well as bins used to distinguish between PartMode (Nx2N, nLx2N, nRx2N). This reduces the number of CABAC contexts in memory and eliminates the associated CABAC context update step.  It is asserted that this has negligible impact on compression efficiency. For HM-4.0, high efficiency common test conditions the proposed change shows an average BD bitrate impact of –

(Without class F sequences)

RA_HE Y:0.00% U:0.02% V:0.02%; LB_HE Y:-0.02% U:0.07% V:0.00%.

(With class F sequences)

RA_HE Y:0.00% U:0.01% V:0.00%; LB_HE Y:-0.04% U:0.09% V:-0.04%.

# Introduction

In the current HM, the CABAC codes syntax elements using either the regular coding engine or the bypass coding engine. The regular coding engine maintains a context in memory and is updated for every bin coded. The bypass coding engine on the other hand assumes an approximately equal probability distribution when coding bins and does not need to maintain a context in memory or perform updates. Therefore using bypass coding engine instead of the regular coding engine results in complexity reduction.

# Proposed Change

In HM-4.0, the syntax element pred_type along with (log2CUSize, Log2MinCUSize, inter_4x4_enabled_flag, slice_type) is used to derive variables (PredMode, PartMode, IntraSplitFlag). For B-slices, the bins of pred_type which are used to distinguish between PartMode (2NxN, 2NxnU, 2NxnD) are highlighted in red in Table 1. The bins colored in are coded using two CABAC contexts, the context chosen for coding by CABAC’s regular coding engine is determined based on the bin position being coded. Similarly, for B-slices, the bins of pred_type which are used to distinguish between PartMode (Nx2N, nLx2N, nRx2N) are highlighted in green in Table 1. The bins colored in green are coded using two CABAC contexts, the context chosen for coding by CABAC’s regular coding engine is determined based on the bin position being coded. The decoder needs to be maintained in memory the associated CABAC contexts and perform an update of the context after every bin coding.

Table 1 – Binarization for pred_type

| Slice type | Value of pred_type | PredMode | PartMode | Bin string |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cLog2CUSize > Log2MinCUSize | cLog2CUSize = = Log2MinCUSize |  |
|  |  |  |  |  | cLog2CUSize = = 3 && !inter_4x4_enabled_flag | cLog2CUSize > 3 ¦ ¦ inter_4x4_enabled_flag |
| B | 0 | MODE_INTER | PART_2Nx2N | 1 | 1 | 1 |
|  | 1 | MODE_INTER | PART_2NxN | 011 | 01 | 01 |
|  | 2 | MODE_INTER | PART_Nx2N | 001 | 001 | 001 |
|  | 4 | MODE_INTER | PART_2NxnU | 0100 | - | - |
|  | 5 | MODE_INTER | PART_2NxnD | 0101 | - | - |
|  | 6 | MODE_INTER | PART_nLx2N | 00100 | - | - |
|  | 7 | MODE_INTER | PART_nRx2N | 00101 | - | - |
|  | 3 | MODE_INTER | PART_NxN | - | - | 0001 |
|  | 4 | MODE_INTRA | PART_2Nx2N | 000 | 000 0 | 0000 0 |
|  | 5 | MODE_INTRA | PART_NxN | - | 000 1 | 0000 1 |

It is proposed that the contexts corresponding to bins colored in red and green for syntax element pred_type be coded using the CABAC bypass coding engine thereby circumventing the need to maintain associated contexts in memory and eliminate the associated CABAC context update step. The proposed change results in negligible impact on compression efficiency while reducing complexity.

# Results

The proposed change was implemented in HM 4.0 [1]. All sequences (including class F) were then code using the configuration files described in JCTVC-F900 [2]. Full results are available in the attached XLS sheets. The rate-distortion summary is as follows:

## Without Class F sequences

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.02% | -0.13% | -0.16% |
| Class B | -0.02% | 0.02% | 0.03% |
| Class C | 0.00% | 0.05% | -0.01% |
| Class D | 0.03% | 0.13% | 0.22% |
| Class E |  |  |  |
| Overall | 0.00% | 0.02% | 0.02% |
|  | 0.00% | 0.02% | 0.03% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.00% | 0.05% | -0.20% |
| Class C | 0.00% | 0.11% | 0.09% |
| Class D | -0.01% | -0.12% | 0.07% |
| Class E | -0.07% | 0.33% | 0.15% |
| Overall | -0.02% | 0.07% | 0.00% |
|  | -0.01% | 0.00% | -0.03% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

## With Class F sequences

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.02% | -0.13% | -0.16% |
| Class B | -0.02% | 0.02% | 0.03% |
| Class C | 0.00% | 0.05% | -0.01% |
| Class D | 0.03% | 0.13% | 0.22% |
| Class E |  |  |  |
| Class F | -0.01% | -0.04% | -0.09% |
| Overall | 0.00% | 0.01% | 0.00% |
|  | 0.00% | 0.01% | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.00% | 0.05% | -0.20% |
| Class C | 0.00% | 0.11% | 0.09% |
| Class D | -0.01% | -0.12% | 0.07% |
| Class E | -0.07% | 0.33% | 0.15% |
| Class F | -0.14% | 0.17% | -0.19% |
| Overall | -0.04% | 0.09% | -0.04% |
|  | -0.04% | 0.04% | -0.05% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

The summary results above confirm that for B-slices using the bypass coding engine to code bins of syntax element pred_type used to distinguish between PartMode (2NxN, 2NxnU, 2NxnD) as well as bins used to distinguish between PartMode (Nx2N, nLx2N, nRx2N) results in negligible impact on compression performance.

# Conclusion

This document proposed the use of bypass coding engine to code bins of syntax element pred_type, for B-slices, used to distinguish between PartMode (2NxN, 2NxnU, 2NxnD) as well as bins used to distinguish between PartMode (Nx2N, nLx2N, nRx2N). The performance impact was measured by implementing the change in HM-4.0 and was found to be negligible.

We propose adoption of the technique into the HM.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] High efficiency test model (HM) version 4.0 URL: https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

[2] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-F900/m21452, 6th Meeting: Torino, IT, 14-22 July, 2011.