| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G712 |
| --- | --- |

| Title: | Non-CE9:The Simplification of MVP for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Krit Panusopone Limin Wang | Tel: Email: | +1-858-404-2251 yyu@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This contribution proposes a simplification of MVP design for HEVC. Simulation results show that a loss of 0.1% for LBHE and a loss of 0.3% for RAHE are observed compared to original AMVP while the complexity of the proposed method is reduced at least 50%, and up to 66.7% as compared to the MVP selection procedure of AMVP when a CU consist of two PUs.

# Introduction

In HEVC [1], motion vectors (MV) are predictive coded. For a current PU with one MV, MV predictor (MVP) is derived from MVs of the spatial neighboring PUs or temporal collocated PUs of the current PU. The difference between a current MV and its MVP is then coded. If a PU uses both reference list0 and list1, two MVP are needed. In JCTVC-A124 [2]. AMVP is proposed to pick the best MVP selection from a candidate set of MVPs for a current PU. In [2], the candidate set of MVPs for MV of a current PU is composed of MVs of spatial neighboring and collocated PUs. The candidate set of MVPs is reduced by removing the duplicated MVP candidates. An index indicating the selected MVP candidate, mvpIdx, may be transmitted in the bitstream, if necessary. In [3], a fixed number of MVP candidates are introduced.

In HEVC, a CU may consist of more than one PUs. For a CU with two PUs, if both PUs use reference list 0 and list 1, there are up to four MVP indexes that may be needed to indicate the selected MVP candidates. As a result, it increases the complexity and may also results in a poor coding performance.

# The Simplification of MVP Calculation for HEVC

If there are two PUs within a CU, as shown in the Figure 1 where PU0 is considered as the first PU in a CU, followed by PU1 in terms of coding order, these two PUs should have high correlation and the MVP for PU0 and PU1 should be very similar.

Figure 1. Two examples of a CU which consist of PUs.

This contribution proposes that the MVP for the second PU in a CU should be derived from the MVP of the first PU.

Specifically, we propose that the MVP0_PU1for PU1 is calculated from MVP0_PU0.

MVP0_PU1 = (TR0_PU1/ TR0_PU0) * MVP0_PU0                           	(1)

Similarly, the MVP1_PU1for PU1 is calculated from MVP1_PU0.

MVP1_PU1 = (TR1_ PU1/ TR1_ PU0) * MVP1_PU0                      	(2)

where TR0_PU0 be the difference between the current picture POC and the reference picture POC pointed by MV0_PU0, TR0_PU1 be the difference between the current picture POC and the reference picture POC pointed by MV0_PU1, TR1_PU0 is the difference between the current picture POC and the reference picture POC pointed by MV1_PU0, and TR1_PU1 is the difference between the current picture POC and the reference picture POC pointed by MV1_PU1, respectively.

A PU may not use both reference list0 and list1. For example, PU0 may only utilize either reference list0 or reference list1.  On the other hand, PU1 may still use both reference list0 and list1. In that case, both MVP0_PU1 and MVP1_PU1 can be derived from either MVP0_PU0 or MVP1_PU0. For example, if only MVP0_PU0 is available, MVP0_PU1 and MVP1_PU1 can be determined as

MVP0_PU1 = (TR0_PU1/ TR0_PU0) * MVP0_PU0                                  	(3)

MVP1_PU1 = (TR1_ PU1/ TR0_ PU0) * MVP0_PU0                                   (4)

# The Simulation Results

The proposed method was implemented in the HM-4.0 and tested to evaluate its coding performance and complexity. The binaries were compiled using Microsoft Visual Studio 2010 64 bits. The coding conditions used for the simulations are specified in [4]. Experimental result is given in Table 1.

From Table 1, we can see that there is minor loss. However, the proposed method reduces at least half and up to 66.7% of complexity of finding MVP compared to original AMVP if a CU consists of two PUs.

Table 1: Coding performance and complexity of the proposed method

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4% | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% |
| Class B | 0.2% | 0.3% | 0.3% | 0.2% | 0.2% | 0.3% |
| Class C | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.4% |
| Class D | 0.3% | 0.3% | 0.4% | 0.2% | 0.1% | 0.3% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.4% |
|  | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.4% |
| Enc Time[%] | 88% |  |  | 88% |  |  |
| Dec Time[%] | 81% |  |  | 83% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | -0.2% | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.0% |
| Class D | 0.2% | 0.5% | 0.4% | 0.2% | 0.2% | 0.0% |
| Class E | 0.0% | -0.3% | 0.5% | 0.0% | 0.0% | -0.6% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | -0.1% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 87% |  |  | 88% |  |  |
| Dec Time[%] | 83% |  |  | 84% |  |  |

# Conclusions

In this contribution, a simplification of MVP design for HEVC is proposed if a CU consists of two PUs. Simulation results show that there is a minor loss while reducing at least 50% and up to 66.7% of complexity of finding MVP compared to original AMVP.

# References

[1] “HEVC Specification Working Draft 2,” JCT-VC document, JCTVC-D503, Jan. 2011.

[2] Ken McCann, Woo-Jin Han, Il-Koo Kim, Jung-Hye Min, Elena Alshina, Alexander Alshin, Tammy Lee, Jianle Chen, Vadim Seregin, Sunil Lee, Yoon-Mi Hong, Min-Su Cheon, Nikolay Shlyakhov, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, April, 2010.

[3] Toshiyasu Sugio, Takahiro Nishi, “Parsing Robustness for Merge/AMVP,” JCT-VC document, JCTVC-F470, July, 2011.

[4] Frank Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-D600, Jan. 2011.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).