| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G713 |
| --- | --- |

| Title: | Long term picture referencing using wrapped POC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Sachin Deshpande, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com sdeshpande@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes a technique to signal long term reference pictures using “wrapped POC” scheme.  It is asserted that the method enables: (a) Each picture to be uniquely identifiable, (b) Reference picture signaling for a picture to be independent of other pictures, and (c) including the long term pictures in reference picture sets.  The method consists of using a POC cycle parameter when referencing long term pictures.

# Introduction

JCTVC-F493 [1] proposed absolute signaling of reference pictures which supports error robust management of reference pictures using Reference Picture Sets (previously called Buffer Descriptions). In particular JCTVC-F493 allows reference pictures to be signaled and identified independently of previous (or future) pictures.

JCTVC-F493 originally proposed “wrapped POC” signaling scheme where the POC numbering would wrap around to 0 after a maximum value (MaxPOC-1), allowed by the fixed number of bits being used for POC numbering, is reached. In case a previous long term reference picture exists in the decoded picture buffer (DPB) and its POC is assigned to a newer picture, then the decoder is unable to distinguish between the newer picture using the same POC as the older long term reference picture. A potential solution discussed in AHG21 [2] to this issue proposed “stepping over” POC which are in-use. However, this leads to shrinking of the POC numbering space. Another potential solution is to increase the “number of bits” being used for POC numbering however this impacts overall compression efficiency.

In this contribution we propose a signaling scheme for long term reference pictures which overcomes above drawbacks while using the “wrapped POC” approach. The proposed approach has following advantages:

- It makes full use of the available POC numbering space [0,…,MaxPOC-1] as the stepping over of POC and associated POC space shrinkage issue is addressed.

- The proposed approach for signaling is self-contained in each picture, thus error resilience is better compared to a scheme which relies on information propagation from previous pictures (which could get lost/dropped as a part of a temporal layer that is dropped).

- If the number of bits allocated to POC numbering is sufficient then the proposed change requires minimal additional bits.

Figure 1: Example illustration of poc_cycle assignment for a picture

# Proposed Change

The document proposes the use of a poc_cycle field for referencing long term pictures. The poc_cycle field is determined as follows:

- The poc_cycle for the current picture being decoded is 0.

- Pictures other than that being decoded determine their poc_cycle with respect to the picture being decoded. Figure 1illustrates an example showing the poc_cyle associated with different [0, …, MaxPOC-1] picture sets.

## Syntax and Semantics:

### Reference picture set RBSP syntax

| ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| num_longterm_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_longterm_pics; i++ ) { |  |
| longterm_poc [ i ] | ue(v) |
| longterm_poc_cycle[ i ] | se(v) |
| } |  |
| } |  |

longterm_poc [ i ] specifies the picture order count of the i-th long term picture being referenced.

longterm_poc_cycle[ i ] specifies the cycle parameter for the picture order count of the i-th long term picture being referenced.

## Example illustrating utility of proposed change

Figure 2: Example illustrating long term picture referencing using proposed changes

Figure 2 illustrates an example where the picture being decoded (in poc_cycle=0) reference a fixed long term picture. The working draft syntax in [3] would signal the long term picture using a deltaPOC method, where the value of deltaPOC is different for every picture. This can be achieved by:

  - Using picture parameter set: Signal multiple reference picture sets with different deltaPOC in the picture parameter set and having the picture being decoded refer to these individual reference picture sets, or

  - Using slice header: Signaling a deltaPOC for the long term picture in every picture being decoded

Both the above listed approaches are inefficient.

With the proposed change the long term picture may be signaled by:

- Using picture parameter set: Defining a reference picture set containing the POC and poc_cycle for the long term picture signaled in the picture parameter set. A picture referencing the long term picture may then point to the appropriate reference picture set in the picture parameter set.

- Using slice header: The POC and poc_cycle for the long term picture may be  signaled in current picture being decoded

The proposed changes result in long term picture signaling with better or comparable compression efficiency.

# Conclusion

A method to signal long term pictures using wrapped POC and a poc_cycle field is proposed. It is asserted that this approach enables signaling of reference pictures for a picture to be independent of other pictures, while maintain the simplicity associated with wrapped POC signaling. It is also proposed that long term picture signaling be added to reference picture sets to achieve better compression efficiency as compared to deltaPOC when signaling long term pictures. The efficacy of the proposed solution is demonstrated.

We propose to include the approach in any HEVC design that includes the reference picture set concept.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] R. Sjöberg, J. Samuelsson (Ericsson), "Absolute signaling of reference pictures," JCTVC-F493/m20923 6th Meeting: Torino, IT, 14-22 July, 2011.

[2] R. Sjöberg, J. Samuelsson  (Ericsson), "JCT-VC AHG report: xxxx (AHG21)” JCTVC-G021/mxxxx 7th Meeting: Geneva, CH, 21-30 November, 2011.

[3] ftp://ftp.hhi.de/ahg21/JCTVC-F803_d5_RPS_r1.doc Candidate Working Draft text by AHG 21