| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G714 |
| --- | --- |

| Title: | AHG21: On picture referencing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Sachin Deshpande, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com sdeshpande@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes syntax to: signal long-term pictures using absolute POC; signal reference picture sets containing long term pictures only; replace, delete or append entries to a reference picture set defined in the picture parameter set. It is asserted that this change enables: (a) Signaling of reference picture set containing long term pictures and (b) Efficient signaling of reference picture sets for the current picture if the desired reference picture set differs from that defined in the picture parameter set for only a small subset of entries.

# Introduction

JCTVC-F493 [1] proposed absolute signaling of reference pictures which supports error robust management of reference pictures using Buffer Descriptions. JCTVC-F493 used wrapped POC signaling scheme. To study possible improvements based on [1] to the reference picture list construction process an ad-hoc group AHG21 [2] was formed at the 6th JCT-VC meeting [4]. One of the outputs of AHG21 has been a candidate working draft specification [3] for signaling reference pictures. [3] suggests an alternative POC numbering scheme where the MSB of a POC is determined based on the MSB (POC) and LSB (POC) of previously decoded picture (at temporalID=0)  and explicit transmission of LSB (POC) in the slice header is proposed in [3]. The MSB/LSB approach proposed in [3] achieves lower bit consumption at the cost of introducing a dependency in “POC determination of current picture” to “POC determination of a previous picture” in the bitstream.

It is asserted that the working draft syntax in [3] does not adequately address the signaling of long term pictures. In this contribution we propose:

- A signaling scheme for long term reference pictures using absolute POC.

- A signaling scheme for reference picture sets containing long term pictures only.

- A signaling scheme to selectively replace, delete or add entries within a reference picture set defined in the picture parameter set.

# Proposed Change

The document proposes the use of the following syntax to signal long-term pictures using absolute POC; signal reference picture sets containing long term pictures only; replace, delete or add entries to a reference picture set defined in the picture parameter set and referenced by a picture in the slice header.

## Syntax and Semantics:

We define the appropriate syntax and semantics based on syntax defined in [3].

### Reference picture set RBSP syntax

| ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| num_longterm_pics | ue(v) |
| longterm_pics_only_flag | u(1) |
| if (longterm_pics_only_flag==0) { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| for( i = 0; i < num_longterm_pics; i++ ) { |  |
| longterm_poc [ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |

num_longterm_pics specifies the number of long term pictures specified by syntax elements longterm_poc[ i ], longterm_poc_cycle[ i ], used_by_curr_pic_pic_lt_flag[ i ]. The value of num_longterm_pics shall be in the range of 0 to max_num_ref_frames – num_negative_pics,- num_positive_pics inclusive.

longterm_pics_only_flag equals to 1 identifies that the reference picture set contains long term pictures only. longterm_pics_only_flag equals to 0 identifies that the reference picture set contains both short term and long term pictures.

longterm_poc [ i ] specifies the absolute picture order count of the i-th long term picture being referenced. The longterm_poc [ i ] is the sum of MSB (POC) and LSB (POC) for the long term picture being referenced.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long term picture is not used for reference by the current picture. used_by_curr_pic_lt_flag[ i ] equal to 1 specifies that the i-th long term picture is used for reference by the current picture.

### Slice header RBSP syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| ref_pic_set_pps_flag | u(1) |
| if( ! ref_pic_set_pps_flag ) |  |
| ref_pic_set( num_ref_pic_sets ) |  |
| else { |  |
| ref_pic_set_idx | u(v) |
| rps_pps_replace_count | ue(v) |
| if (rps_pps_replace_count>0) |  |
| for (j=0; j< rps_pps_replace_count; j++) { |  |
| rps_pps_replace_idx[j] | ue(v) |
| rps_pps_poc_replace[j] | ue(v) |
| rps_use_by_curr_pic_replace_flag[j] | u(1) |
| } |  |
| rps_pps_delete_count | ue(v) |
| if (rps_pps_delete_count>0) |  |
| for (j=0; j< rps_pps_delete_count; j++) |  |
| rps_pps_delete_idx[j] | ue(v) |
| rps_pps_append_count | ue(v) |
| if (rps_pps_append_count>0) |  |
| for (j=0; j< rps_pps_append_count; j++) { |  |
| rps_pps_poc_append[j] | ue(v) |
| rps_use_by_curr_pic_append_flag[i] | u(1) |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

A derived list POCBD_pps is created by concatenating the negative deltaPOC, positive deltaPOC and absolute POC entries from reference picture set with index ref_pic_set_idx in the picture parameter set and converting them to POC values.

A derived list UsedByCurrPicBD_pps by concatenating the used_by_curr_pic_s0_flag used_by_curr_pic_s0_flag and used_by_curr_pic_longterm_flag entries from reference picture set with index ref_pic_set_idx in the picture parameter set.

rps_pps_replace_count specifies the number of entries that are to be replaced in the derived list POCBD_pps and UsedByCurrPicBD_pps, for the current picture only. rps_pps_replace_count shall be in the range of 0 to (num_negative_pics + num_positive_pics + num_longterm_pics) if longterm_pics_only_flag equals 0. rps_pps_replace_count shall be in the range of 0 to num_longterm_pics if longterm_pics_only_flag equals 1.

rps_pps_replace_idx[j] specifies the index of the reference picture entry in the derived list POCBD_pps and UsedByCurrPicBD_pps which shall be replaced. rps_pps_replace_idx[j] shall be in the range of 0 to (num_negative_pics + num_positive_pics + num_longterm_pics - 1) if longterm_pics_only_flag equals 0. rps_pps_replace_idx[j] shall be in the range of 0 to (num_longterm_pics - 1) if longterm_pics_only_flag equals 1.

rps_pps_poc_replace[j] specifies the value used to replace the entry POCBD_pps[rps_pps_replace_idx[j]].

rps_use_by_curr_pic_replace_flag[j] specifies the value used to replace the entry UsedByCurrPicBD_pps[rps_pps_replace_idx[j]].

rps_pps_delete_count specifies the number of entries that are to be deleted in the derived list POCBD_pps and UsedByCurrPicBD_pps, for the current picture only. rps_pps_delete_count shall be in the range of 0 to (num_negative_pics + num_positive_pics + num_longterm_pics - 1) if longterm_pics_only_flag equals 0. rps_pps_delete_count shall be in the range of 0 to (num_longterm_pics - 1) if longterm_pics_only_flag equals 1.

rps_pps_delete_idx[j] ] specifies the index of the reference picture entry in the derived list POCBD_pps and UsedByCurrPicBD_pps which shall be deleted, for the current picture only. rps_pps_delete_idx[j] shall be in the range of 0 to (num_negative_pics + num_positive_pics + num_longterm_pics - 1) if longterm_pics_only_flag equals 0. rps_pps_delete_idx[j] shall be in the range of 0 to (num_longterm_pics - 1) if longterm_pics_only_flag equals 1.

rps_pps_append_count specifies the number of entries that are to be appended to the derived list POCBD_pps and UsedByCurrPicBD_pps, for the current picture only. rps_pps_append_count shall be in the range of 0 to (max_num_ref_frames  - num_negative_pics - num_positive_pics - num_longterm_pics + rps_pps_delete_count) if longterm_pics_only_flag equals 0. rps_pps_append_count shall be in the range of 0 to (max_num_ref_frames  - num_longterm_pics + rps_pps_delete_count) if longterm_pics_only_flag equals 1.

rps_pps_poc_append[j] specifies the absolute POC of the reference picture entry to be appended to the list POCBD_pps. rps_use_by_curr_pic_append_flag[i] specifies the corresponding used by current picture flag value to be appended to the list UsedByCurrPicBD_pps.

The POCBD_pps and UsedByCurrPicBD_pps list is then used by the reference picture list construction process for the current picture.

# Conclusion

The document proposed syntax to signal long-term pictures using absolute POC; signal reference picture sets containing long term pictures only; replace, delete or add entries within a reference picture set defined in the picture parameter set. This enables (a) Signaling of reference picture set containing long term pictures and (b) Efficient signaling of reference picture sets for the current picture if the desired reference picture set differs from that defined in the picture parameter set for only a small subset of entries. We propose to include the approach in any HEVC design that includes the reference picture set concept.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] R. Sjöberg, J. Samuelsson (Ericsson), "Absolute signaling of reference pictures," JCTVC-F493/m20923 6th Meeting: Torino, IT, 14-22 July, 2011.

[2] R. Sjöberg, J. Samuelsson  (Ericsson), "JCT-VC AHG report: xxxx (AHG21)” JCTVC-G021/mxxxx 7th Meeting: Geneva, CH, 21-30 November, 2011.

[3] ftp://ftp.hhi.de/ahg21/JCTVC-F803_d5_RPS_r1.doc Candidate Working Draft text by AHG 21

[4] 6th JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 meeting: Turin, IT, 14-22 July 2011.