| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G714 |
| --- | --- |

| Title: | AHG21: On picture referencing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Sachin Deshpande, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com sdeshpande@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes syntax to delete or append entries to a reference picture set defined in the picture parameter set. It is asserted that this change enables efficient signaling of reference pictures when the desired reference picture set differs from that defined in the picture parameter set for only a small subset of entries.

# Introduction

JCTVC-F493 [1] proposed absolute signaling of reference pictures which supports error robust management of reference pictures using Buffer Descriptions. F493 used wrapped POC signaling scheme.

In this contribution we propose a signaling scheme to delete or add entries within a reference picture set defined in the picture parameter set.

# Proposed Change

The document proposes the use of the following syntax to delete or add entries to a reference picture set defined in the picture parameter set and referenced by a picture in the slice header.

## Syntax and Semantics:

We define the appropriate syntax and semantics required for performing the delete/append operation based on syntax defined in [3].

### Buffer description syntax

| buffer_description( ) { | Descriptor |
| --- | --- |
| bd_reference_flag | u(1) |
| if(bd_reference_flag = = 1) { |  |
| bd_idx | u(v) |
| bd_poc_delete_count | ue(v) |
| if (bd_poc_delete_count >0 ) |  |
| for( j = 0; j < bd_poc_delete_count; j++ ) { |  |
| bd_pps_delete_idx[j] | ue(v) |
| } |  |
| bd_poc_append_count | ue(v) |
| if (bd_poc_append_count >0 ) |  |
| for( j = 0; j < bd_poc_append_count; j++ ) { |  |
| poc_pps_append[j] | ue(v) |
| } |  |
| } else { |  |
| number_of_negative_pictures | ue(v) |
| for( i = 0; i < number_of_negative_pictures; i++ ) { |  |
| negative_delta_poc_minus_one[i] | ue(v) |
| if( bits_for_temporal_id_in_buffer_descriptions > 0 ) |  |
| temporal_id_negative[i] | u(v) |
| } |  |
| if( positive_pictures_in_buffer_descriptions_flag ){ |  |
| number_of_positive_pictures | ue(v) |
| for( i = 0; i < number_of_positive_pictures; i++ ) { |  |
| delta_poc_minus_one[i] | ue(v) |
| if( bits_for_temporal_id_in_buffer_descriptions > 0 ) |  |
| temporal_id_positive[i] | u(v) |
| } |  |
| } |  |
| if( number_of_negative_pictures + number_of_positive_pictures < max_num_ref_frames ) { |  |
| combine_with_reference_flag | u(1) |
| if( combine_with_reference_flag ) |  |
| bd_combination_idx | u(v) |
| } |  |
| } |  |
| } |  |

A derived list POCBD_pps[bd_idx] is created by concatenating the negative deltaPOC and positive deltaPOC entries from buffer description with index bd_idx in the picture parameter set and converting them to POC values.

bd_poc_delete_count specifies the number of entries to be deleted from the list POCBD_pps[bd_idx] for the current picture.

bd_pps_delete_idx[j] specifies that for the current picture the POCBD_pps[bd_idx][bd_pps_delete_idx[j]] entry is deleted.

bd_poc_ append_count specifies the number of entries to be appended to lists: POCBD_pps[bd_idx] for the current picture.

poc_pps_append[j] specifies the POC value to be appended to the list POCBD_pps[bd_idx] for the current picture.

The POCBD_pps list is then used to initialize the POCBD list of the current picture.

# Conclusion

The document proposed syntax to delete or add entries within a reference picture set defined in the picture parameter set. This allows efficient reference picture signaling when only a small subset of reference pictures differ from those defined within a reference picture set in the picture parameter set.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] R. Sjöberg, J. Samuelsson (Ericsson), "Absolute signaling of reference pictures," JCTVC-F493/m20923 6th Meeting: Torino, IT, 14-22 July, 2011.

[2] R. Sjöberg, J. Samuelsson  (Ericsson), "JCT-VC AHG report: xxxx (AHG21)” JCTVC-G021/mxxxx 7th Meeting: Geneva, CH, 21-30 November, 2011.

[3] ftp://ftp.hhi.de/ahg21/ JCTVC-F803_d1_Buffer_Descriptions_r0.doc Candidate Working Draft text by AHG 21