| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G718 |
| --- | --- |

| Title: | Context reduction for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This proposal reduces the number of contexts in CABAC. The number is reduced by 56 contexts. Experimental results reportedly show 0.00℅, -0.01℅ and 0.12℅ BD-rate changes in high efficiency intra-only, random access and low-delay test conditions respectively.

# Introduction

In this proposal, the number of CABAC contexts for syntax elements, pred_type, merge_idx, inter_pred_flag, ref_idx_lx, cbf_cb, cbf_cr, coeff_abs_level_greater1_flag, and coeff_abs_level_greater2_flag are investigated. The modifications reduce the number of contexts by 56 (around 16% of the total) with negligible coding efficiency changes.

# Proposed Solution

Each of the following sections described the changes made on the top of HM4.0 for each syntax elements. Results for each case are given using the common conditions. The results of the combination of the contexts reductions is provided at the end.

## Pred_type, and Asymmetric partition

### Pred_type

In HM4.0, different binarizations on syntax elements, pred_type, are used in P and B slice as shown in Table 1. The proposed method unifies different binarizations by removing the binarization used in P slice as shown in Table 2. Table 3 shows the coding performance impact on P slice under common test condition [1].

| Slice type | Value of pred_type | PredMode | PartMode | Bin string |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cLog2CUSize > Log2MinCUSize | cLog2CUSize = = Log2MinCUSize |  |
|  |  |  |  |  | cLog2CUSize = = 3 && !inter_4x4_enabled_flag | cLog2CUSize > 3 ¦ ¦ inter_4x4_enabled_flag |
| I | 0 | MODE_INTRA | PART_2Nx2N | - | 1 | 1 |
|  | 1 | MODE_INTRA | PART_NxN | - | 0 | 0 |
| P | 0 | MODE_INTER | PART_2Nx2N | 0 1 | 0 1 | 0 1 |
|  | 1 | MODE_INTER | PART_2NxN | 0 011 | 0 01 | 0 01 |
|  | 2 | MODE_INTER | PART_Nx2N | 0 0011 | 0 00 | 0 001 |
|  | 4 | MODE_INTER | PART_2NxnU | 0 0100 | - | - |
|  | 5 | MODE_INTER | PART_2NxnD | 0 0101 | - | - |
|  | 6 | MODE_INTER | PART_nLx2N | 0 00100 | - | - |
|  | 7 | MODE_INTER | PART_nRx2N | 0 00101 | - | - |
|  | 3 | MODE_INTER | PART_NxN | - | - | 0 000 |
|  | 4 | MODE_INTRA | PART_2Nx2N | 1 | 11 | 11 |
|  | 5 | MODE_INTRA | PART_NxN | - | 10 | 10 |
| B | 0 | MODE_INTER | PART_2Nx2N | 1 | 1 | 1 |
|  | 1 | MODE_INTER | PART_2NxN | 011 | 01 | 01 |
|  | 2 | MODE_INTER | PART_Nx2N | 0011 | 001 | 001 |
|  | 4 | MODE_INTER | PART_2NxnU | 0100 | - | - |
|  | 5 | MODE_INTER | PART_2NxnD | 0101 | - | - |
|  | 6 | MODE_INTER | PART_nLx2N | 00100 | - | - |
|  | 7 | MODE_INTER | PART_nRx2N | 00101 | - | - |
|  | 3 | MODE_INTER | PART_NxN | - | - | 0001 |
|  | 4 | MODE_INTRA | PART_2Nx2N | 000 | 000 0 | 0000 0 |
|  | 5 | MODE_INTRA | PART_NxN | - | 000 1 | 0000 1 |

Table 1 Binarization for pred_type in HM4.0

| Slice type | Value of pred_type | PredMode | PartMode | Bin string |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cLog2CUSize > Log2MinCUSize | cLog2CUSize = = Log2MinCUSize |  |
|  |  |  |  |  | cLog2CUSize = = 3 && !inter_4x4_enabled_flag | cLog2CUSize > 3 ¦ ¦ inter_4x4_enabled_flag |
| I | 0 | MODE_INTRA | PART_2Nx2N | - | 1 | 1 |
|  | 1 | MODE_INTRA | PART_NxN | - | 0 | 0 |
| P or B | 0 | MODE_INTER | PART_2Nx2N | 1 | 1 | 1 |
|  | 1 | MODE_INTER | PART_2NxN | 011 | 01 | 01 |
|  | 2 | MODE_INTER | PART_Nx2N | 0011 | 001 | 001 |
|  | 4 | MODE_INTER | PART_2NxnU | 0100 | - | - |
|  | 5 | MODE_INTER | PART_2NxnD | 0101 | - | - |
|  | 6 | MODE_INTER | PART_nLx2N | 00100 | - | - |
|  | 7 | MODE_INTER | PART_nRx2N | 00101 | - | - |
|  | 3 | MODE_INTER | PART_NxN | - | - | 0001 |
|  | 4 | MODE_INTRA | PART_2Nx2N | 000 | 000 0 | 0000 0 |
|  | 5 | MODE_INTRA | PART_NxN | - | 000 1 | 0000 1 |

Table 2 Binarization for pred_type in proposed method

| BD-rate | Low delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.02% | 0.16% | 0.26% |
| Class C | 0.01% | 0.05% | -0.12% |
| Class D | -0.02% | -0.10% | -0.12% |
| Class E | 0.02% | 0.03% | 0.05% |
| All | 0.01% | 0.04% | 0.03% |
| Enc T[%] | 101% |  |  |
| Dec T[%] | 101% |  |  |

Table 3.Coding performance for unified binarization on pred_type

### Asymmetric partitions

When asymmetric partitions are enabled, four contexts, equally divided into two context sets are used on the last two bins for signalling the pred_type of asymmetric partitions, i.e. PART_2NxnU, PART_2NxnD, PART_nLx2N, PART_nRx2N. One context set is applied depending on the partition being divided along the horizontal direction or the vertical direction.  The second last bin is used to specify whether the current CU has symmetric partitions or asymmetric partitions. The last bin is to specify whether the size of the first partition is one-fourth or three-fourth of the CU size.

In the proposed solution, one context is used on the second last bin and bypass mode is used on the last bin. As a result, context number is reduced from 4 to 1. Table 4 shows the coding performance associate to the proposed modifications.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  | 0.03% | -0.01% | -0.17% |  |  |  |
| Class B |  |  |  | 0.02% | -0.03% | 0.05% | 0.01% | 0.00% | -0.24% |
| Class C |  |  |  | -0.01% | -0.03% | -0.02% | -0.01% | -0.03% | 0.02% |
| Class D |  |  |  | 0.01% | 0.07% | -0.05% | 0.01% | 0.06% | 0.03% |
| Class E |  |  |  |  |  |  | 0.00% | 0.30% | 0.39% |
| All |  |  |  | 0.01% | 0.00% | -0.04% | 0.00% | 0.06% | 0.01% |
| Enc T[%] |  |  |  | 101% |  |  | 99% |  |  |
| Dec T[%] |  |  |  | 100% |  |  | 99% |  |  |

Table 4. Coding performance of the proposed method on pred_type

## Merge Index

In HM4.0, one of the five merge candidates are signalled when the current PU is in merge mode. A unary code is utilized to represent the syntax element merge_idx. For CABAC, each bin uses one context. In the proposed method, one context is repeatedly used in all four bins. Therefore, the number of contexts is reduced by 3. Table 5 shows the coding performance associate to the proposed modifications.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  | 0.04% | -0.21% | -0.15% |  |  |  |
| Class B |  |  |  | 0.00% | -0.03% | 0.03% | 0.00% | -0.07% | 0.01% |
| Class C |  |  |  | 0.02% | -0.04% | 0.00% | 0.00% | -0.12% | 0.03% |
| Class D |  |  |  | 0.07% | -0.11% | -0.04% | 0.03% | 0.35% | 0.02% |
| Class E |  |  |  |  |  |  | -0.02% | 0.10% | 0.91% |
| All |  |  |  | 0.03% | -0.10% | -0.04% | 0.00% | 0.05% | 0.18% |
| Enc T[%] |  |  |  | 99% |  |  | 99% |  |  |
| Dec T[%] |  |  |  | 100% |  |  | 100% |  |  |

Table 5. Coding performance of the proposed method on merge_idx

## Inter prediction flag

Inter prediction flag specifies whether uni-prediction or bi-prediction is used for the current PU. Currently, the context index is equal to the current CU depth. In the proposed method, the context index is equal to the current CU depth and capped at 2. As a result, 3 contexts are used to code the syntax element inter_pred_flag instead of 4. The proposed method also modifies the CABAC initialization table for inter_pred_flag. For fair comparsion, table 6 shows the coding performance on the initialization table changes while the number of contexts is not changed. Table 7 shows the coding performance of the proposed method reducing the number of contexts from 4 to 3.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  | 0.03% | -0.15% | -0.11% |  |  |  |
| Class B |  |  |  | -0.03% | 0.01% | -0.03% | 0.03% | -0.02% | -0.11% |
| Class C |  |  |  | 0.00% | -0.12% | 0.06% | -0.03% | -0.16% | 0.01% |
| Class D |  |  |  | -0.01% | -0.04% | 0.01% | -0.09% | 0.51% | 0.20% |
| Class E |  |  |  |  |  |  | 0.10% | -0.03% | 0.65% |
| All |  |  |  | -0.01% | -0.07% | -0.02% | 0.00% | 0.07% | 0.14% |
| Enc T[%] |  |  |  | 99% |  |  | 99% |  |  |
| Dec T[%] |  |  |  | 98% |  |  | 100% |  |  |

Table 6. Coding performance of HM4.0 with modified CABAC initialization on inter_pred_flag.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  | 0.05% | -0.11% | -0.14% |  |  |  |
| Class B |  |  |  | -0.01% | -0.03% | 0.02% | 0.00% | -0.01% | -0.15% |
| Class C |  |  |  | -0.02% | -0.14% | -0.02% | 0.01% | 0.01% | -0.03% |
| Class D |  |  |  | 0.03% | -0.01% | -0.01% | -0.09% | -0.12% | -0.01% |
| Class E |  |  |  |  |  |  | -0.12% | 0.08% | 0.45% |
| All |  |  |  | 0.01% | -0.07% | -0.03% | -0.04% | -0.01% | 0.03% |
| Enc T[%] |  |  |  | 100% |  |  | 99% |  |  |
| Dec T[%] |  |  |  | 99% |  |  | 100% |  |  |

Table 7. Coding performance of the proposed method on inter_pred_flag.

## Reference frame Index

Reference frame index is signalled by using a truncated unary code with respect to the active reference frame in the associate list. Three contexts are used on bin 0, bin 1, and the rest of the bins, respectively. In the proposed method, two contexts are used, one on bin 0 and another on the rest of the bins. Table 8 shows the coding performance associate to the proposed modifications.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  | -0.01% | -0.11% | -0.16% |  |  |  |
| Class B |  |  |  | -0.01% | 0.00% | -0.01% | 0.01% | -0.12% | 0.01% |
| Class C |  |  |  | -0.01% | 0.02% | 0.03% | -0.04% | -0.14% | -0.07% |
| Class D |  |  |  | 0.03% | 0.06% | 0.11% | -0.06% | 0.19% | -0.09% |
| Class E |  |  |  |  |  |  | -0.06% | -0.34% | 0.48% |
| All |  |  |  | 0.00% | -0.01% | -0.01% | -0.03% | -0.09% | 0.05% |
| Enc T[%] |  |  |  | 99% |  |  | 100% |  |  |
| Dec T[%] |  |  |  | 99% |  |  | 100% |  |  |

Table 8. Coding performance of the proposed method on ref_idx_lx.

## Chroma coded block flag

For syntax elements cbf_cb and cbf_cr, two different context sets (5 contexts in each context set) are used and the index is equal to the current transform depth. In the proposed method,  cbf_cb and cbf_cr shares one context set and the index is equal to the current transform depth. Therefore, the contexts reduction is 5. Table 9 shows the coding performance associate to the proposed modifications.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.01% | 0.59% | -1.06% | 0.02% | 0.56% | -1.71% |  |  |  |
| Class B | 0.00% | 0.59% | -1.07% | -0.01% | 0.68% | -1.32% | 0.01% | 1.06% | -1.89% |
| Class C | -0.01% | 0.17% | -0.75% | 0.00% | 0.09% | -0.63% | -0.01% | 0.21% | -0.97% |
| Class D | 0.00% | -0.17% | -0.51% | 0.04% | -0.23% | -0.80% | 0.04% | -0.36% | -0.45% |
| Class E | 0.00% | -0.36% | 0.24% |  |  |  | 0.04% | 0.36% | 0.40% |
| All | 0.00% | 0.21% | -0.70% | 0.01% | 0.30% | -1.13% | 0.02% | 0.36% | -0.87% |
| Enc T[%] | 98% |  |  | 98% |  |  | 95% |  |  |
| Dec T[%] | 100% |  |  | 99% |  |  | 99% |  |  |

Table 9. Coding performance of the proposed method on cbf_cb and cbf_cr.

## Coefficient level coding

In current HM4.0, there are twelve context sets for both coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. The context sets are equally assigned for luma and chroma component, i.e. 6 context sets for luma and 6 contexts for chroma. Each context set consists of 5 contexts. The index  of the context set, ctxSet, is selected based on the previous coeff_abs_level_greater1_flag. For coeff_abs_level_greater1_flag, the index of the context within a context set, greater1Ctx, is determined based on the trailing ones to a maximum of 4. The context index can be represented as:

ctxIdx_level_greater1  =  ( ctxSet  *  5 )  +  Min( 4, greater1Ctx )           (1)

For coeff_abs_level_greater2_flag,  the index of the context within a context set, greater2Ctx, is based on the number of coeff_abs_level_greater1_flag being 1 to a maximum of 4. The context index can be represented as:

ctxIdx_level_greater2  =  ( ctxSet  *  5 )  +  Min( 4, greater2Ctx )           (2)

In this proposal, the context sets have been set to better match the bin usage, so context sets have different number of the contexts. For coeff_abs_level_greater1_flag, context set 0 and 3 have 5 contexts and context set 1, 2, 4 and 5 have each 2 contexts. For coeff_abs_level_greater2_flag, context set 0, 1, and 2 have 5 contexts and context set 3, 4, and 5 have 2 contexts. This method can be represented as:

ctxIdx_level_greater1  =  ( ctxSet  *  5 )  +  Min( Thres_ greater1 , greater1Ctx )                               (3)

if ctxSet =0 or ctxSet =3, Thres_ greater1 = 4;

otherwise, Thres_ greater1 = 1.

ctxIdx_level_greater2  =  ( ctxSet  *  5 )  +  Min(Thres_ greater2, greater2Ctx )                                 (4)

if ctxSet  <3, Thres_ greater2 = 4;

otherwise, Thres_ greater2 = 1

The CABAC initialization tables of coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag are also modified for the context sets that Thres_ greater1 or Thres_ greater2 equal to 1. The modifications move the initialization of the fifth context ahead to be the initialization of the second context.

The proposed method reduces the number of the contexts from 120 to 78, that is, a reduction of 35% of contexts while keeping the same performance, as Table 9 shows.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.04% | 0.01% | 0.06% | -0.37% | 0.05% |  |  |  |
| Class B | 0.02% | 0.03% | 0.02% | -0.02% | 0.08% | -0.10% | 0.01% | -0.05% | -0.10% |
| Class C | 0.00% | 0.04% | 0.02% | 0.03% | -0.04% | 0.10% | 0.03% | 0.05% | -0.08% |
| Class D | 0.01% | 0.00% | -0.04% | 0.07% | 0.33% | -0.06% | 0.02% | -0.36% | -0.12% |
| Class E | 0.01% | -0.05% | -0.02% |  |  |  | 0.07% | -0.67% | 0.54% |
| All | 0.01% | 0.00% | 0.00% | 0.03% | 0.00% | -0.01% | 0.03% | -0.22% | 0.02% |
| Enc T[%] | 100% |  |  | 101% |  |  | 98% |  |  |
| Dec T[%] | 100% |  |  | 98% |  |  | 100% |  |  |

Table 9. Coding performance of the proposed method on coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag.

## List of context reductions

Tables 10 lists the number of the contexts for all the syntax elements mentioned in previous sections. The total reduction is 56 contexts.

| # of Contexts | HM4.0 | Proposed method |
| --- | --- | --- |
| pred_type | 10 | 6 |
| merge_idx | 4 | 1 |
| inter_pred_flag | 4 | 3 |
| ref_idx_lc, ref_idx_l0, ref_idx_l1 | 3 | 2 |
| cbf_cb, cbf_cr | 10 | 5 |
| coeff_abs_level_greater1_flag | 60 | 36 |
| coeff_abs_level_greater2_flag | 60 | 42 |
| Total | 151 | 95 |

Table 10. Comparison of number of contexts in the proposed method and HM4.0

# Simulation Results

This method is implemented in HM4.0. Simulation results are presented for the high efficiency common test conditions [1]. Table 11 shows the results for the modifications from 2.1 to 2.6 combined. The BD-rate is 0.00℅, 0.01℅ and 0.13℅ for AI, RA and LB configurations, respectively. Table 12 shows the results combining the modifications described in sections 2.1 to 2.7. The BD-rate is 0.00℅, -0.01℅ and 0.12℅ for AI, RA and LB configurations, respectively.

## Combined results on 2.1-2.5

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.01% | 0.59% | -1.06% | 0.03% | 0.51% | -1.90% |  |  |  |
| Class B | 0.00% | 0.59% | -1.07% | -0.04% | 0.72% | -1.32% | -0.05% | 1.05% | -1.63% |
| Class C | -0.01% | 0.17% | -0.75% | 0.00% | 0.00% | -0.64% | -0.09% | 0.02% | -1.06% |
| Class D | 0.00% | -0.17% | -0.51% | 0.04% | -0.31% | -0.87% | -0.13% | -0.46% | -0.44% |
| Class E | 0.00% | -0.36% | 0.24% |  |  |  | -0.31% | 0.54% | 0.62% |
| All | 0.00% | 0.21% | -0.70% | 0.01% | 0.26% | -1.19% | -0.13% | 0.32% | -0.77% |
| Enc T[%] | 98% |  |  | 98% |  |  | 98% |  |  |
| Dec T[%] | 100% |  |  | 99% |  |  | 100% |  |  |

Table 11. Coding performance of the proposed method on sections 2.1-2.5.

## Combined results on 2.1-2.6

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.59% | -1.10% | 0.00% | 0.74% | -0.73% |  |  |  |
| Class B | 0.01% | 0.58% | -1.06% | -0.04% | 0.74% | -1.40% | -0.04% | 1.17% | -1.86% |
| Class C | -0.01% | 0.13% | -0.74% | -0.01% | 0.03% | -0.79% | -0.10% | 0.05% | -0.89% |
| Class D | 0.00% | -0.19% | -0.55% | 0.02% | -0.14% | -0.78% | -0.10% | -0.33% | -0.95% |
| Class E | 0.01% | -0.43% | 0.20% |  |  |  | -0.33% | 0.75% | 1.02% |
| All | 0.00% | 0.19% | -0.71% | -0.01% | 0.37% | -0.95% | -0.12% | 0.43% | -0.85% |
| Enc T[%] | 99% |  |  | 99% |  |  | 98% |  |  |
| Dec T[%] | 100% |  |  | 99% |  |  | 100% |  |  |

Table 12. Coding performance of the proposed method on sections 2.1-2.6.

# Conclusions

The proposal investigates the CABAC context modelling of several syntax elements in HM4.0. 56 contexts are removed from the current scheme without sacrificing the coding performance. Based on the results, we suggest including this modification into HM.

# References

[1] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).