| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G721 |
| --- | --- |

| Title: | CE4 Subtest 1.1.a: QP adaptation at sub-CU level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xue Fang Jae Hoon Kim Krit Panusopone Limin Wang Luthra Ajay | Tel: Email: | +1-858-404-2723 X.Fang@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document describes implementation details and reports the results of Motorola Mobility’s QP adaptation at sub-CU level. Two versions of HEVC syntax are also described. Simulation results show that there is a loss between 0.2% and 1.0% for intra, between 0.3% and 1.0% for low delay and between 0.3% and 1.6% for random access conditions respectively. The QP modulation of sub-CU is based on existing activity calculation in HM4.0.  Cross-Check will be provided by Ericsson and NEC.

# Introduction

QP adaptation at sub-CU level was proposed [1] to allow finer QP granularity in HEVC and is included in subtest 1.1 of CE4 [2]. In this proposal, a hybrid of sub-LCU [3] and sub-CU (TU) dQP signaling is implemented. This hybrid method uses the same QP minimum size (QpMinCuSize) [4], max_cu_qp_delta_depth in HM4.0, to control dQP signaling for both CUs and TUs. When depths of leaf CUs are larger than or equal to max_cu_qp_delta_depth, only single dQP will be signaled the same way as sub-LCU dQP control. If depth of a leaf CU is below the max_cu_qp_delta_depth, a TU can signal its own dQP if the full depth (CU depth + transform depth) of that TU is less than the given threshold. Same as sub-LCU, leaf TUs will share a single TU dQP when their full depths are over the max_cu_qp_delta_depth threshold.

# Implemented dQP Signaling Method

## Algorithm

The delta QP for each TU (dQPTU) in a CU is signaled, if all of the following conditions are met:

- CU depth (CUdepth) is smaller than max_cu_tu_qp_delta_depth;

- TU has at least one coefficient in either Luma,Y or Chroma U or V;

- The full depth, which is CU depth plus transform depth (Trdepth) of a TU, is smaller than or equal to max_cu_tu_qp_delta_depth. Or the full depth of a TU is larger than max_cu_tu_qp_delta_depth, but it’s the first TU which has at least one nonzero coefficient in either Luma or Chroma.

The final QP for the TU, QPTU, is derived from dQPTU and predicted QPp-TU as follows;

QPTU=QPP−TU+dQPTU                                                                        (1)

The method for QPp-TU prediction is the same used in current HM4.0. Table1 is the brief summary of implemented dQP signaling method.

Table 1: Sub-CU based dQP Signaling With Various Configurations

| max_cu_qp_delta_depth | CU | TU |
| --- | --- | --- |
| 0 | LCU dQP only |  |
| A | if ( depth >= A ) shared sub-LCU dQP | if ((CUdepth + Trdepth) < A) Own sub-CU dQP if ((CUdepth + Trdepth) >= A ) Shared sub-CU dQP |

## Syntax

To minimize the impact of code, there are total two places changed in HM4.0 based syntax. The flag cu_qp_delta_enabled_flag in seq_parameter_set_rbsp() is changed to cu_tu_qp_delta_enabled_flag  in Table 2 with possible setting in Table 3. Highlighted yellow shows changes or new added syntax.

#### Table 1. Example of proposed syntax in Sequence parameter set RBSP syntax in HM

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| … |  |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| adaptive_loop_filter_enabled_flag | u(1) |
| cu_tu_qp_delta_enabled_flag | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Table 3: Example of cu_tu_qp_delta_enabled_flag

| cu_tu_qp_delta_enabled_flag | level of delta QP |
| --- | --- |
| 0 | no delta QP |
| 1 | delta QP at sub-LCU |
| 2 | delta QP at sub-LCU and sub-CU (TU) sub-LCU (only if CU depth is equal to or larger than max_cu_tu_qp_delta_depth) |

In this CE test version, the position of cu_qp_delta is kept inside transform_coeff().  A new tu_delta_qp() function unit in Table4 is added for TU dQP signaling.

Table 4: Example of proposed syntax of tu_qp_delta in HM

| tu_qp_delta_unit ( x0, y0, log2TrafoSize, trafoDepth ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if(log2CUSize >= log2MinCUDQPSize ) |  |
| IsTuQpDeltaCoded = 0 |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| tu_qp_delta( x0, y0, log2TrafoSize − 1, trafoDepth + 1) |  |
| tu_qp_delta ( x1, y0, log2TrafoSize − 1, trafoDepth + 1) |  |
| tu_qp_delta ( x0, y1, log2TrafoSize − 1, trafoDepth + 1) |  |
| tu_qp_delta ( x1, y1, log2TrafoSize − 1, trafoDepth + 1) |  |
| } else { |  |
| if(!IsTuQpDeltaCoded ) { |  |
| tu_qp_delta | se(v) ¦ ae(v) |
| IsTuQpDeltaCoded = 1 |  |
| } |  |
| } |  |
| } |  |
| } |  |

The last change is inside the coding_unit() Table 5 where the tu_delta_qp() is signaled. Two versions are evaluated in this subtest. The first version places dQP syntax after transform coefficient syntax. The second version places dQP syntax before transform coefficient syntax in consistent with the current CU dQP position which puts dQP of the CU ahead of any coefficients [5].  As we can see from two cross checks, only CABAC based dQP signaling shows very minor difference because of the different statistic model.

Table 5: Example of proposed syntax in coding unit syntax in HM

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 ) |  |
| prediction_unit( x0, y1 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 ) |  |
| prediction_unit( x1, y0 ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 ) |  |
| prediction_unit( x0, y2 ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 ) |  |
| prediction_unit( x0, y3 ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 ) |  |
| prediction_unit( x2, y0 ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 ) |  |
| prediction_unit( x3, y0 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 ) |  |
| prediction_unit( x1, y0 ) |  |
| prediction_unit( x0, y1 ) |  |
| prediction_unit( x1, y1 ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| if( tu_qp_delta_enabled_flag == 2 && log2CUSize > log2MinCUDQPSize) |  |
| tu_qp_delta_unit ( x0, y0, log2CUSize,0) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

# Simulation Results

The proposed method was implemented in the HM-4.0 and tested to evaluate its coding performance and complexity. The binaries were compiled using Microsoft Visual Studio 2010 64 bits. The coding conditions used for the simulations are specified in [2]. Three sets of results, as shown in Tables 3-6, are reported for three different MaxCUdQPdepth parameters; i.e., 1, 2, and 3, respectively. Detailed results are included in the attachment (JCTVC-G721.xls).

Table 3: Coding performance and complexity of the proposed method with MaxCUdQPdepth set to 1

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.6% | 1.4% | 0.9% | 15.6% | 0.8% | 0.7% | -0.1% | 14.4% |
| Class B | 0.2% | 0.5% | 0.7% | 9.1% | 0.4% | 0.2% | 0.1% | 8.0% |
| Class C | 0.0% | 0.1% | 0.1% | 1.2% | 0.0% | 0.0% | 0.0% | 0.6% |
| Class D | 0.0% | 0.1% | 0.1% | 0.8% | 0.0% | 0.0% | 0.0% | 0.3% |
| Class E | 0.1% | 0.6% | 0.4% | 3.8% | 0.1% | 0.0% | 0.0% | 2.0% |
| Class F | 0.0% | 0.1% | 0.2% | 1.4% | 0.0% | 0.0% | 0.1% | 0.7% |
| Overall | 0.2% | 0.5% | 0.4% | 5.6% | 0.2% | 0.2% | 0.0% | 4.6% |
|  | 0.2% | 0.5% | 0.4% |  | 0.2% | 0.2% | 0.0% |  |
| Enc Time[%] | 106% |  |  |  | 118% |  |  |  |
| Dec Time[%] | 84% |  |  |  | 103% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 1.0% | 0.4% | 0.9% | 17.3% | 0.9% | 0.2% | -0.2% | 13.1% |
| Class B | 0.3% | 0.4% | 0.6% | 7.5% | 0.3% | 0.2% | 0.2% | 6.6% |
| Class C | 0.1% | 0.0% | 0.0% | 2.9% | 0.1% | 0.2% | 0.1% | 2.5% |
| Class D | 0.0% | 0.2% | 0.2% | 0.6% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | 0.2% | 0.8% | 0.7% | 5.8% | 0.2% | 0.6% | 0.7% | 4.9% |
| Overall | 0.3% | 0.4% | 0.5% | 6.0% | 0.3% | 0.2% | 0.2% | 4.8% |
|  | 0.3% | 0.3% | 0.5% |  | 0.3% | 0.2% | 0.2% |  |
| Enc Time[%] | 109% |  |  |  | 110% |  |  |  |
| Dec Time[%] | 82% |  |  |  | 83% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.5% | 0.6% | 0.5% | 8.4% | 0.5% | 0.6% | 0.3% | 7.1% |
| Class C | 0.2% | 0.0% | -0.2% | 4.1% | 0.1% | 0.2% | 0.1% | 3.5% |
| Class D | 0.1% | 0.0% | -0.2% | 0.6% | 0.0% | 0.0% | -0.4% | 0.5% |
| Class E | 0.7% | 2.8% | 1.2% | 5.9% | 0.5% | 2.4% | 1.6% | 5.4% |
| Class F | 0.4% | 1.1% | 0.6% | 8.6% | 0.4% | 0.9% | 0.8% | 6.8% |
| Overall | 0.4% | 0.8% | 0.3% | 4.6% | 0.3% | 0.7% | 0.4% | 4.0% |
|  | 0.4% | 0.7% | 0.4% |  | 0.3% | 0.7% | 0.4% |  |
| Enc Time[%] | 112% |  |  |  | 119% |  |  |  |
| Dec Time[%] | 85% |  |  |  | 90% |  |  |  |

Table 4: Coding performance and complexity of the proposed method with MaxCUdQPdepth set to 2

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 0.7% | 2.3% | 1.8% | 18.7% | 1.0% | 0.9% | 0.2% | 20.4% |
| Class B | 0.4% | 1.1% | 1.4% | 10.4% | 0.6% | 0.2% | 0.2% | 11.1% |
| Class C | 0.0% | 0.4% | 0.5% | 2.1% | 0.1% | 0.0% | 0.0% | 1.8% |
| Class D | 0.0% | 0.3% | 0.2% | 1.5% | 0.1% | 0.0% | 0.0% | 0.9% |
| Class E | 0.3% | 1.9% | 1.4% | 5.6% | 0.5% | 0.2% | 0.1% | 5.7% |
| Class F | 0.0% | 0.6% | 0.2% | 2.1% | 0.2% | 0.0% | 0.1% | 2.5% |
| Overall | 0.2% | 1.1% | 0.9% | 7.0% | 0.4% | 0.2% | 0.1% | 7.4% |
|  | 0.2% | 1.1% | 1.0% |  | 0.4% | 0.2% | 0.1% |  |
| Enc Time[%] | 106% |  |  |  | 91% |  |  |  |
| Dec Time[%] | 84% |  |  |  | 78% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 1.1% | 0.8% | 1.2% | 22.3% | 1.1% | -0.1% | -1.2% | 20.9% |
| Class B | 0.5% | 1.0% | 1.6% | 10.9% | 0.6% | 0.3% | 0.4% | 10.9% |
| Class C | 0.0% | 0.4% | 0.3% | 3.6% | 0.0% | 0.1% | 0.0% | 3.1% |
| Class D | -0.1% | 0.0% | -0.2% | 0.7% | -0.3% | -0.4% | -0.3% | 0.2% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | 0.1% | 0.9% | 0.6% | 5.4% | -0.1% | 0.3% | 0.1% | 6.0% |
| Overall | 0.3% | 0.6% | 0.7% | 7.6% | 0.3% | 0.0% | -0.2% | 7.3% |
|  | 0.3% | 0.6% | 0.7% |  | 0.3% | 0.0% | -0.2% |  |
| Enc Time[%] | 110% |  |  |  | 110% |  |  |  |
| Dec Time[%] | 82% |  |  |  | 84% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.0% | 2.0% | 2.4% | 12.7% | 1.1% | 1.3% | 1.3% | 12.7% |
| Class C | 0.1% | 0.6% | 0.3% | 4.4% | 0.1% | 0.4% | 0.5% | 4.8% |
| Class D | -0.3% | -0.3% | -0.2% | 0.7% | -0.4% | -0.4% | -0.2% | 0.5% |
| Class E | 1.5% | 4.6% | 3.2% | 10.7% | 2.1% | 4.2% | 3.8% | 14.5% |
| Class F | 0.3% | 0.7% | 0.9% | 10.0% | 0.1% | 0.7% | 0.7% | 11.1% |
| Overall | 0.5% | 1.4% | 1.3% | 6.5% | 0.5% | 1.1% | 1.1% | 7.3% |
|  | 0.5% | 1.4% | 1.3% |  | 0.5% | 1.1% | 1.1% |  |
| Enc Time[%] | 114% |  |  |  | 119% |  |  |  |
| Dec Time[%] | 86% |  |  |  | 90% |  |  |  |

Table 6: Coding performance and complexity of the proposed method with MaxCUdQPdepth set to 3

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 1.1% | 2.8% | 2.8% | 20.0% | 1.5% | 1.4% | 0.8% | 22.4% |
| Class B | 1.0% | 2.7% | 3.5% | 15.3% | 1.4% | 1.0% | 1.1% | 17.8% |
| Class C | 0.4% | 2.6% | 3.1% | 7.2% | 0.7% | 0.7% | 0.8% | 7.6% |
| Class D | 0.3% | 2.1% | 2.3% | 6.0% | 0.5% | 0.5% | 0.5% | 5.8% |
| Class E | 1.0% | 4.0% | 3.2% | 9.8% | 1.4% | 1.4% | 1.2% | 12.6% |
| Class F | 0.4% | 1.7% | 2.1% | 5.2% | 0.8% | 0.9% | 1.0% | 6.6% |
| Overall | 0.7% | 2.6% | 2.8% | 10.9% | 1.0% | 1.0% | 0.9% | 0.0% |
|  | 0.7% | 2.6% | 2.8% |  | 1.0% | 1.0% | 0.9% |  |
| Enc Time[%] | 108% |  |  |  | 113% |  |  |  |
| Dec Time[%] | 94% |  |  |  | 95% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | 1.6% | 0.9% | 0.8% | 20.3% | 1.6% | 0.3% | -0.1% | 20.7% |
| Class B | 1.3% | 2.6% | 3.8% | 15.5% | 1.4% | 0.9% | 1.0% | 15.7% |
| Class C | 0.7% | 1.8% | 2.0% | 9.5% | 0.8% | 0.6% | 0.8% | 9.4% |
| Class D | 0.5% | 2.0% | 2.1% | 9.2% | 0.6% | 0.5% | 0.4% | 8.3% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | 0.6% | 2.0% | 2.1% | 7.9% | 0.6% | 1.2% | 1.1% | 8.6% |
| Overall | 0.9% | 1.9% | 2.2% | 11.1% | 1.0% | 0.7% | 0.6% | 11.1% |
|  | 0.9% | 1.8% | 2.2% |  | 1.0% | 0.7% | 0.7% |  |
| Enc Time[%] | 106% |  |  |  | 111% |  |  |  |
| Dec Time[%] | 81% |  |  |  | 88% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.9% | 4.4% | 5.5% | 16.0% | 1.9% | 2.3% | 2.1% | 13.8% |
| Class C | 1.1% | 2.4% | 3.0% | 10.5% | 1.2% | 1.9% | 1.8% | 10.1% |
| Class D | 1.0% | 3.1% | 3.5% | 11.8% | 1.1% | 1.5% | 0.8% | 10.1% |
| Class E | 2.1% | 6.2% | 4.3% | 11.6% | 3.0% | 5.1% | 4.1% | 17.1% |
| Class F | 1.0% | 2.2% | 1.8% | 13.6% | 1.1% | 2.0% | 1.2% | 14.1% |
| Overall | 1.4% | 3.6% | 3.7% | 10.8% | 1.6% | 2.4% | 1.9% | 10.8% |
|  | 1.4% | 3.5% | 3.6% |  | 1.6% | 2.5% | 1.9% |  |
| Enc Time[%] | 113% |  |  |  | 114% |  |  |  |
| Dec Time[%] | 86% |  |  |  | 89% |  |  |  |

# Conclusions

In this report, the performance of QP adaptation at sub-CU level is evaluated. The proposed sub-CU dQP signaling harmonizing with the adopted JCTVC-D038, JCTVC-D258, JCTVC-E051 and existing HM4.0  based QP prediction methods, provides a more flexible, potential better performance, better visual quality and less cost way for dQP signaling with minimum syntax and code changes.  Two version of syntax are implemented and their simulation results are reported. To minimize change to existing HEVC syntax, we recommend the HEVC adopts the proposed second syntax to allow QP adaptation at sub-CU level.

# References

- K. Panusopone, A. Luthra, X. Fang, L. Wang, “QP adaptation at sub_CU level,” JCTVC-F577

- K. Sato, M. Budagavi, M. Coban, H. Aoki, X. Li, “Description of Core Experiment 4: Quantization,” JCTVC-F904

- Madhukar Budagavi, Minhua Zhou, “Delta QP signaling at sub-LCU level” JCTVC-D038

- Tzu-Der Chuang , Ching-Yeh Chen , Yu-Lin Chang, Yu-Wen Huang, Shawmin Lei, “AhG Quantization: Sub-LCU Delta QP” JCTVC-E051

- L. Dong, W. Liu, K. Sato, “CU Adaptive Quantization Syntax Change for Better Decoder pipelining” JCTVC-D258

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).