| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G723 |
| --- | --- |

| Title: | Support for finer QP granularity in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Ajay Luthra Xue Fang Jae Hoon Kim Limin Wang | Tel: Email: | +1-858-404-3618 krit@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This contribution explains why sub-CU level QP adaptation needs to be supported in HEVC. Specifically, QP setting at the finer granularity improves visual quality, provides more accurate rate control, and allows greater flexibility in encoding strategy. It is recommended to support sub-CU level QP adaptation in HEVC.

# Introduction

QP adaptation is utilized in HEVC to support perceptual quantization and rate control. The current HM [1] allows QP to change at sub-TB level. Thus, all TUs within a sub-TB use the same QP and dQP is signaled when QP is changed from one sub-TB to another sub-TB [2]. The size of sub-TB area is set according to CU size. To control amount of dQP overhead, a global threshold is also used to determine the minimum size of sub-TB area [3]. This scheme used in HM assumes that the change inside a CU is negligible and the QP control at a sub-TB can capture content characteristics and provides a flexible buffer control.

In HM, CU can be of various sizes; e.g., 64x64, 32x32. Hence, QP adjustment at CU or larger may not be fast enough to respond to the change in content characteristics and buffer conditions. For example, 64x64 CU may select 2NxN PU type where two PU’s have very different characteristics, like one can be on the edge of an object and the other can be in the background. In this example, it will be beneficial to have the freedom to use different QPs for each PU to assign different bits to different objects thus achieving better quality and buffer control. This QP changes within a CU is not supported in the current HM. Therefore, it is proposed to allow QPs to be changed at a level finer than CU, for example at TU level.

# Benefits of QP adaptation at sub-CU level in HEVC

There are several benefits offered by the finer QP granularity. Compared to the QP granularity supported in the current HM, the QP adaptation at the sub-CU level in HEVC improves visual quality, provides more accurate rate control, reduces operation complexity, and allows greater flexibility in encoding strategy.

## Benefits in terms of visual quality control

Visual quality control can be improved with QP adaptation by using appropriate QP to match local characteristics. For example, bit allocation can be done to improve visual quality when flat area is set to use lower QP than busy area to take advantage of spatial masking.

By allowing area as small as TU to set its own QP, QP adaptation at sub-CU level improves visual quality control in two ways. First, this level of granularity matches the granularity of the quantization operation and hence the quantization of a TU can be performed independent of another TU. Second, among possible coding units, TU is a unit with the highest level of consistency in local characteristics, which makes issue with mixed flat area and busy area less likely.

## Benefits in terms of bitrate control

The goal of bit rate control is to provide a coded sequence with maximum quality in a given bit budget and a rate controller determines the target bits according to the buffer status. For example, if the buffer is expected to overflow, it changes QPs to reduce bits for coding units and if the buffer is expected to underflow, it also changes QPs to increase bits for coding units. At the high bit rates, CU size tends to be smaller and CU level QP adaptation may provide enough flexibility in rate control. However, at the low bit rates, CU size tends to be larger or close to LCU thus, CU level QP adaptation may not provide flexible bit rate control. The problem is that at the low bit rates, buffer size tends to be smaller and CU level QP control can be too slow to control overflow/underflow quickly.

By allowing area as small as TU to set its own QP, QP adaptation at sub-CU level improves bitrate control by allowing QP change at higher frequency.

## Benefits in terms of operation complexity

Operation complexity; e.g., power consumption, computation cycle, can be reduced with finer QP granularity. In HEVC code block structure, CU is at a higher level than TU and provides less coding efficiency and higher power consumption if forced to code a block of the same size as TU. For example, it requires more overhead to code an area with four 8x8 CUs each with one associated TU than with a 16x16 CU with four 8x8 TUs. By allowing area as small as TU to use an appropriate QP, CU can be coded at a larger size since it is no longer forced to split to accommodate varying QP.

Another difference between CU operations and TU operations is that TU is considered lightweight; i.e., change in TU coding mode produces less impact on its neighbors. Hence, it takes more computation cycles to handle the same amount of QP changes occurred at CU level than at TU level.

## Benefits in terms of encoder flexibility

Rate control is not specified in normative part of the standard and many rate control strategies are developed and utilized for video encoder in the past. Allowing QP to be changed at finer granularity provides greater flexibility in setting rate control strategy since encoder can change QP at any granularity from once every TU, to once every CU, to once every slice, or any combination of QP granularity.

One example of setting QP at different level of granularities is to set quantization parameter to be the same for all the TUs of the same size within the CUs. Encoder may choose this QP setting option if varying quantization at TU level increases bit rate and precise bitrate control is not critical.

Another example of rate control strategy is to minimize QP change within a slice. This strategy can be better accomplished with finer level of granularity since it provides more accurate boundary between two QP areas. Coarser level of granularity can be chosen in the flat area where QP change is unnecessary.

# Unified LCU, sub-LCU, and sub-CU Delta QP Signaling

Current proposals for QP signaling lack flexibility to achieve both sub-CU and sub-LCU due to the syntax limitation. Unified LCU, sub-LCU, and sub-CU delta QP signaling is possible with another depth control parameter, tu_delta_qp_depth, for sub-CU (TU) dQP signaling. This tu_delta_qp_depth is a relative depth starting from the root of CU. The overhead of the sub-CU (TU) adaptive Quantization Parameter (QP) can be controlled by setting the tu_delta_qp_depth independent of max_cu_qp_delta_depth as proposed in [6].

# Conclusion and Recommendations

Current approach in the standard will fix the quantization to be the same for all the units below CU level. It was shown that the characteristics of the local activity changes at a level finer than the CU size [5]. This behaviour can help improve encoder design in terms of improving visual quality, more accurate rate control, and greater flexibility in encoding strategy. Therefore, to be able to adapt quantization according the characteristics of the content, the flexibility of adjusting the quantization level at the level finer than CU is needed.

It is recommended to modify current HEVC syntax to support sub-CU level QP adaptation and adopt QP prediction method proposed in [4].

Consider doing further studies on unifying LCU, sub-LCU, and sub-CU dQP signalling as proposed in [6] is also recommended.

# References

[1] “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC document, JCTVC-E503, Mar. 2011.

[2] M. Shima, S. C. Lim, Y. W. Huang, C. S. Lim, K. Sato, M. Budagavi, “Support for Sub-LCU-Level QP in HEVC,” JCTVC-E202r1

[3] T. D. Chuang, C. Y. Chen, Y. L. Chang, Y. W. Huang, S. Lei, “AhG Quantization: Sub-LCU Delta QP,” JCTVC-E051r2

[4] X. Fang, J.-H. Kim, K. Panusopone, L. Wang, A. Luthra, “CE4 Subtest 1.1.a: QP adaptation at sub-CU level,” JCTVC-G721

[5] K. Panusopone, A. Luthra, X. Fang, L. Wang, “QP adaptation at sub_CU level,” JCTVC-F577

[6] X. Fang, A. Luthra, K. Panusopone, J.-H. Kim, L. Wang, “A Unified LCU, sub-LCU and sub-CU (TU) Delta QP Signaling,” JCTVC-G893

# Patent rights declaration(s)

Motorola Mobility Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).