| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G724-r1 |
| --- | --- |

| Title: | Entropy Coding for Non Square Blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Ying Kung Krit Panusopone 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2350 viviankung@motorola.com +1-858-404-3618 krit@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

Current HM4.0 [1] non square blocks are first mapped to square blocks before fed into the entropy encoder. The reverse operation of mapping the decoded square blocks to the non square blocks needs to be done at the decoding procedure. This contribution document proposes a modified transform coefficient coding for CABAC, which is able to handle both the square and the non-square blocks so that no mapping operations between square and non-square blocks are necessary. The proposed scheme is implemented based on wave front scan and it could be extended to other scan patterns. The experimental results show that there is a no BD-rate loss while saving the mapping time every time when non square blocks are used.

# Introduction

Since current HM code is optimized based on the assumption of square TU blocks, the recent adopted Non-Square-Quad-Tree (NSQT) [2] implementation in HM4.0 takes advantage of the square TU block coding by mapping the non-square 2Nx0.5N TU blocks to NxN TU blocks, which is a costly procedure required for both encoder and decoder. For example, at the encoder side, each 32x8 block requires to copy 256 positions of the coefficients one by one to form a 16x16 block while each 16x4 block requires to copy 64 positions of the coefficients to form an 8x8 block. At the decoder, each 32x8 non square block is formed for reconstruction by copying 256 positions of coefficients from a 16x16 square block, and each 16x4 non square block is formed for reconstruction by copying 64 positions of coefficients from an 8x8 square block.

# Entropy Coding for Non Square Blocks

We propose a generalized transform coefficient coding process for the TU block (square or non square) without the costly mapping procedures. The detailed changes to the current HM transform coefficient coding process are listed as follows:

- Scanning order is set according to the TU block shape.  Fig 1 shows wave front scan for 16x4 and 4x16 TU blocks.

- Cabac coding of last position of the non zero coefficient is modified.  Current HM [3] entropy coder expects square TU blocks only.  Therefore, the context models of the last position (x, y) on both horizontal and vertical directions are all depending on the block width M.  For a non square TU block with size MxN, a reasonable context model for vertical direction should depend on the block height N.  Thus, we suggest two separate context model for x and y, each corresponds to its possible maximum size.

- Cabac coding of the significant map.  In the latest HM4.0 [3], larger blocks utilize the context model of each significant bit based on its location in the block and the neighboring coded significant bits. According to the locations, larger blocks are separated to three zones, which are low, median and high frequency zones.  In the low frequency zone, context model is selected based on the block width and the specific location in the block. In the medium and high frequency zone, context model is selected based on the neighboring coded significant bits from the same TU. Context model of medium and high frequency zone are stored separately. These arrangements may not be suitable for the non square TU blocks.  For an MxN TU, we propose to separate non square TU block into two zones.  The first zone covers both the low frequency zone and the medium frequency zone and use context model based on the neighboring coded significant bits from the same TU. The second zone covers both the high frequency zone and use context model based on the neighboring coded significant bits from the same TU. Context model of medium and high frequency zone are stored separately.

Fig. 1 Wave front for 16x4 and 4x16 TU

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. Detailed results are included in the attachment (JCTVC-G724.xls).

Table 1 Simulation results

| Random Access HE. Y; U; V. Class A; 0.00%; -0.14%; -0.15%. Class B; -0.02%; 0.07%; 0.10%. Class C; -0.02%; -0.10%; -0.01%. Class D; -0.04%; -0.10%; -0.16%. Class E. Class F; 0.03%; 0.02%; 0.01%. Overall; -0.01%; -0.04%; -0.04%. -0.01%; -0.06%; -0.04%. Enc Time[%]; 91.97%. Dec Time[%]; 85.14%. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.04% | 0.10% | 0.11% |
| Class C | -0.06% | -0.22% | -0.19% |
| Class D | -0.06% | 0.15% | 0.26% |
| Class E | 0.05% | -0.15% | 0.62% |
| Class F | -0.24% | -0.24% | 0.04% |
| Overall | -0.05% | -0.01% | 0.17% |
|  | 0.00% | -0.07% | 0.17% |
| Enc Time[%] | 92.65% |  |  |
| Dec Time[%] | 85.13% |  |  |

|  | Low delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.02% | 0.32% | 0.33% |
| Class C | -0.02% | 0.03% | -0.09% |
| Class D | 0.00% | -0.37% | -0.36% |
| Class E | 0.06% | -0.15% | -0.47% |
| Class F | -0.21% | -0.16% | -0.25% |
| Overall | -0.03% | -0.04% | -0.13% |
|  | -0.03% | -0.03% | -0.17% |
| Enc Time[%] | 84.69% |  |  |
| Dec Time[%] | 86.70% |  |  |

# References

[1] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

[2] Y. Yuan (Tsinghua), X. Zheng (HiSilicon), X. Peng (USTC), J. Xu (Microsoft), L. Liu (HiSilicon), Y. Wang (Tsinghua), X. Cao (Tsinghua), C. Lai (HiSilicon), J. Zheng (HiSilicon), Y. He (Tsinghua), H. Yu (Huawei), “CE2: Non-Square Quadtree Transform for symmetric motion partition”, JCTVC-F410, July 2011

[3] “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, July, 2011

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).