| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G725-r1 |
| --- | --- |

| Title: | Simplification of significant map coding for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Ying Kung Krit Panusopone 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2350 viviankung@motorola.com +1-858-404-3618 krit@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution document proposes to remove context model based on the block width and the specific location in the block for larger TU block. The experimental results show that there is not more than 0.06% loss in both intra and inter conditions.

# Introduction

In the latest HM4.0 [1], larger TU blocks utilize the context model of each significant bit based on its location in the block and the neighboring coded significant bits. According to the locations, larger blocks are separated to three zones, which are low, median and high frequency zones, as shown in Fig 1 (a).  In the low frequency zone, context model is selected based on the block width and the specific location in the block. In the medium and high frequency zone, context model is selected based on the neighboring coded significant bits from the same TU. Context model of medium and high frequency zone are stored separately.  This contribution proposes to simplify significant map coding by combining low and medium frequency zone and eliminate 3 context models for the low frequency zone.

# Reduce Context Models in CABAC Coding of Significant Map

For an MxN TU, we propose to separate a TU block into two zones, as shown in Fig 1 (b).  The first zone covers both the low frequency zone and the medium frequency zone and use context model based on the neighboring coded significant bits from the same TU. The second zone covers the high frequency zone and uses context model based on the neighboring coded significant bits from the same TU. Context model of medium and high frequency zone are stored separately.

3

3

1

1

1

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

2

(b)

2

(a)

Fig 1 Significant bit context zoning (a) 3 zones of HM4.0 (b) proposed 2 zones

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. Timing results are unreliable since anchor and test software were run on different computing platforms. Detailed results are included in the attachment (JCTVC-G725.xls).

Table 1 Simulation results

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.03% | 0.01% | 0.11% |
| Class B | 0.03% | 0.06% | 0.06% |
| Class C | 0.02% | 0.06% | 0.04% |
| Class D | 0.03% | 0.05% | 0.03% |
| Class E | 0.10% | -0.12% | 0.02% |
| Class F | 0.00% | -0.08% | 0.04% |
| Overall | 0.03% | 0.00% | 0.05% |
|  | 0.03% | -0.01% | 0.03% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 70% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.04% | -0.29% | -0.19% |
| Class B | 0.02% | 0.08% | 0.03% |
| Class C | 0.05% | -0.11% | 0.03% |
| Class D | 0.06% | 0.04% | -0.17% |
| Class E |  |  |  |
| Class F | -0.04% | 0.08% | -0.27% |
| Overall | 0.03% | -0.04% | -0.11% |
|  | 0.03% | -0.04% | -0.08% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 83% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.03% | -0.34% | -0.46% |
| Class C | 0.11% | -0.16% | -0.32% |
| Class D | 0.03% | 0.64% | 0.25% |
| Class E | 0.08% | -1.01% | -0.24% |
| Class F | -0.14% | -0.53% | 0.05% |
| Overall | 0.02% | -0.25% | -0.15% |
|  | 0.02% | -0.27% | -0.16% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 84% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.01% | 0.01% | -0.22% |
| Class C | 0.07% | -0.07% | -0.12% |
| Class D | 0.02% | -0.05% | -0.31% |
| Class E | 0.04% | -1.30% | -0.91% |
| Class F | 0.06% | -0.11% | -0.38% |
| Overall | 0.04% | -0.24% | -0.35% |
|  | 0.04% | -0.20% | -0.42% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 104% |  |  |

# References

[1] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0/

[2] “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, July, 2011

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).