| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G727 |
| --- | --- |

| Title: | CE3: Adaptive resolution on motion vector difference |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents a coding method to signal the resolution of the motion vector difference. The motion accuracy of the motion vector difference can be adaptively selected to be two different accuracy and signaled via a motion resolution flag. A joint coding of motion resolution flag and motion vector difference are also proposed.  Two adaptations, half-pel/quarter pixel and  quarter pixel/one-eighth pixel, are tested. Experimental results reportedly show 0.2℅ and 0.3% BD-rate saving on  the adaptation of half-pel/quarter pixel and quarter pixel/one-eighth pixel,  respectively. 2.5% BD-rate saving is observed for low delay P configuration with the adaptation of quarter pixel/one-eighth pixel.

# Introduction

In [1], an adaptive motion vector resolution method was proposed and further testing was suggested in CE3[2]. In [1], the motion accuracy can be adaptively selected to be 1/4th pixel or 1/8th pixel for each motion vector and it is extended to support 1/2 pixel or 1/4th pixel adaptation in this contribution. The choice of motion vector resolution is signaled to the decoder. A syntax element, named motion vector resolution flag (mvres_flag), is designed for this purpose.

# Proposed Solution

In this proposal, the motion accuracy can be adaptively selected to be two different resolution for each motion vector difference (mvd) and signaled via a motion resolution flag. A threshold is selected with respect to each reference frame to restrict the amplitude of mvd in higher resolution and signaled in the slice header. In the current design, the threshold is selected to be 2. If the mvd is greater than the selected threshold, the motion resolution flag is always set to true. Otherwise, the flag can be either 1 (lower resolution) or 0 (higher resolution).

Let abs_mvd_x and abs_mvd_y represents the absolute value of the motion vector difference mvd_x and mvd_y. In the current HM4.0, mvd_x and mvd_y are coded separately as shown in Figure 1. In the proposed method motion resolution flag, mvd_x and mvd_y are coded jointly as shown in Figure 2.

Figure 1. The motion vector difference signaling in HM4.0

Figure 2. The motion resolution flag and motion vector difference signaling in the proposed method

In Figure 2, abs_mvd_equal_2 is defined in Table 1 and a unary binarization is used for signaling.

| abs_mvd_equal_2 | abs_mvd_x | abs_mvd_y |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 1 | 2 |
| 2 | 2 | 1 |

# Simulation Results

The proposed method is implemented on HM 4.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM4.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate. Table 4.1 and 4.2 summarizes the experimental results.

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4% | -1.0% | -0.9% | -0.5% | -0.6% | -0.7% |
| Class B | -0.2% | -0.5% | -0.4% | -0.3% | -0.5% | -0.3% |
| Class C | -0.2% | -0.5% | -0.5% | -0.3% | -0.5% | -0.5% |
| Class D | 0.1% | -0.2% | -0.3% | 0.0% | -0.4% | -0.3% |
| Class E |  |  |  |  |  |  |
| All | -0.2% | -0.6% | -0.5% | -0.3% | -0.5% | -0.4% |
| Enc Time[%] | 95% |  |  | 92% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.5% | -0.5% | -0.3% | -0.2% | -0.1% |
| Class C | -0.2% | -0.2% | -0.3% | -0.3% | -0.1% | -0.4% |
| Class D | 0.1% | 0.0% | -0.1% | 0.0% | 0.2% | -0.3% |
| Class E | -0.1% | -0.2% | 0.5% | -0.3% | 0.2% | 0.2% |
| All | -0.1% | -0.3% | -0.2% | -0.2% | 0.0% | -0.2% |
| Enc Time[%] | 92% |  |  | 91% |  |  |
| Dec Time[%] | 94% |  |  | 92% |  |  |

|  | Low delay P |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | -0.2% | 0.1% | 0.0% | 0.1% |
| Class D | 0.3% | 0.3% | 0.3% | 0.3% | 0.4% | -0.1% |
| Class E | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | -0.2% |
| All | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 97% |  |  | 95% |  |  |
| Dec Time[%] | 99% |  |  | 95% |  |  |

Table 3.1, BD-rate saving of the proposed algorithm (half-pel/quarter pixel) relative to HM4.0

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2% | -0.2% | -0.4% | 0.3% | -0.6% | -0.5% |
| Class B | -0.1% | -0.2% | -0.4% | -0.2% | -0.6% | -0.7% |
| Class C | -0.6% | -0.7% | -0.5% | -0.7% | -1.3% | -1.2% |
| Class D | -0.8% | -0.9% | -1.2% | -1.2% | -1.8% | -2.3% |
| Class E |  |  |  |  |  |  |
| All | -0.3% | -0.5% | -0.6% | -0.4% | -1.1% | -1.1% |
| Enc Time[%] | 118% |  |  | 117% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | -0.3% | -0.5% | 0.3% | -1.1% | -1.2% |
| Class C | -0.2% | -0.6% | -0.5% | -0.5% | -1.6% | -2.0% |
| Class D | -0.8% | -0.6% | -1.9% | -1.3% | -2.5% | -3.6% |
| Class E | 1.3% | 0.8% | 1.0% | 1.0% | 0.0% | -0.1% |
| All | 0.1% | -0.2% | -0.6% | -0.2% | -1.4% | -1.8% |
| Enc Time[%] | 116% |  |  | 120% |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |

|  | Low delay P |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.7% | -1.5% | -1.7% | -1.4% | -1.8% | -1.7% |
| Class C | -3.2% | -2.7% | -2.6% | -2.8% | -3.2% | -3.5% |
| Class D | -5.1% | -4.8% | -5.5% | -4.9% | -5.2% | -6.4% |
| Class E | -1.7% | -0.7% | -0.9% | -1.0% | -1.5% | -0.8% |
| All | -2.9% | -2.5% | -2.7% | -2.5% | -2.9% | -3.2% |
| Enc Time[%] | 114% |  |  | 117% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |

Table 3.2, BD-rate saving of the proposed algorithm (quarter pixel/one-eighth pixel)relative to HM4.0

# Conclusions

This contribution proposed a coding method for the resolution of the motion vector difference. The simulation results show the average performance gain of 0.2℅ and 0.3% on  the adaptation of half-pel/quarter pixel and quarter pixel/one-eighth pixel,  respectively..

# References

[1] W.-J. Chien, X. Wang, M. Karczewicz, “Adaptive resolution on motion vector difference”,  JCTVC-F567, Torino, July. 2011.

[2] T. Chujoh, E. Alshina, ” CE3: Motion Compensation”, JCTVC-F903, Torino, July. 2011.

[3] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July. 2011.

[4] Y. Chiu, W. Zhang, L. Xu, Y. Han, “CE3: Cross-verification of Qualcomm’s adaptive motion vector resolution (JCTVC-G727) by Intel”, JCTVC-G342, Geneva, November 2011.

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).