| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G728 |
| --- | --- |

| Title: | CE4 Subtest 1: Spatial QP prediction (test 1.3.e): combination of test 1.3.b and test 1.3.d |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results of spatial QP prediction based on intra prediction direction (JCTVC-G066) and spatially neighbouring CUs (test 1.3.d) from JCTVC-F332. Simulation results show on average 0.4% BD-rate reduction for AI cases, and 0.1%, BD-rate gain for RA, LB and LP cases, respectively under the test condition defined in CE4 Subtest 1.

# Introduction

In CE4 Subtest 2, it was reported that the performance of the QP prediction based on intra prediction (2.3.g) [1] was the best for AI_HE and AI_LC configurations under the test conditions for CE4 Subtest 2 [3]. Spatially neighbouring CU QP prediction methods (test 2.3.b,) for CE4 subtest 2 [2] provides simple QP prediction method for inter coded CUs. This contribution reports the experimental results of the combination of 1.3.b with 1.3.d.

# Algorithm description

## CE4 Subtest 1.3.d

We propose to use left top most neighbor (L) CU’s QP as the predictor for coding of the current CU’s QP. If the left neighbor is not available then the top left most (T) neighboring CU’s QP is used as the predictor, as show in Figure 1.

Figure 1: CU level QP assignment

Hence delta QP calculations becomes

dQPC0 = QPC0 –  QPA5

dQPC1 = QPC1 –  QPC0

dQPC5 = QPC5 –  QPA12

dQPA0 = QPA0 –  QPB8

…………

If left CU is not available then the top CU’s QP is used as a predictor. If the neighboring predictor CU is a skipped CU, then the QP value of the predictor is the QP value of its predictor QP. Basically skip CU’s QP is assumed to of its predictor QP. Top QP predictor gets used at the beginning of each LCU row.

## CE4 Subtest 1.3.e

Intra coded blocks use JCTVC-F159 method for QP prediction. Non-directional intra prediction modes (DC and Planar) modes use Subtest 1.3.d method. Inter coded CUs use the left CU’s QP as the predictor if left CU is inter coded or both left and above CUs are intra coded, otherwise, above CU’s QP is used as the predictor. If left CU QP is not available, above CU’s QP is used.

# Experiments

## Setup

The proposed QP prediction algorithm has been implemented onto the software for CE4 anchor software [4]. Experiments to evaluate performance of the proposed method were conducted under the CE4 Subtest 1 test conditions that are based on the JCTVC common test condition defined in [5]. The simulation results are from a 64-bit Linux cluster.

## Results

Table 1 shows summary of experimental results for CE4 subtest 1.3.e. Error: Reference source not found shows summary of experimental results for combination of CE4 subtest 1.3.d.

Table 1: Summary results of test 1.3.e  CE4 anchor with MinCUDQPSize = 8x8.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.1% | -0.2% | -4.9% | -0.2% | -0.2% | -0.2% | -5.4% |
| Class B | -0.3% | -0.3% | -0.3% | -7.9% | -0.3% | -0.3% | -0.3% | -8.7% |
| Class C | -0.4% | -0.4% | -0.4% | -9.3% | -0.5% | -0.5% | -0.5% | -10.5% |
| Class D | -0.4% | -0.3% | -0.3% | -8.6% | -0.4% | -0.4% | -0.4% | -10.0% |
| Class E | -0.7% | -0.8% | -0.7% | -13.5% | -0.8% | -0.7% | -0.8% | -14.3% |
| Overall | -0.4% | -0.4% | -0.4% | -8.6% | -0.4% | -0.4% | -0.4% | -9.5% |
|  | -0.4% | -0.4% | -0.4% |  | -0.4% | -0.4% | -0.4% |  |
| Enc Time[%] | 99% |  |  |  | 97% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 98% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.3% | -1.7% | -0.1% | -0.1% | -0.2% | -2.0% |
| Class B | -0.2% | -0.2% | -0.1% | -3.6% | -0.2% | -0.2% | -0.2% | -4.3% |
| Class C | -0.2% | -0.1% | -0.3% | -4.7% | -0.3% | -0.4% | -0.4% | -5.9% |
| Class D | -0.1% | 0.1% | -0.1% | -3.7% | -0.2% | -0.4% | -0.1% | -4.7% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.2% | -3.4% | -0.2% | -0.3% | -0.2% | -4.2% |
|  | -0.1% | 0.0% | -0.2% |  | -0.2% | -0.3% | -0.2% |  |
| Enc Time[%] | 102% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.0% | -1.5% | -0.1% | -0.1% | -0.2% | -1.9% |
| Class C | -0.1% | -0.1% | 0.2% | -2.2% | -0.2% | -0.1% | -0.2% | -3.1% |
| Class D | -0.1% | -0.5% | 0.1% | -1.5% | -0.1% | -0.5% | -0.6% | -2.4% |
| Class E | 0.0% | 0.2% | 0.1% | -0.5% | -0.1% | -0.9% | -0.5% | -0.8% |
| Overall | -0.1% | -0.1% | 0.1% | -1.5% | -0.1% | -0.4% | -0.4% | -2.1% |
|  | -0.1% | -0.1% | 0.1% |  | -0.1% | -0.3% | -0.4% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 99% |  |  |  |

Table 2: Summary results of test 1.3.d  CE4 anchor with MinCUDQPSize = 8x8.

| All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | -0.2% |
| 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | -0.2% |
| 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | -0.4% |
| 0.0% | 0.0% | 0.0% | -0.5% | 0.0% | 0.0% | 0.0% | -0.6% |
| 0.0% | 0.0% | 0.0% | -0.4% | 0.0% | 0.0% | 0.0% | -0.5% |
| 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | -0.4% |
| 0.0% | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| 96% |  |  |  | 95% |  |  |  |
| 101% |  |  |  | 99% |  |  |  |
| Random Access HE |  |  |  | Random Access LC |  |  |  |
| Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| -0.1% | 0.1% | -0.1% | -0.4% | 0.0% | -0.1% | -0.2% | -0.4% |
| 0.0% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | -0.1% | -0.3% |
| 0.0% | 0.0% | 0.0% | -0.4% | 0.0% | 0.0% | 0.0% | -0.7% |
| 0.0% | 0.0% | 0.0% | -0.6% | 0.0% | -0.2% | 0.0% | -0.7% |
| 0.0% | 0.0% | 0.0% | -0.4% | 0.0% | -0.1% | -0.1% | -0.5% |
| 0.0% | 0.0% | 0.0% |  | 0.0% | -0.1% | -0.1% |  |
| 98% |  |  |  | 97% |  |  |  |
| 99% |  |  |  | 99% |  |  |  |
| Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| 0.0% | 0.0% | -0.2% | -0.4% | 0.0% | 0.0% | -0.1% | -0.5% |
| -0.1% | 0.0% | 0.0% | -0.5% | 0.0% | 0.0% | 0.1% | -0.7% |
| -0.1% | -0.3% | 0.5% | -0.6% | 0.0% | 0.1% | -0.1% | -0.7% |
| -0.1% | -0.4% | -0.9% | -1.7% | -0.1% | -0.8% | -0.5% | -1.6% |
| -0.1% | -0.1% | -0.1% | -0.7% | 0.0% | -0.1% | -0.1% | -0.8% |
| -0.1% | -0.2% | -0.1% |  | 0.0% | -0.1% | -0.1% |  |
| 97% |  |  |  | 96% |  |  |  |
| 97% |  |  |  | 101% |  |  |  |

# Conclusions

This contribution reports the experimental results of spatial QP prediction method based on1.3.e of CE4 Subtest 1. Combination of intra mode QP prediction method of 1.3.b with inter mode QP prediction of 1.3.d results in 0.4%, 0.1%, 0.1% BD-rate reduction for AI_HE, RA_HE and LB_HE configurations. Also subtest 1.3.e provides 0.4%, 0.2%, 0.1% reduction in BD-rate for AI_LC, RA_LC and LB_LC configurations.

# References

- H. Aoki, et al., “CE4 Subtest 2: QP prediction based on intra prediction (test 2.3.g),” JCT-VC document JCTVC-F159, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- M. Coban, et al., “CE4 Subtest 2: QP prediction from spatially neighboring CUs (test 2.3.b, 2.3.c),” JCT-VC document JCTVC-F332, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-E704, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

- K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-F904, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC document JCTVC-F803, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 4[1]

The modified or added portions are highlighted with yellow colour.

### A.1. Change of subclause 7.4.9 “Transform coefficient semantics”

cu_qp_delta can change the value of QPY specifies the difference between a luma quantisation parameter to be used and its prediction, for a quantisation group of coding units where the quantisation group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantisation group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantisation group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantisation group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

### A.2. Insertion of new subclause into clause 8.5

### 8.5.x	Derivation process for luma quantisation parameters

Inputs to this process are

–	a variable QpBdOffsetY,

–	a syntax element cu_qp_delta specifying the delta luma quantisation parameter of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Outputs of this process are

–	luma quantisation parameters QPY and QP’Y.

Depending on PredMode, the following applies.

–	If PredMode is equal to MODE_INTRA, quantisation parameter prediction process as specified in subclause 8.5.x.2 is invoked with the output being the reference quantisation parameter QPY, PRED.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED. .

With QPY,PRED, the value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-3)

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-4)

### 8.5.x.1		Default quantisation parameter prediction process

Inputs to this process are

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The predicted quantisation parameter QPY, PRED is derived as follows.

– 	If the left neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the top-right coding unit of the left neighbouring quantisation group of coding units is the same as PredMode, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the above neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the bottom-left coding unit of the above neighbouring quantisation group of coding units is the same as PredMode, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, QPY, PRED is set equal to QPY, PREV, where QPY,PREV is the luma quantisation parameter of the previous quantisation group of coding units in decoding order in the current slice. For the first quantisation group of coding units in the slice, QPY,PREV is initially set equal to SliceQPY at the start of each slice.

### 8.5.x.2		Intra quantisation parameter prediction process

Inputs to this process are

–	a syntax element pcm_flag specifying if the top-left coding unit of the current quantisation group of coding units is coded in PCM mode

–	a variable intraPredMode specifying the intra prediction mode of the top-left prediction unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The rearranged intra prediction order intraPredOrder is derived as follows.

–	If intraPredMode is equal to 0 (Intra_Planar) ,intraPredOrder is set to 0.

–	Otherwise, intraPredOrder is derived as specified in Table 8-7.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If intraPredOrder is greater than 0 and less than 18, and the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if intraPredOrder is greater than or equal to 18, and the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED.