| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G730 |
| --- | --- |

| Title: | Non-CE4: Non-CE4: Layered quantization matrices compression (JCTVC-G530) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami Junichi Tanaka Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 Yoshitaka.Morigami@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross check results of Layered quantization matrices compression proposed by HiSilicon. Using an experiment condition of F904, It is confirmed that the proposal can improved coding efficiency upto 2 times better comparing to F475. In addition, the contents of the source provided by the proponent was investigated in detail.

# Introduction

The proposal in JCTVC-G530 uses layered coding structure, and the proposal encodes each frame respectively. Coding parameters of each layer can be set at QMSizexCodeLevels(4x4,8x8,16x16,32x32), and the residul can be send optionally. Prediction modes are as follows,

- Coding Order within a layer

- Interpolation

- Copy mode

- Symmetry mode

- Residue Coding

- Differential coding

# Simulation Results

## Test1

In test1, only the coding performance of matrix itself is measured. In this simulation, we used the test configuration defined in CE4 Subset3. The detailed results are shown in the attached excel-file.

| Proposal | Symmetry |  | Asymmetry |  |
| --- | --- | --- | --- | --- |
| QMSizexCodeLevels | bits | AvgError | bits | AvgError |
| 3,4,5,6 | 3372 | 0 | 5614 | 0 |
| 3,4,5,5 | 2187 | 0.234191 | 3661 | 0.188235 |
| 3,4,4,5 | 1740 | 0.349632 | 2984 | 0.253676 |
| 3,3,4,5 | 1580 | 0.419853 | 2690 | 0.308456 |
| 3,3,4,4 | 1076 | 0.814338 | 1876 | 0.559926 |
| 3,3,3,3 | 632 | 2.095221 | 1144 | 1.474265 |
| 1,2,2,3 | 314 | 2.297426 | 522 | 1.7625 |
| 1,1,2,3 | 297 | 2.332353 | 493 | 1.819853 |

Table 1: Result of “no dither” test case

| Proposal | Symmetry |  | Asymmetry |  |
| --- | --- | --- | --- | --- |
| QMSizexCodeLevels | bits | AvgError | bits | AvgError |
| 3,4,5,6 | 3372 | 0 | 5614 | 0 |
| 3,4,5,5 | 2187 | 0.234191 | 3661 | 0.188235 |
| 3,4,4,5 | 1740 | 0.349632 | 2984 | 0.253676 |
| 3,3,4,5 | 1580 | 0.419853 | 2690 | 0.308456 |
| 3,3,4,4 | 1076 | 0.814338 | 1876 | 0.559926 |
| 3,3,3,3 | 632 | 2.095221 | 1144 | 1.474265 |
| 1,2,2,3 | 314 | 2.297426 | 522 | 1.7625 |
| 1,1,2,3 | 297 | 2.332353 | 493 | 1.819853 |

Table 2: Result of “dither” test case

## Test2

This experiment is conducted to evaluate the quantization matrix compression impact to the overall HM coding performance, BD rate/PSNR are measured and the compression configuration uses QMSizexCodeLevels = 2,2,2,3.

- symmetric matrices coding

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.7% | -3.1% | -2.9% | -3.7% | -8.0% | -7.4% |
| Class B | -6.3% | -6.7% | -6.6% | -7.3% | -10.0% | -9.9% |
| Class C | -15.5% | -15.9% | -16.3% | -15.1% | -16.3% | -16.9% |
| Class D | -34.6% | -34.1% | -34.4% | -33.4% | -33.6% | -33.8% |
| Class E | -19.0% | -19.2% | -18.0% | -18.7% | -21.8% | -21.2% |
| Class F |  |  |  |  |  |  |
| Overall | -14.8% | -15.2% | -15.1% | -15.1% | -17.3% | -17.3% |
|  | -14.8% | -15.2% | -15.2% | -15.1% | -17.4% | -17.4% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -19.4% | -19.0% | -18.6% | -19.0% | -17.7% | -16.3% |
| Class B | -37.6% | -35.0% | -33.9% | -36.1% | -33.4% | -32.4% |
| Class C | -56.8% | -55.1% | -55.3% | -55.8% | -54.2% | -54.4% |
| Class D | -79.2% | -77.8% | -78.0% | -78.6% | -77.5% | -77.7% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -47.6% | -46.0% | -45.7% | -46.7% | -45.0% | -44.4% |
|  | -47.8% | -46.1% | -45.9% | -46.9% | -45.0% | -44.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 93% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -39.9% | -36.6% | -36.0% | -38.7% | -35.4% | -35.1% |
| Class C | -57.3% | -54.8% | -55.2% | -56.5% | -54.3% | -55.0% |
| Class D | -79.0% | -77.3% | -77.4% | -78.6% | -77.0% | -77.5% |
| Class E | -82.4% | -80.0% | -80.6% | -81.1% | -78.4% | -79.3% |
| Class F |  |  |  |  |  |  |
| Overall | -62.0% | -59.5% | -59.5% | -61.1% | -58.6% | -59.0% |
|  | -62.0% | -59.6% | -59.6% | -61.1% | -58.6% | -59.0% |
| Enc Time[%] | 100% |  |  | 8% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -38.8% | -35.9% | -35.0% | -37.4% | -34.4% | -33.9% |
| Class C | -56.6% | -54.1% | -54.3% | -55.7% | -53.6% | -54.3% |
| Class D | -78.5% | -76.5% | -77.0% | -78.2% | -76.3% | -76.9% |
| Class E | -81.8% | -79.3% | -80.0% | -80.3% | -77.8% | -78.6% |
| Class F |  |  |  |  |  |  |
| Overall | -61.2% | -58.7% | -58.7% | -60.2% | -57.8% | -58.1% |
|  | -61.2% | -58.8% | -58.9% | -60.2% | -57.9% | -58.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 93% |  |  | 92% |  |  |

- asymmetric matrices coding

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.5% | -5.2% | -4.4% | -4.3% | -10.7% | -11.5% |
| Class B | -6.0% | -6.2% | -5.9% | -8.0% | -10.5% | -11.8% |
| Class C | -16.6% | -14.3% | -14.3% | -17.0% | -15.1% | -16.3% |
| Class D | -34.5% | -31.3% | -31.5% | -33.8% | -30.9% | -31.4% |
| Class E | -18.7% | -19.8% | -19.1% | -19.0% | -22.3% | -22.9% |
| Class F |  |  |  |  |  |  |
| Overall | -14.8% | -14.7% | -14.4% | -15.9% | -17.3% | -18.2% |
|  | -14.8% | -14.8% | -14.5% | -15.9% | -17.4% | -18.3% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -20.2% | -19.9% | -19.3% | -20.0% | -19.9% | -18.9% |
| Class B | -37.2% | -33.9% | -32.8% | -35.8% | -32.9% | -32.0% |
| Class C | -56.6% | -53.7% | -53.8% | -55.5% | -52.7% | -52.9% |
| Class D | -78.6% | -76.8% | -76.8% | -77.9% | -76.2% | -76.3% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -47.5% | -45.4% | -45.0% | -46.6% | -44.7% | -44.3% |
|  | -47.6% | -45.4% | -45.1% | -46.7% | -44.7% | -44.3% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -39.1% | -35.3% | -34.5% | -38.7% | -35.4% | -33.9% |
| Class C | -56.6% | -53.8% | -53.7% | -55.7% | -53.4% | -53.2% |
| Class D | -78.3% | -76.5% | -76.2% | -77.7% | -76.2% | -76.0% |
| Class E | -82.4% | -79.9% | -79.8% | -81.0% | -78.9% | -79.4% |
| Class F |  |  |  |  |  |  |
| Overall | -61.4% | -58.6% | -58.2% | -60.6% | -58.2% | -57.8% |
|  | -61.4% | -58.7% | -58.4% | -60.6% | -58.3% | -57.9% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -38.0% | -34.5% | -33.7% | -37.3% | -34.4% | -33.0% |
| Class C | -55.7% | -52.9% | -52.9% | -54.9% | -52.4% | -52.3% |
| Class D | -77.6% | -75.9% | -75.8% | -77.1% | -75.4% | -75.2% |
| Class E | -81.8% | -79.3% | -79.3% | -80.2% | -78.2% | -78.7% |
| Class F |  |  |  |  |  |  |
| Overall | -60.5% | -57.9% | -57.6% | -59.7% | -57.4% | -56.9% |
|  | -60.5% | -58.0% | -57.8% | -59.7% | -57.4% | -57.1% |
| Enc Time[%] | 109% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  |

# Conclusion

This contribution reports the cross check result of Layered quantization matrices compression by JCTVC-G530 (HiSilicon). Using an experiment condition of F904, the proposal is confirmed that the compress ratio can be increased to 2x above comparing to F475. In addition, the source code was investigated and the complexity of the proposal is comparable with other proposals in CE4.

# Patent rights declaration(s)

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).