| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G731 |
| --- | --- |

| Title: | CE4 Subtest 1.3.c: cross-verification of JCTVC-G067 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of spatial QP prediction based on intra prediction and prediction mode (JCTVC-G067) proposal. The results of the crosscheck matched those reported by the proponents. The encoding and decoding times also matched closely.

# Introduction

The proposed method employs the top-left-most CU and the top-left-most PU of each set of CUs where QP is assigned.

Brief algorithm of the proposed method is described as follows.

- If the top-left-most CU is intra-coded, predicted QP is calculated based on intra prediction direction.

- Otherwise, if at least either of the above CU or the left CU have the same prediction mode as that of the current CU, predicted QP is calculated with prediction mode dependent QP prediction.

- Otherwise if at least either of the above CU or the left CU is available, predicted QP is calculated with a simplified version of the former 2.3.e.r1 method [3] so that the use of above-left CU is omitted.

# Results

Results for all configurations, AI-HE, AI-LC, RA-HE, RA-LC, LB-HE and LB-LC, were crosschecked, as specified in common test conditions [2]. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. Encoding as well as decoding was done on a LINUX cluster:

Table 1: BD-rate results for the proposed algorithm for MaxCUDepthDQP=3

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.3% | -0.3% | -6.5% | -0.2% | -0.2% | -0.2% | -6.6% |
| Class B | -0.4% | -0.4% | -0.4% | -9.8% | -0.4% | -0.4% | -0.4% | -10.2% |
| Class C | -0.5% | -0.5% | -0.4% | -11.3% | -0.6% | -0.5% | -0.5% | -12.3% |
| Class D | -0.5% | -0.4% | -0.4% | -10.8% | -0.5% | -0.5% | -0.5% | -12.0% |
| Class E | -0.8% | -0.8% | -0.8% | -14.7% | -0.8% | -0.8% | -0.8% | -15.3% |
| Overall | -0.5% | -0.4% | -0.4% | -10.4% | -0.5% | -0.5% | -0.5% | -11.0% |
|  | -0.5% | -0.4% | -0.4% |  | -0.5% | -0.5% | -0.5% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 97% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | 0.2% | -0.3% | -4.7% | -0.3% | -0.2% | -0.3% | -4.6% |
| Class B | -0.3% | -0.3% | -0.2% | -6.5% | -0.3% | -0.3% | -0.4% | -6.9% |
| Class C | -0.4% | -0.4% | -0.5% | -9.0% | -0.5% | -0.5% | -0.5% | -10.3% |
| Class D | -0.3% | -0.2% | -0.1% | -7.9% | -0.4% | -0.5% | -0.5% | -9.0% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.2% | -0.3% | -7.0% | -0.4% | -0.4% | -0.4% | -7.6% |
|  | -0.3% | -0.2% | -0.3% |  | -0.4% | -0.4% | -0.4% |  |
| Enc Time[%] | 102% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 96% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.3% | -5.4% | -0.3% | -0.4% | -0.4% | -5.2% |
| Class C | -0.4% | -0.5% | -0.3% | -7.8% | -0.6% | -0.6% | -0.4% | -9.0% |
| Class D | -0.4% | -0.3% | -0.3% | -7.0% | -0.4% | -0.3% | -0.7% | -8.0% |
| Class E | -0.4% | -1.0% | -0.6% | -8.4% | -0.5% | -1.1% | -0.7% | -8.4% |
| Overall | -0.4% | -0.4% | -0.3% | -7.0% | -0.4% | -0.5% | -0.5% | -7.5% |
|  | -0.4% | -0.5% | -0.3% |  | -0.4% | -0.5% | -0.6% |  |
| Enc Time[%] | 99% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 95% |  |  |  | 97% |  |  |  |

# Conclusions

We have verified results reported in JCTVC-G067  proposal. The results of the crosscheck matched those reported by the proponents. The encoding and decoding times also matched closely.

# References

[1] H. Aoki, K. Chono, M. Kobayashi, M. Shima, and K. Sato, “CE4 Subtest1: test 1.3.c,” JCTVC-G067, JCT-VC 7th Meeting, Geneva, 2011

[2] K. Sato, M. Budagavi, M. Coban, H. Aoki, and X. Li, “Description of Core Experiment 4: Quantization,” JCTVC-F904, JCT-VC 6th Meeting, Torino, 2011

[3] K. Sato, “CE4: Result of 2.3.d and 2.3.e,” JCT-VC document JCTVC-F420, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.