| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G732 |
| --- | --- |

| Title: | Study on test materials in common test conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610 Japan | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

In JCTVC-E011, the problem of the class E test materials in the common test condition was reported. The contribution investigates the reason why this is happened and proposes to reconsider to replace the class E test materials.

# Class E sequences

The problem of the current class E test materials were reported in JCTVC-E011 as follows.

“

In the last meeting in Daegu, the problems of the current class E sequences were reported. All Vidyo sequences are slightly trembling vertically and it is much visible in chorma components, which are not present in other common sequences. Those sequences are compressed by camera and it maybe one of the reason why characteristic is unusual.

It is encourage to collect more appropriate uncompressed class E test materials, if tele-conference sequences are necessary.

“

The reason of this phenomenon is investigated in this contribution.

Figure 1: Pixel value in each frame

Figure 2: Pixel value in each frame

As shown in Figure 1 and Figure 2, the pixel value is changed frame by frame, even at the still area. This phenomenon could impact on the evaluation of the video coding tools. For example, tools to use this line by line change could improve coding efficiency of the current class E. This could be happened by the followings,

- The original was captured as interlace video and then de-interlace was applied

- The original was shoot as compressed video

To evaluate 720p video conference materials, those sequences should be captured by 720p natively without compression.

# Conclusion

This contribution proposes

- To replace class E test materials

Or

- To issue call for test materials for future replacement of the class E

# Patent rights declaration(s)

Sony Corporation does not have any current or pending patent rights relating to the technology described in this contribution.