| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G0734 WG11 Number: m22303 |
| --- | --- |

| Title: | CE7: Crosscheck of G0304 ROT Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala 5840 Banneker Rd., #230 Columbia, MD 21044 USA | Email: | pankajtva@gmail.com Tel. +1-410-730-6922 |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

This contribution provides a cross-check performed by FastVDO LLC of Samsung’s rotational transform proposal in CE7, JCTVC-G0304. Our crosscheck results matched those reported by Samsung. The encoding and decoding times also matched exactly (which was a slight surprise).

Introduction

A rotational transform (ROT) is a secondary transform, applied after the “core” DCT-like transform.  In this instance, the ROT is an 8x8 transform, which is applied to the upper-left portions of the core transforms of size 8, 16, and 32. The ROT in this instance is developed as a concatenation of (compound) Givens rotations, which actually rotate 2 axes (out of 8) at a time. The rotation angles are selected by training over extensive video data (not specified in the proposal), leading to messy numerical rotations.  The problem is how to compute this in a simple and fast way.  Here, lifting decompositions of rotations come to the aid, allowing fast computation with dyadic rational coefficients (surprise!).  This allows a perfectly invertible transform, in-place computation, and and even 5-bit precision arithmetic. Further simplifications are also made in the encoder design relating to RDOQ to reduce the encoder burden.

# Results

Results for 4 configurations, AI-HE, AI-LC, RA-HE, and RA-LC, were crosschecked, as specified in CE7 test conditions. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. Encoding as well as decoding was done on a LINUX cluster (64 bit).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.1% | -0.1% | -0.2% | -1.1% | 0.1% | -0.2% |
| Class B | -1.1% | 0.5% | 0.5% | -1.0% | 0.6% | 0.6% |
| Class C | -0.6% | 0.2% | 0.2% | -0.7% | 0.4% | 0.3% |
| Class D | -0.6% | 0.3% | 0.2% | -0.7% | 0.5% | 0.4% |
| Class E | -0.8% | 0.1% | 0.1% | -0.8% | 0.3% | 0.5% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
|  | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
| Enc Time[%] | 129% |  |  | 158% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.2% | -0.5% | -0.5% | -0.4% | -0.4% |
| Class B | -0.6% | 0.2% | 0.3% | -0.7% | 0.2% | 0.3% |
| Class C | -0.4% | 0.0% | 0.0% | -0.5% | 0.0% | 0.2% |
| Class D | -0.3% | 0.1% | -0.3% | -0.4% | 0.4% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.5% | 0.0% | -0.1% | -0.5% | 0.1% | 0.1% |
|  | -0.5% | 0.0% | -0.1% | -0.5% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 1: BD-rate results obtained by FastVDO for the proposed rotational transform

# Conclusions

We have verified the coding performance results reported in Samsung’s proposed rotational transform implemented as a lifting factorization. For AI configuration, BD-rate of -0.9% and -0.9% is achieved for HE and LC, respectively. This is achieved at a cost of encoding time increase of 29% and 58%, respectively. For RA configuration, BD-rate of -0.5% and -0.5% is achieved for HE and LC, respectively. In this case, the encoding time increase is very marginal, 3-4%. The encoding and the decoding times were actually identical to those reported by Samsung, which is a first for our crosschecks.