| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G0734 WG11 Number: m22303 |
| --- | --- |

| Title: | CE7: Crosscheck of G0304 ROT Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala 5840 Banneker Rd., #230 Columbia, MD 21044 USA | Email: | pankajtva@gmail.com Tel. +1-410-730-6922 |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

This contribution provides a cross-check performed by FastVDO LLC of Samsung’s rotational transform proposal in CE7, JCTVC-G0304. Our crosscheck results matched those reported by Samsung. The encoding and decoding times also matched exactly (which was a slight surprise).

Introduction

A rotational transform (ROT) is a secondary transform, applied after the “core” DCT-like transform.  In this instance, the ROT is an 8x8 transform, which is applied to the upper-left portions of the core transforms of size 8, 16, and 32. The ROT in this instance is developed as a concatenation of (compound) Givens rotations, which actually rotate 2 axes (out of 8) at a time. The rotation angles are selected by training over extensive video data (not specified in the proposal), leading to messy numerical rotations.  The problem is how to compute this in a simple and fast way.  Here, lifting decompositions of rotations come to the aid, allowing fast computation with dyadic rational coefficients (surprise!).  This allows a perfectly invertible transform, in-place computation, and and even 5-bit precision arithmetic. Further simplifications are also made in the encoder design relating to RDOQ to reduce the encoder burden.

# Results

1.1. Coding Efficiency Test

Results for 4 configurations, AI-HE, AI-LC, RA-HE, and RA-LC, were crosschecked, as specified in CE7 test conditions. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. Encoding as well as decoding was done on a LINUX cluster (64 bit).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.1% | -0.1% | -0.2% | -1.1% | 0.1% | -0.2% |
| Class B | -1.1% | 0.5% | 0.5% | -1.0% | 0.6% | 0.6% |
| Class C | -0.6% | 0.2% | 0.2% | -0.7% | 0.4% | 0.3% |
| Class D | -0.6% | 0.3% | 0.2% | -0.7% | 0.5% | 0.4% |
| Class E | -0.8% | 0.1% | 0.1% | -0.8% | 0.3% | 0.5% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
|  | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
| Enc Time[%] | 129% |  |  | 158% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.2% | -0.5% | -0.5% | -0.4% | -0.4% |
| Class B | -0.6% | 0.2% | 0.3% | -0.7% | 0.2% | 0.3% |
| Class C | -0.4% | 0.0% | 0.0% | -0.5% | 0.0% | 0.2% |
| Class D | -0.3% | 0.1% | -0.3% | -0.4% | 0.4% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.5% | 0.0% | -0.1% | -0.5% | 0.1% | 0.1% |
|  | -0.5% | 0.0% | -0.1% | -0.5% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 1: BD-rate results obtained by FastVDO for the proposed rotational transform

1.2. Arithmetic Complexity Analysis

This transform method uses 4 sets of horizontal and vertical rotational transforms (plus the trivial “identity rotation” wherein no ROT is used), which are composite transforms of three rotations each. The rotations are themselves decomposed into three lifting steps each. The calculations are presented in straightforward pseudocode.  Each lifting step involves one multiplication, one add, and one shift, and are based on using three arrays of lifting coefficients, labeled u, v, and w; the array elements, which are notionally multipliers, are given in tables 1-3 in G304.  These are reproduced below. Due to 18 lines of code, introducing one each of mult, add, and shift, there would be 18 of each, for a 1D transform. Since there are 8 rows and 8 columns to do, we have 18*16=288 mults, adds, and shifts notionally.

|  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| idx = 0 | -7 | -7 | 5 | -4 | -2 | -2 |
| idx = 1 | 4 | -22 | 9 | -7 | -4 | 5 |
| idx = 2 | -3 | -5 | -3 | 2 | -29 | 5 |
| hidx = 3 | -10 | -6 | -4 | -1 | -29 | 6 |

Table 1: u Lifting Multipliers

|  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| idx = 0 | 14 | 13 | -10 | 8 | 3 | 4 |
| idx = 1 | -8 | 30 | -17 | 13 | 7 | -10 |
| idx = 2 | 5 | 9 | 6 | -5 | 32 | -10 |
| idx = 3 | 18 | 11 | 8 | 1 | 32 | -12 |

Table 2: v Lifting Multipliers

|  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| idx = 0 | -7 | -7 | 5 | -4 | -1 | -2 |
| idx = 1 | 4 | -22 | 9 | -7 | -4 | 5 |
| idx = 2 | -3 | -5 | -3 | 2 | -29 | 5 |
| idx = 3 | -10 | -6 | -4 | -1 | -29 | 6 |

Table 3: w Lifting Multipliers

In some cases above, when the array element is already a power of 2, it is convenient to implement the mult as a shift, reducing the mults but increasing the shifts (viewed as less complex, especially in hardware).  Due to the particular constants chosen above in the lifting implementation, these then convert to different numbers of mults, adds, and shifts for arithmetic complexity.

As an example, one can see from the tables above that for idx=0, there are 3 mults for u, 4 mults for v, and 3 mults for w.  Similar computations can be made for the adds and shifts.  Assuming that each mode (according to the idx) is equally probable (including the NO ROT case), a reasonable assumption, one arrives at the following numbers for totals: 1152 adds, 816 mults, and 1488 shifts; now counting in the trivial (no ROT) case, which has 0 complexity, we now divide by 5, to obtain:

230 adds, 163 mults, and 298 shifts.  This agrees with the numbers reported in G304.

# Conclusions

We have verified the coding performance results reported in Samsung’s proposed rotational transform implemented as a lifting factorization. For AI configuration, BD-rate of -0.9% and -0.9% is achieved for HE and LC, respectively. This is achieved at a cost of encoding time increase of 29% and 58%, respectively. For RA configuration, BD-rate of -0.5% and -0.5% is achieved for HE and LC, respectively. In this case, the encoding time increase is very marginal, 3-4%. The encoding and the decoding times were actually identical to those reported by Samsung, which is a first for our crosschecks.