| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G736 |
| --- | --- |

| Title: | On luma/chroma interpolation precision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents a 16-bit biprediction luma/chroma interpolation process with negligible impact on coding efficiency. In the current design, the intermediate values for calculating the biprediction sample values have 17-bit values that are converted to 16-bit values with the use of an offset. When all the intermediate values are reduced to 16-bit, there is no drop in performance. Simulation results show 0.0%, 0.0%, and 0.0% BD rate change for RA_HE, LB_HE and LP_HE cases, respectively.

# Introduction

In current HM 4.0, 2D biprediction interpolation process uses 17-bit intermediate values that have 16-bit range. With the use of a non-normative offset, the intermediate values are converted to 16-bit values. In the interpolation process, besides the 2D biprediction (2D bipred) interpolation part, other intermediate values without the use of non-normative offset is 16-bit values. In 2D bipred process, if the sample values after second stage of filtering are right shifted by 1 extra bit, then 16-bit precision for 2D bipred values are guaranteed without use of an offset [1]. The changes to HM 4.0 could be summarized by the following Table 1.

Table 1: 16-bit 2D bipred process

| 2D Bipred | HM 4.0 | Proposed |
| --- | --- | --- |
| BitDepth = 8 | Hor.Filtering >> 0, Ver. Filtering >> 6 | Hor.Filtering >> 0, Ver. Filtering >> 7 |
| BitDepth = 10 | Hor. Filtering >> 2, Ver. Filtering >> 6 | Hor.Filtering >> 2, Ver. Filtering >> 7 |

All other cases remain the same.

As an alternative to this method, higher precision interpolation can be done with the use of existing offset while maintaining 16-bit intermediate values. In current HM4.0, for 10-bit BitDepth, after the first stage of a 2D interpolation filtering process, filtered values are right shifted by 2. Replacing the right shift by 2 bits operation with right shift by 1 bit would still preserve the 16-bit intermediate value. Right shift by one less bit is compensated by right shift by one more bit after the second stage filtering of a 2D interpolation filtering process [2]. This can be summarized by the following Table 2.

Table 2: Higher precision (17-bit values, with 16-bit range) interpolation process

|  |  | HM 4.0 | Proposed |
| --- | --- | --- | --- |
| Uni-pred | 1D | (Hor/Ver.Filtering + 32)>> 6 | (Hor/Ver.Filtering + 32)>> 6 |
|  | 2D | Hor. Filtering >> 2, (Ver. Filtering + 512) >> 10 | Hor. Filtering >> 1, (Ver. Filtering + 1024) >> 11 |
| Bi-pred | 1D | Hor/Ver.Filtering >> 2 | Hor/Ver.Filtering >> 2 |
|  | 2D | Hor. Filtering >> 2, Ver. Filtering >> 6 | Hor. Filtering >> 1, Ver. Filtering >> 7 |

There is no change for the 8-bit BitDepth case.

# Results

The proposed luma/chroma interpolation precision reduction algorithms are implemented in HM4.0. The simulations are conducted based on the JCT-VC common test condition defined in [3]. Proposed changes for 16-bit interpolation process affect RA, LB and LP configurations. Table 3 shows summary of simulation results. As can be from the results on average there is 0.0% BD rate loss.

Table 3 Summary of simulation results for 16-bit interpolation (BD-rate), HM40 anchor

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.0% | 0.3% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.3% | 0.0% |
| Class D | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class E | 0.0% | 0.2% | 0.5% | 0.0% | -0.1% | -0.2% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Proposed changes for higher resolution interpolation process affect RA_HE, LB_HE and LP_HE configurations only. Table 3 shows summary of simulation results. As can be from the results on average there is 0.0% BD rate gain.

Table 4: Higher resolution interpolation process.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.3% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.3% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | -0.1% | 0.8% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | 0.3% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

# Conclusions

This contribution has presented two alternative interpolation processes. First one is a modification to HM4.0 that guarantees 16-bit precision for intermediate interpolation values without the use of additional non-normative sample offset. The method guarantees 16-bit range for storage of intermediate values for bi prediction interpolation process without loss in coding efficiency. The second one increases the precision of intermediate values for 2D filtering case.

# References

- M. Coban, et al., “Modification to JCTVC-F537: 16-bit bi-prediction interpolation process”, JCTVC-F733, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- M. Coban, et al., “Luma/chroma interpolation process”, JCTVC-F585, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC document JCTVC-F803, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 4[4]

- Algorithm 1: (16-bit precision)

###### Luma sample interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL[ xL, yL ] and interp2DFlagLXL.

Figure 8-1 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation

In Figure 8-1, the positions labelled with upper-case letters Ai, j within shaded blocks represent luma samples at full-sample locations inside the given two-dimensional array refPicLXL of luma samples. These samples may be used for generating the predicted luma sample value predSampleLXL[ xL, yL ]. The locations ( xAi, j, yAi, j ) for each of the corresponding luma samples Ai, j inside the given array refPicLXL of luma samples are derived as follows:

xAi, j = Clip3( 0, PicWidthInSamplesL – 1, xIntL +i )		(8-1)
yAi, j = Clip3( 0, PicHeightInSamplesL – 1, yIntL +j )		(8-2)

Variables shift1, shift2 and shift3 are derived as follows.

–	The variable shift1 is set equal to BitDepthY – 8, the variable shift2 is set equal to 6 + (predFlagL0&predFlagL1), and the variable shift3 is set equal to 14 – BitDepthY.

Given the luma samples Ai, j at full-sample locations ( xAi, j, yAi, j ), the luma samples ‘a0,0’ to ‘r0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples:

a0,0 = ( −A−3,0 + 4*A−2,0 − 10*A−1,0 + 57*A0,0 +	 19*A1,0 − 7*A2,0 + 3*A3,0 − A4,0 ) >> shift1	(8-3)

b0,0 = ( −A−3,0 + 4*A−2,0 − 11*A−1,0 + 40*A0,0 +	 40*A1,0 − 11*A2,0 + 4*A3,0 − A4,0 ) >> shift1	(8-4)

c0,0 = ( −A−3,0 + 3*A−2,0 − 7*A−1,0 + 19*A0,0 +	 57*A1,0 − 10*A2,0 + 4*A3,0 − A4,0 ) >> shift1	(8-5)

d0,0 = ( −A0,−3 + 4*A0,−2 − 10*A0,−1 + 57*A0,0 +	 19*A0,1 − 7*A0,2 + 3*A0,3 − A0,4 ) >> shift1	(8-6)

h0,0 = ( −A0,−3 + 4*A0,−2 − 11*A0,−1 + 40*A0,0 +	 40*A0,1 − 11*A0,2 + 4*A0,3 − A0,4 ) >> shift1	(8-7)

n0,0 = ( −A0,−3 + 3*A0,−2 − 7*A0,−1 + 19*A0,0 +	 57*A0,1 − 10*A0,2 + 4*A0,3 − A0,4 ) >> shift1	(8-8)

–	The samples labelled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by applying the 8-tap filter to the samples a0,i, b0,i and c0,i where i = −3..4 in vertical direction and interp2DFlagLXL is set equal to 1:

e0,0 = ( −a0,−3 + 4*a0,−2 − 10*a0,−1 + 57*a0,0 +	 19*a0,1 − 7*a0,2 + 3*a0,3 − a0,4 ) >> shift2	(8-9)

f0,0 = ( −a0,−3 + 4*a0,−2 − 11*a0,−1 + 40*a0,0 +	 40*a0,1 − 11*a0,2 + 4*a0,3 − a0,4 ) >> shift2	(8-10)

g0,0 = ( −a0,−3 + 3*a0,−2 − 7*a0,−1 + 19*a0,0 +	 57*a0,1 − 10*a0,2 + 4*a0,3 − a0,4 ) >> shift2	(8-11)

i0,0 = ( −b0,−3 + 4*b0,−2 − 10*b0,−1 + 57*b0,0 +	 19*b0,1 − 7*b0,2 + 3*b0,3 − b0,4 ) >> shift2	(8-12)

j0,0 = ( −b0,−3 + 4*b0,−2 − 11*b0,−1 + 40*b0,0 +	 40*b0,1 − 11*b0,2 + 4*b0,3 − b0,4 ) >> shift2	(8-13)

k0,0 = ( −b0,−3 + 3*b0,−2 − 7*b0,−1 + 19*b0,0 +	 57*b0,1 − 10*b0,2 + 4*b0,3 − b0,4 ) >> shift2	(8-14)

p0,0 = ( −c0,−3 + 4*c0,−2 − 10*c0,−1 + 57*c0,0 +	 19*c0,1 − 7*c0,2 + 3*c0,3 − c0,4 ) >> shift2	(8-15)

q0,0 = ( −c0,−3 + 4*c0,−2 − 11*c0,−1 + 40*c0,0 +	 40*c0,1 − 11*c0,2 + 4*c0,3 − c0,4 ) >> shift2	(8-16)

r0,0 = ( −c0,−3 + 3*c0,−2 − 7*c0,−1 + 19*c0,0 +	 57*c0,1 − 10*c0,2 + 4*c0,3 − c0,4 ) >> shift2	(8-17)

The positions labelled with lower-case letters within un-shaded blocks represent luma samples at quarter-pel sample fractional locations. The luma location offset in fractional-sample units ( xFracL, yFracL ) specifies which of the generated luma samples at full-sample and fractional-sample locations is assigned to the predicted luma sample value predSampleLXL[ xL, yL ]. This assignment is done according to Table 8-5. The value of predSampleLXL[ xL, yL ] shall be the output.

Table 8-5 – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predSampleLXL[ xL, yL ] | A << shift3 | d | h | n | a | e | i | p | b | f | j | q | c | g | k | r |

###### Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC[ xC, yC ] and interp2DFlagLXC

Figure 8-2 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation

In Figure 8-1, the positions labelled with upper-case letters Bi, j within shaded blocks represent chroma samples at full-sample locations inside the given two-dimensional array refPicLXC of chroma samples. These samples may be used for generating the predicted chroma sample value predSampleLXC[ xC, yC ]. The locations ( xBi, j, yBi, j ) for each of the corresponding chroma samples Bi, j inside the given array refPicLXC of chroma samples are derived as follows:

xBi, j = Clip3( 0, PicWidthInSamplesC – 1, xIntC +i )		(8-18)
yBi, j = Clip3( 0, PicHeightInSamplesC – 1, yIntC +j )		(8-19)

Variables shift1, shift2 and shift3 are derived as follows.

–	The variable shift1 is set equal to BitDepthC – 8, the variable shift2 is set equal to BitDepthC – 2 + (predFlagL0&predFlagL1), and the variable shift3 is set equal to 14 – BitDepthC.

Given the chroma samples Bi, j at full-sample locations ( xBi, j, yBi, j ), the chroma samples ‘ab0,0’ to ‘hh0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled ab0,0, ac0,0, ad0,0, ae0,0, af0,0, ag0,0, and ah0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ab0,0 = ( −3*B−1,0 + 60*B0,0 + 8*B1,0 – B2,0 ) >> shift1		(8-20)

ac0,0 = ( −4*B−1,0 + 54*B0,0 + 16*B1,0 – 2*B2,0 ) >> shift1	(8-21)

ad0,0 = ( −5*B−1,0 + 46*B0,0 + 27*B1,0 – 4*B2,0 ) >> shift1	(8-22)

ae0,0 = ( −4*B−1,0 + 36*B0,0 + 36*B1,0 – 4*B2,0 ) >> shift1	(8-23)

af0,0 = ( −4*B−1,0 + 27*B0,0 + 46*B1,0 – 5*B2,0 ) >> shift1	(8-24)

ag0,0 = ( −2*B−1,0 + 16*B0,0 + 54*B1,0 – 4*B2,0 ) >> shift1	(8-25)

ah0,0 = ( −B−1,0 + 8*B0,0 + 60*B1,0 – 3*B2,0 ) >> shift1		(8-26)

–	The samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ba0,0 = ( −3*B0,−1 + 60*B0,0 + 8*B0,1 – B0,2 ) >> shift1		(8-27)

ca0,0 = ( −4*B0,−1 + 54*B0,0 + 16*B0,1 – 2*B0,2 ) >> shift1	(8-28)

da0,0 = ( −5*B0,−1 + 46*B0,0 + 27*B0,1 – 4*B0,2 ) >> shift1	(8-29)

ea0,0 = ( −4*B0,−1 + 36*B0,0 + 36*B0,1 – 4*B0,2 ) >> shift1	(8-30)

fa0,0 = ( −4*B0,−1 + 27*B0,0 + 46*B0,1 – 5*B0,2 ) >> shift1	(8-31)

ga0,0 = ( −2*B0,−1 + 16*B0,0 + 54*B0,1 – 4*B0,2 ) >> shift1	(8-32)

ha0,0 = ( −B0,−1 + 8*B0,0 + 60*B0,1 – 3*B0,2 ) >> shift1		(8-33)

–	The samples labelled bX0,0, cX0,0, dX0,0, eX0,0, fX0,0, gX0,0 and hX0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by applying the 4-tap filter to the intermediate values aX0,i where i = −1..2 in vertical direction and interp2DflagLXC is set equal to 1:

bX0,0 = ( −3*aX0,−1 + 60*aX0,0 + 8*aX0,1 – aX0,2 ) >> shift2	(8-34)

cX0,0 = ( −4*aX0,−1 + 54*aX0,0 + 16*aX0,1 – 2*aX0,2 ) >> shift2	(8-35)

dX0,0 = ( −5*aX0,−1 + 46*aX0,0 + 27*aX0,1 – 4*aX0,2 ) >> shift2	(8-36)

eX0,0 = ( −4*aX0,−1 + 36*aX0,0 + 36*aX0,1 – 4*aX0,2 ) >> shift2	(8-37)

fX0,0 = ( −4*aX0,−1 + 27*aX0,0 + 46*aX0,1 – 5*aX0,2 ) >> shift2	(8-38)

gX0,0 = ( −2*aX0,−1 + 16*aX0,0 + 54*aX0,1 – 4*aX0,2 ) >> shift2	(8-39)

hX0,0 = ( −aX0,−1 + 8*aX0,0 + 60*aX0,1 – 3*aX0,2 ) >> shift2	(8-40)

The positions labelled with lower-case letters within un-shaded blocks represent chroma samples at eighth-pel sample fractional locations. The chroma location offset in fractional-sample units ( xFracC, yFracC ) specifies which of the generated chroma samples at full-sample and fractional-sample locations is assigned to the predicted chroma sample value predSampleLXC[ xC, yC ]. This assignment is done according to Table 8-6. The value of predSampleLXC[ xC, yC ] shall be the output.

Table 8-6 – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | B << shift3 | ba | ca | da | ea | fa | ga | ha | aY | bY | cY | dY | eY | fY | gY | hY |

##### Weighted sample prediction process

[Ed.: (KU) only the default weighted prediction part is added for bi-directional averaging]

Inputs to this process are:

–	a location ( xB, yB ) specifying the top-left sample of the current prediction unit relative to the top left sample of the current coding unit,

–	the width and height of this prediction unit, nPSW and nPSH,

–	two (nPSW)x(nPSH) arrays predSamplesL0 and predSamplesL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	the bit-depth of the chroma component, bitDepth,

–	two 2D interpolation flags interp2DFlagLX.

Outputs of this process are:

–	the (nPSW)x(nPSH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to 14 – bitDepth and the variable shift2 is set equal to 15 – bitDepth,

–	The variable offset1 is set equal to 1 << ( shift1 – 1 ) and the variable offset2 is set equal to 1 << ( shift2 – 1 ).

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x, y ] with x = 0..(nPSW)-1 and y = 0..(nPSH)-1 are derived as follows.

–	If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

predSamples[ x, y ] = ( predSamplesL0[ x, y ] + offset1 ) >> shift1	(8-41)

–	Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

predSamples[ x, y ] = ( predSamplesL1[ x, y ] + offset1 ) >> shift1	(8-42)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1),

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , 
								 ((predSamplesL0[ x, y ]  << interp2DFlagL0) + (predSamplesL1[ x, y ] ]  << interp2DFlagL1) + offset2 ) >> shift2 )	(8-43)

- Algorithm 2: (Higher precision)

###### Luma sample interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL[ xL, yL ] and interp1DFlagLXL.

Figure 8-3 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation

In Figure 8-1, the positions labelled with upper-case letters Ai, j within shaded blocks represent luma samples at full-sample locations inside the given two-dimensional array refPicLXL of luma samples. These samples may be used for generating the predicted luma sample value predSampleLXL[ xL, yL ]. The locations ( xAi, j, yAi, j ) for each of the corresponding luma samples Ai, j inside the given array refPicLXL of luma samples are derived as follows:

xAi, j = Clip3( 0, PicWidthInSamplesL – 1, xIntL +i )		(8-44)
yAi, j = Clip3( 0, PicHeightInSamplesL – 1, yIntL +j )		(8-45)

Variables shift1, shift2 and shift3 are derived as follows.

–	The variable shift1 is set equal to BitDepthY – 8 – 1, if the value of shift1 is less than 0, the variable shift1 is set to 0, the variable shift2 is set equal to 6 + (predFlagL0&predFlagL1), and the variable shift3 is set equal to 14 – BitDepthY.

Given the luma samples Ai, j at full-sample locations ( xAi, j, yAi, j ), the luma samples ‘a0,0’ to ‘r0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples and interp1DFlagLXL is set equal to 1 if shift1 is not equal to 0:

a0,0 = ( −A−3,0 + 4*A−2,0 − 10*A−1,0 + 57*A0,0 +	 19*A1,0 − 7*A2,0 + 3*A3,0 − A4,0 ) >> shift1	(8-46)

b0,0 = ( −A−3,0 + 4*A−2,0 − 11*A−1,0 + 40*A0,0 +	 40*A1,0 − 11*A2,0 + 4*A3,0 − A4,0 ) >> shift1	(8-47)

c0,0 = ( −A−3,0 + 3*A−2,0 − 7*A−1,0 + 19*A0,0 +	 57*A1,0 − 10*A2,0 + 4*A3,0 − A4,0 ) >> shift1	(8-48)

d0,0 = ( −A0,−3 + 4*A0,−2 − 10*A0,−1 + 57*A0,0 +	 19*A0,1 − 7*A0,2 + 3*A0,3 − A0,4 ) >> shift1	(8-49)

h0,0 = ( −A0,−3 + 4*A0,−2 − 11*A0,−1 + 40*A0,0 +	 40*A0,1 − 11*A0,2 + 4*A0,3 − A0,4 ) >> shift1	(8-50)

n0,0 = ( −A0,−3 + 3*A0,−2 − 7*A0,−1 + 19*A0,0 +	 57*A0,1 − 10*A0,2 + 4*A0,3 − A0,4 ) >> shift1	(8-51)

–	The samples labelled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by applying the 8-tap filter to the samples a0,i, b0,i and c0,i where i = −3..4 in vertical direction:

e0,0 = ( −a0,−3 + 4*a0,−2 − 10*a0,−1 + 57*a0,0 +	 19*a0,1 − 7*a0,2 + 3*a0,3 − a0,4 ) >> shift2	(8-52)

f0,0 = ( −a0,−3 + 4*a0,−2 − 11*a0,−1 + 40*a0,0 +	 40*a0,1 − 11*a0,2 + 4*a0,3 − a0,4 ) >> shift2	(8-53)

g0,0 = ( −a0,−3 + 3*a0,−2 − 7*a0,−1 + 19*a0,0 +	 57*a0,1 − 10*a0,2 + 4*a0,3 − a0,4 ) >> shift2	(8-54)

i0,0 = ( −b0,−3 + 4*b0,−2 − 10*b0,−1 + 57*b0,0 +	 19*b0,1 − 7*b0,2 + 3*b0,3 − b0,4 ) >> shift2	(8-55)

j0,0 = ( −b0,−3 + 4*b0,−2 − 11*b0,−1 + 40*b0,0 +	 40*b0,1 − 11*b0,2 + 4*b0,3 − b0,4 ) >> shift2	(8-56)

k0,0 = ( −b0,−3 + 3*b0,−2 − 7*b0,−1 + 19*b0,0 +	 57*b0,1 − 10*b0,2 + 4*b0,3 − b0,4 ) >> shift2	(8-57)

p0,0 = ( −c0,−3 + 4*c0,−2 − 10*c0,−1 + 57*c0,0 +	 19*c0,1 − 7*c0,2 + 3*c0,3 − c0,4 ) >> shift2	(8-58)

q0,0 = ( −c0,−3 + 4*c0,−2 − 11*c0,−1 + 40*c0,0 +	 40*c0,1 − 11*c0,2 + 4*c0,3 − c0,4 ) >> shift2	(8-59)

r0,0 = ( −c0,−3 + 3*c0,−2 − 7*c0,−1 + 19*c0,0 +	 57*c0,1 − 10*c0,2 + 4*c0,3 − c0,4 ) >> shift2	(8-60)

The positions labelled with lower-case letters within un-shaded blocks represent luma samples at quarter-pel sample fractional locations. The luma location offset in fractional-sample units ( xFracL, yFracL ) specifies which of the generated luma samples at full-sample and fractional-sample locations is assigned to the predicted luma sample value predSampleLXL[ xL, yL ]. This assignment is done according to Table 8-5. The value of predSampleLXL[ xL, yL ] shall be the output.

Table 8-7 – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predSampleLXL[ xL, yL ] | A << shift3 | d | h | N | a | e | i | p | b | f | j | q | c | g | k | r |

###### Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC[ xC, yC ] and interp1DFlagLXC

Figure 8-4 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation

In Figure 8-1, the positions labelled with upper-case letters Bi, j within shaded blocks represent chroma samples at full-sample locations inside the given two-dimensional array refPicLXC of chroma samples. These samples may be used for generating the predicted chroma sample value predSampleLXC[ xC, yC ]. The locations ( xBi, j, yBi, j ) for each of the corresponding chroma samples Bi, j inside the given array refPicLXC of chroma samples are derived as follows:

xBi, j = Clip3( 0, PicWidthInSamplesC – 1, xIntC +i )		(8-61)
yBi, j = Clip3( 0, PicHeightInSamplesC – 1, yIntC +j )		(8-62)

Variables shift1, shift2 and shift3 are derived as follows.

–	The variable shift1 is set equal to BitDepthC – 8, – 1, if the value of shift1 is less than 0, the variable shift1 is set to 0, the variable shift2 is set equal to BitDepthC – 2 + (predFlagL0&predFlagL1) , and the variable shift3 is set equal to 14 – BitDepthC.

Given the chroma samples Bi, j at full-sample locations ( xBi, j, yBi, j ), the chroma samples ‘ab0,0’ to ‘hh0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled ab0,0, ac0,0, ad0,0, ae0,0, af0,0, ag0,0, and ah0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value and interp1DflagLXC is set equal to 1 if shift1 is not equal to 0:

ab0,0 = ( −3*B−1,0 + 60*B0,0 + 8*B1,0 – B2,0 ) >> shift1		(8-63)

ac0,0 = ( −4*B−1,0 + 54*B0,0 + 16*B1,0 – 2*B2,0 ) >> shift1	(8-64)

ad0,0 = ( −5*B−1,0 + 46*B0,0 + 27*B1,0 – 4*B2,0 ) >> shift1	(8-65)

ae0,0 = ( −4*B−1,0 + 36*B0,0 + 36*B1,0 – 4*B2,0 ) >> shift1	(8-66)

af0,0 = ( −4*B−1,0 + 27*B0,0 + 46*B1,0 – 5*B2,0 ) >> shift1	(8-67)

ag0,0 = ( −2*B−1,0 + 16*B0,0 + 54*B1,0 – 4*B2,0 ) >> shift1	(8-68)

ah0,0 = ( −B−1,0 + 8*B0,0 + 60*B1,0 – 3*B2,0 ) >> shift1		(8-69)

–	The samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value and interp1DflagLXC is set equal to 1 if shift1 is not equal to 0:

ba0,0 = ( −3*B0,−1 + 60*B0,0 + 8*B0,1 – B0,2 ) >> shift1		(8-70)

ca0,0 = ( −4*B0,−1 + 54*B0,0 + 16*B0,1 – 2*B0,2 ) >> shift1	(8-71)

da0,0 = ( −5*B0,−1 + 46*B0,0 + 27*B0,1 – 4*B0,2 ) >> shift1	(8-72)

ea0,0 = ( −4*B0,−1 + 36*B0,0 + 36*B0,1 – 4*B0,2 ) >> shift1	(8-73)

fa0,0 = ( −4*B0,−1 + 27*B0,0 + 46*B0,1 – 5*B0,2 ) >> shift1	(8-74)

ga0,0 = ( −2*B0,−1 + 16*B0,0 + 54*B0,1 – 4*B0,2 ) >> shift1	(8-75)

ha0,0 = ( −B0,−1 + 8*B0,0 + 60*B0,1 – 3*B0,2 ) >> shift1		(8-76)

–	The samples labelled bX0,0, cX0,0, dX0,0, eX0,0, fX0,0, gX0,0 and hX0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by applying the 4-tap filter to the intermediate values aX0,i where i = −1..2 in vertical direction:

bX0,0 = ( −3*aX0,−1 + 60*aX0,0 + 8*aX0,1 – aX0,2 ) >> shift2	(8-77)

cX0,0 = ( −4*aX0,−1 + 54*aX0,0 + 16*aX0,1 – 2*aX0,2 ) >> shift2	(8-78)

dX0,0 = ( −5*aX0,−1 + 46*aX0,0 + 27*aX0,1 – 4*aX0,2 ) >> shift2	(8-79)

eX0,0 = ( −4*aX0,−1 + 36*aX0,0 + 36*aX0,1 – 4*aX0,2 ) >> shift2	(8-80)

fX0,0 = ( −4*aX0,−1 + 27*aX0,0 + 46*aX0,1 – 5*aX0,2 ) >> shift2	(8-81)

gX0,0 = ( −2*aX0,−1 + 16*aX0,0 + 54*aX0,1 – 4*aX0,2 ) >> shift2	(8-82)

hX0,0 = ( −aX0,−1 + 8*aX0,0 + 60*aX0,1 – 3*aX0,2 ) >> shift2	(8-83)

The positions labelled with lower-case letters within un-shaded blocks represent chroma samples at eighth-pel sample fractional locations. The chroma location offset in fractional-sample units ( xFracC, yFracC ) specifies which of the generated chroma samples at full-sample and fractional-sample locations is assigned to the predicted chroma sample value predSampleLXC[ xC, yC ]. This assignment is done according to Table 8-6. The value of predSampleLXC[ xC, yC ] shall be the output.

Table 8-8 – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | B << shift3 | ba | ca | da | ea | fa | ga | ha | aY | bY | cY | dY | eY | fY | gY | hY |

##### Weighted sample prediction process

[Ed.: (KU) only the default weighted prediction part is added for bi-directional averaging]

Inputs to this process are:

–	a location ( xB, yB ) specifying the top-left sample of the current prediction unit relative to the top left sample of the current coding unit,

–	the width and height of this prediction unit, nPSW and nPSH,

–	two (nPSW)x(nPSH) arrays predSamplesL0 and predSamplesL1,

–	prediction list utilization flags, predFlagL0 and predFlagL1,

–	the bit-depth of the chroma component, bitDepth,

–	two 1D interpolation flags interp1DFlagLX.

Outputs of this process are:

–	the (nPSW)x(nPSH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to 14 – bitDepth and the variable shift2 is set equal to 15 – bitDepth,

–	The variable offset1 is set equal to 1 << ( shift1 – 1 ) and the variable offset2 is set equal to 1 << ( shift2 – 1 ).

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x, y ] with x = 0..(nPSW)-1 and y = 0..(nPSH)-1 are derived as follows.

–	If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

predSamples[ x, y ] = ( predSamplesL0[ x, y ] + offset1 ) >> shift1	(8-84)

–	Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

predSamples[ x, y ] = ( predSamplesL1[ x, y ] + offset1 ) >> shift1	(8-85)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1),

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , 
								 ((predSamplesL0[ x, y ]  >> interp1DFlagL0) + (predSamplesL1[ x, y ] ]  >> interp1DFlagL1) + offset2 ) >> shift2 )	(8-86)