| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: November, 14-22 July, 2011 | Document: JCTVC-G737-r2 WG11 Doc: m22306 |
| --- | --- |

| Title: | CE10: Full Factorization Core Transforms for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin, Wonjae Lee JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Kulbhushan Pachauri 560 093 -Bagmane Lakeview B Block, #66/1, Bagmane Tech Park, C V Raman Nagar, Byrasandra Bangalore Karnataka India 560093 Pankaj Topiwala 5840 Banneker Rd., #230, Columbia, MD 21044 USA 750 N. Atlantic Ave., #1504, Cocoa Beach, FL 32931 USA | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.com alexander_b.alshin@samsung.com ellio.lee@samsung.com jeonghoon@samsung.com kb.pachauri@samsung.com pankajtva@gmail.com |
| Source: | Samsung Electronics, Ltd, FastVDO LLC |  |  |

_____________________________

# Abstract

This is the CE10 submission by Samsung and FastVDO, follow-on to F251. All transform components have been retained from F251 (no updates). Since the transform design has a simple matrix multiplication definition, it is so presented, in the Appendix. It is claimed that the proposed scheme supports all three forms of implementation: matrix multiplication, partial butterfly and full-factorization, which are identical in performance. This reported equivalence allows choosing the implementation according to platform: i.e. either matrix multiplication or partial butterfly may be preferred for SIMD implementations, while full-factorization may be preferred for platforms with strict limitations for number of multiplications. By utilizing Loeffler’s factorization in the 32pt transform, it is reported that a 4X reduction in the number of multiplications compared to partial butterfly, and a 12X reduction compared to matrix multiplication, is achieved. Performance testing under common test conditions appears to show virtually identical performance to the HM transforms, in both HM3.0 and HM4.0: difference for Luma component (0.0%), while for Chroma component (-0.1%). Scalar quantization is reportedly unchanged compared to HM design for all transform sizes. This proposal also verifies h/w test performed by independent expert ST Micro; who reported 33-61% G/C reduction for proposed transforms compare to HM transforms. SIMD implementation for proposed core transforms was done by Samsung and compared with SIMD implementation for partial butterfly provided by Cisco. We implemented the proposed design in 4 different equivalent forms: naive and fast matrix multiplication, partial and full butterfly. Our comparison claims that for small block sizes partial butterfly is faster but for large transforms full factorization provides the best speed (up to 20% faster).

# Introduction

HM4.0 uses core transforms introduced by [1]. This transform is an approximation of the DCT. It is implemented in two equivalent forms in HM4.0: as matrix multiplications and as so-called partial butterfly. This convertibility allows a variety of implementations according to platform. Partial butterfly is fast transform implementation providing about 3 times reduction for number of multiplications compared to matrix multiplications form. Quantization process introduced in [1] uses scalar quantization which essentially reduces memory for quantization matrices.

In this proposal we keep all these advantages but use different approximations of the DCT which additionally allow full (Loeffler’s) factorization. Briefly, proposed changes of core transforms compared to [1] can be described in 2 steps. At first step, transform matrix elements were multiplied by 64. Each 1D transform in HM3.0 uses its own de-scaling left shift (in order to keep output in predetermined range). Values of all these shifts were increased by 6 in order to compensate for transform matrix element increment. So dynamic range of each buffer for temporal data storage is not changed compare to HM3.0. The first step of our modifications does not change performance. At second step, the scaled (by 64) transform matrix elements were modified in absolute values in order to allow higher degree of factorization compare to partial butterfly. These modifications lead to performance change.

Modified approximations of DCT matrices still keep their fundamental symmetries. In particular 4x4 transform matrix is embedded into 8x8, 16x16 and 32x32 matrices, 8x8 is embedded into 16x16 and 32x32 and 16x16 transform matrix is part of 32x32 transform.

It develops that the transform matrix for 4x4 is unchanged from the HM in our proposal, while higher transforms are different.

The proposed transforms can be implemented in three ways:

- matrix multiplications,

- partial butterfly,

- full-factorization form.

First and second forms are also supported by HM4.0 transforms, while the third one becomes available with the proposed transform matrices. The code for matrix multiplication and partial butterfly is similar to HM4.0 but uses different values of transform matrices elements. The HM4.0 transform matrix elements are represented in 8 bits. For this proposal, transform matrix elements are represented in 14 bits.

All equivalent forms of the design use multiplications. In matrix multiplication and partial butterfly forms, first multiplier is transform matrix element (14 bits in our design) and another multiplier is de-quantized coefficient or content of temporal buffer after 1st inverse transform. Both are supposed to be within 16 bits. If proposed here transform is implemented as matrix multiplication or partial butterfly then both multipliers don’t exceed 16 bits.

The third form (full-factorization) is targeting for h/w platforms with strict restrictions for number of multiplications (~4X less than partial butterfly). The full-factorization form may have up to two cascade multiplications. One multiplier is 8 bits number in proposed design. Another multiplier is 16 bits number (as well as for partial butterfly) for first multiplication in a path. But for second multiplications in a path one multiplier can be over 16 bits.

Our transform design remains unchanged from the Turin meeting, please refer to [2] for more details.

# HW test

The hardware test was an important part of our CE10 activity. As the 32 pt inverse transform is the bottleneck among transforms in the HM design, this size was the main target of our h/w tests. Independent expert ST Micro performed a key h/w test, testing all proposed designs, with additional results by Samsung and TI.

As it was agreed in Turin, in order to equalize the level of code optimization, an automatic RTL generator was used. Input C-code to the automatic RTL generator was available to all CE10 members in August, with modest updates. Each proponent had enough time to prepare, correct and optimize their code. Actually the only optimization applied (to all designs) was to un-roll all loops; without this the automatic RTL generator fails to generate the code with low latency.

Then ST Mciro and Samsung started automatic RTL generation. ST Micro used SynphonyCC, Samsung used CATAPULT C. Settings for both tools were the same except ST Micro and Samsung used different 45 nm, company-proprietary libraries. An input parameter for both automatic RTL generators was frequency, while latency was an output parameter to both systems.

After RTL generation, the Synopsys Design Compiler was used both by Samsung and ST Micro for synthesis and G/C measurement. However, the precise use of the Compiler differed between the companies (company-proprietary constraints), which details are not releasable. So even with the same RTL code input, we did not expect a perfect match in resulting G/C measurements.

It was agreed in CE10 to represent test data in the form of table (G/C depends on latency L=2,3,…and frequency F=150-500 MHz). Test results provided by ST Micro are shown in the Table 1: HM4.0 and F-251 core transform G/C and ratios between them. Empty cells mean that this combination of frequency/latency was not tested or not available.

RTL generated by SynphonyCC was shared by ST Micro to all CE10 members and Samsung has cross-checked them. First we have checked that RTL and C-code provide identical outputs as transforms. Then we synthesized ST Micro-provided RTL using our constraints of Synopsys Design Compiler. As mentioned, the G/C in our measurement is not the same as that reported by ST Micro (due to the difference in Design Compiler settings); but differences are not substantial, and confirm the performance, but indicate a degree of variability of results based on tools, settings. Cross-check results by Samsung are reported in the right-hand part of Table 1.

Table 1. Gate Count as measured by ST Micro using SynphonyCC RTL Generator and its Compiler settings, and as recompiled by Samsung from ST Micro’s RTLs but Samsung’s Compiler settings.

| HM 4.0 32 pt 1D inverse core transform F ¦ L; 2; 3; 4; 5; 6; 7; F ¦ L; 2; 4; 7. 150; 108364; 134553; 147701; 182063; 150; 112303; 172404; 221962. 200; 110269; 134615; 147711; 182016; 200; 112353; 172402; 221960. 250; 123155; 143120; 151092; 182387; 250; 118946; 172404; 221960. 300; 126322; 143793; 159014; 204536; 300; 175013; 223463. 350; 129150; 153378; 168162; 204752; 350; 237577. 400; 165196; 178534; 216075; 400; 248890. 450; 450; 266743. 500; 500; 272321. |
| --- |
| F-251 32 pt 1D inverse core transform F ¦ L; 2; 3; 4; 5; 6; 7; F¦L; 2; 4; 7. 150; 49647; 57657; 67446; 77805; 81256; 150; 62955; 73963; 106494. 200; 57870; 70142; 68929; 78830; 82065; 200; 71342; 85011; 116988. 250; 71707; 74467; 83120; 84958; 92561; 250; 85515; 91854; 123290. 300; 77493; 81463; 84758; 96416; 98556; 300; 108377; 127537. 350; 86402; 87763; 89053; 100879; 100297; 350; 129365. 400; 103183; 99331; 90470; 101509; 103581; 400; 131448. 450; 450; 134332. 500; 500; 142764. |
| G/C ration between F-251 and HM4.0 32 pt 1D inverse core transform F ¦ L; 2; 3; 4; 5; 6; 7; F ¦ L; 2; 4; 7. 150; 46%; 39%; 45%; 150; 56%; 43%; 48%. 200; 52%; 47%; 45%; 200; 63%; 49%; 53%. 250; 58%; 49%; 51%; 250; 72%; 53%; 56%. 300; 61%; 51%; 48%; 300; 62%; 57%. 350; 67%; 52%; 49%; 350; 54%. 400; 56%; 48%; 400; 53%. 450; 450; 50%. 500; 500; 52%. Based on this measurement we can conclude that area required for full-factorized 32 pt transform is only 39% - 67% of the area required for partial butterfly (E243, or HM). We note that with higher frequencies, the gap appears to narrow a bit (though at latency L=7 there is not much trend with frequency). In addition to retesting ST Micro’s RTL, Samsung independently generated RTLs using CATAPULT C, and proceeded to use Synopsys Design Compiler to measured G/C the same way as ST Micro. Test results are reported in Table 2. Table 2. Gate Count as measured by Samsung’s CATAPULT C RTL Generator, and Samsung’s Compiler settings. These results are in addition to the retest of ST Micro’s results of Table 1. HM 4.0 32 pt 1D inverse core transform F ¦ L; 2; 3; 4; 5; 6; 7. 150; 91233; 89058; 89071; 97726; 100014. 200; 105970; 105196; 111071; 111407. 250; 128179; 116126; 119688; 120154. 300; 128587; 126759. 350; 139775. 400. 450. 500. |
| F-251 32 pt 1D inverse core transform F ¦ L; 2; 3; 4; 5; 6; 7. 150; 119476; 127993; 139148; 153902. 200; 149149; 161630; 168979. 250; 163631; 178468; 185112. 300; 199785. 350. 400. 450. 500. |
| G/C ration between F-251 and HM4.0 32 pt 1D inverse core transform F ¦ L; 2; 3; 4; 5; 6; 7. 150; 76%; 70%; 64%; 63%. 200; 71%; 65%; 66%. 250; 78%; 65%; 65%. 300; 64%. 350. 400. 450. 500. As we can see, these results using RTL generated by CATAPULT C is not as good as those generated by SynphonyCC (which is known as a very powerful tool). So we also tried to make RTL manually. This was naive straight-forward conversion from C-code; the hand-written RTL is not releasable. With simple hand-written RTL, we could reduce the G/C as compared to the RTL from CATAPULT C, and make it work at a slightly higher frequency (with CATAPULT, we could not reach F=200 for both F251 and HM). However, we could not match the gate counts guaranteed by SynphonyCC RTL, nor reach the higher frequencies at low latencies that it did (e.g., F=350 for HM, and F=400, for L=2). These results suggest that the SynphonyCC is a powerful automatic RTL generator, whose results appear to be more fully optimized, and therefore more reliable as indicators of true performance. The results generated by ST Micro using SynphonyCC are also the only ones in all the hardware tests done in CE10 that have been cross-checked. Table 3. G/C as measured by Samsung using its hand-written RTL, for L=2. F | F251 | HM | F251/HM |
| 150 | 78935 | 111283 | 71% |
| 200 | 86867 | 111918 | 78% |

# SIMD implementation study

In order to study the s/w impat of full-factorized transforms, a SIMD implementation was done. This is standalone application realizes 2D inverse transform (both vertical and horizontal). We run it in a loop. Loop size is 10 000 000. We measured run-time and computed clock cycles for one 2D transform.

We studied 4 forms of transform implementations:

- “Mat Normal”: Normal (naive) matrix multiplication

- “Mat Fast”: Fast matrix multiplication

- “PB”: Partal Buterfly

- “FB”: Full Butterfly

Remarks:

- “Mat Normal”and “Mat Fast” can be applied to any kind of matrix multiplication, in particular to E243 or F251 (both with the same speed).

- PB utilizes symmetry and anti-symmetry of DCT coefficients, and again there is no difference between E243 and F251 if both are realized in PB form.

- FB is supported by F-251 only (among these two).

Test results are summarized in the Table 4.

We also investigated the SIMD code kindly provided by Cisco, who implemented a PB form (no difference between F446 and F251 in this case). Table 4 includes s/w test results reported by Cisco in [5].

For 16pt and 32 pt FB is the fastest one in our test. For smaller sizes (like 4 and 8) we have not seen much difference between PB and FB.

In this table, "C" means that plain C code and not SIMD-optimized code was used; we show this data just to illustrate how much SIMD optimization speeds-up the process (about 4-5X).

We try to keep the same level of optimization for all implementation forms. In any case, no specific additional optimization was applied in the FB implementation as compared to PB.

There are 2 different categories of multiplications in FB. For the first category multipliers are 16 bits, but for the second category multipliers can be over 16 bits. For these two categories of multiplication different SIMD instructions are needed (pmaddwd and pmulld). For PB all multipliers don’t exceed 16 bits (only pmulld is enough). The latency pmaddwd and pmulld instructions have depends on processor type: for Intel’s Nehalem and Sandy Bridge this latency is different. So clock cycles estimated with another processor will be different from reported.

The setup used for this test is Ubutnu Linux 11.10 (64 bit), running on an Intel® Core™ i5 chip @2.66GHz , with 4GB of RAM.

Table 4. S/w test results.

| Code Proponent | Samsung | Samsung | Cisco | Cisco | Samsung |
| --- | --- | --- | --- | --- | --- |
| Tester | Samsung | Samsung | Cisco | Samsung | Cisco |
| System Details | Intel i5, 2.66GHz, 4GB RAM, Ubuntu 9.10 (64bit) | Intel i7(Sandybridge), 4.4GHz, 4GB RAM, Ubuntu 11.10 (64bit) | Intel i5, ??, ?? RAM, Win7 (64bit), Intel Compiler | Intel i5, 2.66GHz, 4GB RAM, Win7 (32bit), Intel Compiler |  |
| 8pt |  |  |  |  |  |
| C Mat | 1308 | 831 |  |  |  |
| C PB | 778 | 763 |  |  |  |
| C FB | 417 | 381 |  |  |  |
| SSE Mat Normal | 685 | 672 |  |  |  |
| SSE Mat Fast | 290 | 249 |  |  |  |
| SSE PB | 156 | 144 | 173 | 187 |  |
| SSE FB | 197 | 167 |  |  |  |
| 16pt |  |  |  |  |  |
| C Mat | 11230 | 5700 |  |  |  |
| C PB | 4852 | 5079 |  |  |  |
| C FB | 3176 | 3141 |  |  |  |
| SSE Mat Normal | 3349 | 3116 |  |  |  |
| SSE Mat Fast | 1736 | 1479 |  |  |  |
| SSE PB | 933 | 861 | 950 | 1038 |  |
| SSE FB | 1041 | 898 |  |  |  |
| 32pt |  |  |  |  |  |
| C Mat | 209491 | 58383 |  |  |  |
| C PB | 26543 | 23631 |  |  |  |
| C FB | 18042 | 18254 |  |  |  |
| SSE Mat Normal | 17494 | 15938 |  |  |  |
| SSE Mat Fast | 14512 | 11473 |  |  |  |
| SSE PB | 6664 | 5918 | 6781 | 7156 |  |
| SSE FB | 6110 | 4912 |  |  |  |

Some updated results below.

Based on s/w test results in Table 4 we can conclude the following. Among all possible variation PB and FB look preferable. Which one among them is better depends on TU size. In our tests for 16pt and 8 pt PB provides the best speed, but for 32pt already FB is faster (up to 20% with Intel i7 Sandybridge). As we may see in Table 4 SIMD code speed is dependent on processor type. But tendency is the same: if one implementation form is fastest for one processor then that tends to hold with other processors as well.

This highlights the advantages of being able to convert between different equivalent forms to obtain an optimal design for a variety of architectures.

# Number of operations

As mentioned above, the F251 core transforms are very flexible and can be implemented in different forms from naive matrix multiplications to full factorization. Full factorization is a cascace of butterflies and Givens rotations. Each Givens rotation can be also implemented with 2 forms: 4 mults and 2 additions or 3 mults and 3 additions.

Table 5. Multiplications number comparison for different implementation forms.

| Number of mults in 2D transform | HM4.0 and G737 |  | Full-factorization G737 |  |
| --- | --- | --- | --- | --- |
|  | Matrix multiplication | Partial-butterfly | 4 mults per Givens rotation | 3 mults per Givens rotation |
| 8-point | 1024 | 384 | 224 | 176 |
| 16-point | 8192 | 2816 | 1216 | 992 |
| 32-point | 65536 | 22016 | 6912 | 5568 |

As mentioned in sections 2-3, there is no more than 2 multiplication stages in the longest execution path. In the worst case execution path uses 2 multiplications and 5 additions. There are no left (de-scaling) shifts at any intermediate stages.

We now analyze some information presented at this meeting that aims to reduce the complexity of implementations. In particular, [6] and [7] have several nice ideas for h/w simplifications that we examine.

Reference [6] claims that because there are only a few distinct coefficients in each row/column of transforms, that fact can be utilized in h/w implementation. The number of different multipliers (transform coefficients) determines the number of multiplier modules needed. And these multiplier modules can also be implemented in shifts and adds if needed. We’ve borrowed Table 6 from [6], which gives the number of distinct h/w multiplier modules needed for each size transform in the HM. We have also added the same analysis for the full-factorized form for G737.

Table 6. Number of multiplier modules needed in h/w for HM from [6], with G737 data added.

| N | HM-4.0 | G737 (all mults) | G737 (w/ some shifts) |
| --- | --- | --- | --- |
| 4 | 3 | 3 | 2 |
| 8 | 7 | 7 | 6 |
| 16 | 15 | 14 | 13 |
| 32 | 29 | 32 | 30 |

We provide C-code for the full-factorized 32 pt 1D (row/column) inverse transform. This code was used as input for the automatic RTL generator (both by ST Micro and Samsung). We note that the G737 full-factorized transform has a fully embedded structure, meaning that the 4pt is within the 8pt, and so on. So the 32pt transform reuses code for all transform sizes, and so reuses multiplier modules as well. So it is enough to analyze 32pt transform to understand the complexity also of the smaller transform sizes.

All multipliers used in our design are listed in Table 7. We grouped them according to transform size. 32 pt transform uses them all, 16 pt uses only 4x4, 8x8 and 16x16 constants, etc. No other multipliers are needed in h/w implementation (please refer the C-code in APPENDIX). Some of constants are the same, some are power of 2, so multiplication can be implemented as add/shifts only. The numbers of different multipliers in our design according to the transform sizes are shown in the Table 6: with and without taking into account that “*64” and “*16” can be implemented as shifts.

Table 7. Table of multipliers in the proposed full-factorized transform design.

/// 4x4 /// constants 83 = cos( 8*/32)*64*sqrt(2.0) 64 = cos(16*/32)*64*sqrt(2.0) 36 = cos(24*/32)*64*sqrt(2.0) /// 8x8 /// constants 90 = sqrt(2.0) * 64 63 = cos( 4*/64)*64 53 = cos(12*/64)*64 36 = cos(20*/64)*64 12 = cos(28*/64)*64 /// 16x16 /// constants 64 = cos(2 */64)*64 61 = cos(6 */64)*64 56 = cos(10*/64)*64 49 = cos(14*/64)*64 41 = cos(18*/64)*64 30 = cos(22*/64)*64 19 = cos(26*/64)*64 6 = cos(30*/64)*64 /// 32x32 /// constants 64 = cos(1 */64)*64 63 = cos(3 */64)*64 62 = cos(5 */64)*64 60 = cos(7 */64)*64 58 = cos(9 */64)*64 55 = cos(11*/64)*64 51 = cos(13*/64)*64 47 = cos(15*/64)*64 43 = cos(17*/64)*64 38 = cos(19*/64)*64 33 = cos(21*/64)*64 27 = cos(23*/64)*64 22 = cos(25*/64)*64 16 = cos(27*/64)*64 9 = cos(29*/64)*64 3 = cos(31*/64)*64 89 = cos( 4*/64)*64*sqrt(2.0) 75 = cos(12*/64)*64*sqrt(2.0) 50 = cos(20*/64)*64*sqrt(2.0) 18 = cos(28*/64)*64*sqrt(2.0)

On a different topic, [7] claims that some area can be shared between forward and inverse transform in h/w implementation of HM4.0 transforms. This is both interesting and valuable.  Because multipliers in forward and inverse trasnform are the same in our design, all muptiplier modules can be shared between forward and inverse trasnform as well, which can lead to some area sharing.

Such an analysis would be difficult for an automatic RTL generator to take this into account. So sucha  detailed h/w test for joint forward/inverse trasnform design probably cannot be done systematically inside this CE (no hand written RTL sharing is possible, and so no cross-checks can be done). Nevertheless, they provide useful insights on possible further optimizations available.

# Quantization/De-quantization matrices

There is no change of quantization process compare to HM4.0:  6 scalar values for quantization and 6 scalar values for de-quantization.

# Coding Performance test

According to CE10 requirements [3] all proposal should be tested using implementation based on HM3.0. Since our design and code was not changed performance is un-changed compared to that reported in [2]. Additionally we provide HM4.0 based implementation (attached to this contribution). It was tested under common test conditions [4] (including class F and LD(P), which are optional). Table 4 provides test results for normal QP range

Table 4. Normal QP test for HM4.0 based implementation.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.1% | -0.1% | 0.3% | 0.0% | -0.1% |
| Class B | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | 0.0% |
| Class F | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.1% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 99% |  |  | 96% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | -0.3% | 0.1% | 0.2% | 0.0% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.4% | 0.0% | -0.1% | 0.1% |
| Class C | 0.1% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.4% | -0.3% |
| Class E | 0.1% | 0.5% | 0.4% | 0.0% | -0.1% | -0.3% |
| Class F | -0.1% | -0.4% | -0.3% | 0.1% | 0.2% | 0.1% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.2% | -0.1% | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | -0.4% | -0.8% | 0.0% | -0.1% | -0.4% |
| Class E | 0.0% | 1.1% | -0.5% | 0.0% | -0.5% | -0.5% |
| Class F | 0.0% | -0.2% | -0.3% | 0.0% | 0.3% | 0.1% |
| Overall | 0.0% | 0.1% | -0.4% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.1% | -0.4% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Performance is pretty similar to the reported in [2] for HM3.0 based implementation: 0.0% in avg. for Luma, with some improvement for Chroma (0.1% gain in avg).

# Description for Working Draft

10.1. Basis vectors

The transform matrices  the 4x4, 8x8, 16x6 and 32x32 transforms are shown in Appendix A.

10.2 Intermediate scaling in inverse transform and dequantization.

Definitions

B	= internal bit depth (as specified by InternalBitDepth in the common config files)

W, H 	= transform size (horizontal and vertical)

Q	= f(QP%6), 	where f(x)   =   {26214,23302,20560,18396,16384,14564}, x=0,…,5

IQ 	= g(QP%6), 	where g(x)  =   {40,45,51,57,64,72}, x=0,…,5

Inverse transform consists of 3 steps: dequantizer, 1st (column) and second (row) inverse transforms. Output of each step is storied in TU size temporal buffer of 16 bits size. Intermediate calculation requires 32 bits precision.

The dequantizer is specified as follows

coeffQ 	=  ((level*IQ << (QP/6)) + offset)>>(M-1+(B-8)),  offset = 1<<(M-2+(B-8)),

coeffQ = min(32767,max(-32768,coeffQ)).

Here level is quantized coefficients, M is

M	= log2(W*H)/2

The clipping ensures that the value of the quantized transform coefficients, coeffQ, can be represented with 16 bits.

The inverse transform is specified as a matrix multiplication in two stages, horizontal and vertical.

The first stage is column transform:

.

After the first stage, the resulting transform coefficients are de-scaled as follows

temp  = (register1 + offset1)>>shift1)

shift1 = 13; offset1 = 1<<( shift1-1).

The first stage is row transform:

.

After the second stage, the resulting transform coefficients are scaled as follows

rec_residual  =  ( + offset2)>>shift2;

shift2=(18-(B-8)),offset2 = 1<<( shift2-1).

# Conclusion

# Based on test results:

# 0.0%(Y) 0.0% (U) -0.1%(V) (average performance across all 8 test cases in normal QP tests)

# we would like to recommend adoption for proposed core transform modification for HM5.0 and WD.

# This will keep possibility of 2 equivalent implementations:

# matrix multiplication

# partial butterfly

# which are available in HM4.0 and add one more equivalent implementation

# full-factorized form

# which uses 4 times smaller number of multiplications in 32 pt transform compare to default HM4.0 configuration (partial butterfly) and so 20% faster in SIMD implementation (by Samsung’s measurement) and requires 2X less G/C in h/w implementation (according to ST Micro’s independent analysis).

# References

[1] A. Fuldseth, G. Bjøntegaard (Cisco), M. Sadafale, M. Budagavi (TI), “Transform design for HEVC with 16 bit intermediate data representation”, JCTVC-E243, 5th. JCTVC, Meeting, Geneva, Mar. 2011

[2] E.Alshina, A.Alshin, I-K. Kim, P. Topiwala “CE10: Full-factorized core transform proposal by Samsung/FastVDO”, JCTVC-F251, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

[3] P. Topiwala, M. Budagavi, A. Fuldseth, R. Joshi, I.-K. Kim, “CE10: Core Transform Design”JCTVC-F910, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

[4] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

[5] A. Fuldseth, L. P. Endresen, S. Selnes (Cisco), V. Arbatov, F. Franchetti (SpiralGen), M. Puschel (ETH Zurich) “SIMD optimization of proposed HEVC core transforms” , JCTVC-G497, 7th JCT-VC Meeting, Geneva, Switzerland, November, 2011

[6] M. Tikekar, C.-T. Huang, C. Juvekar, A. Chandrakasan (MIT) “Core Transform Property for Practical Throughput Hardware Design” , JCTVC-G265, 7th JCT-VC Meeting, Geneva, Switzerland, November, 2011

[7] M. Budagavi (TI), V. Sze (TI), M. Sadafale  “Hardware analysis of transform and quantization” , JCTVC-G132, 7th JCT-VC Meeting, Geneva, Switzerland, November, 2011

# Patent rights declaration(s)

Samsung Electronics, Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

FastVDO LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

APPENDIX  A

Transform matrices for proposed core transform.

DST:  =

{

{1856,   3520,    4736,   5376},

{4736,   4736,       0,  -4736},

{5376,  -1856,   -4736,   3520},

{3520,  -5376,    4736,  -1856}};

=

{

{ 4096, 4096, 4096, 4096},

{ 5312, 2304,-2304,-5312  },

{ 4096,-4096,-4096, 4096},

{ 2304,-5312, 5312,-2304  }

};

=

{

{ 4096, 4096, 4096, 4096, 4096, 4096, 4096, 4096,},

{ 5696, 4800, 3264, 1088,-1088,-3264,-4800,-5696,},

{ 5312, 2304,-2304,-5312,-5312,-2304, 2304, 5312,},

{ 4770,-1080,-5670,-3240, 3240, 5670, 1080,-4770,},

{ 4096,-4096,-4096, 4096, 4096,-4096,-4096, 4096,},

{ 3240,-5670, 1080, 4770,-4770,-1080, 5670,-3240,},

{ 2304,-5312, 5312,-2304,-2304, 5312,-5312, 2304,},

{ 1088,-3264, 4800,-5696, 5696,-4800, 3264,-1088,},};

=

{

{	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	},

{	5760,	5504,	5120,	4480,	3712,	2688,	1664,	512,	-512,	-1664,	-2688,	-3712,	-4480,	-5120,	-5504,	-5760,	},

{	5696,	4800,	3264,	1088,	-1088,	-3264,	-4800,	-5696,	-5696,	-4800,	-3264,	-1088,	1088,	3264,	4800,	5696,	},

{	5543,	3568,	619,	-2802,	-5096,	-5747,	-4506,	-1639,	1639,	4506,	5747,	5096,	2802,	-619,	-3568,	-5543,	},

{	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	},

{	5167,	474,	-4379,	-5528,	-1806,	3773,	5728,	2591,	-2591,	-5728,	-3773,	1806,	5528,	4379,	-474,	-5167,	},

{	4770,	-1080,	-5670,	-3240,	3240,	5670,	1080,	-4770,	-4770,	1080,	5670,	3240,	-3240,	-5670,	-1080,	4770,	},

{	4410,	-2700,	-5490,	540,	5760,	1710,	-5040,	-3690,	3690,	5040,	-1710,	-5760,	-540,	5490,	2700,	-4410,	},

{	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	},

{	3690,	-5040,	-1710,	5760,	-540,	-5490,	2700,	4410,	-4410,	-2700,	5490,	540,	-5760,	1710,	5040,	-3690,	},

{	3240,	-5670,	1080,	4770,	-4770,	-1080,	5670,	-3240,	-3240,	5670,	-1080,	-4770,	4770,	1080,	-5670,	3240,	},

{	2591,	-5728,	3773,	1806,	-5528,	4379,	474,	-5167,	5167,	-474,	-4379,	5528,	-1806,	-3773,	5728,	-2591,	},

{	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	},

{	1639,	-4506,	5747,	-5096,	2802,	619,	-3568,	5543,	-5543,	3568,	-619,	-2802,	5096,	-5747,	4506,	-1639,	},

{	1088,	-3264,	4800,	-5696,	5696,	-4800,	3264,	-1088,	-1088,	3264,	-4800,	5696,	-5696,	4800,	-3264,	1088,	},

{	512,	-1664,	2688,	-3712,	4480,	-5120,	5504,	-5760,	5760,	-5504,	5120,	-4480,	3712,	-2688,	1664,	-512,	},

};

=

{

{	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	},

{	5760,	5696,	5632,	5440,	5248,	4992,	4608,	4288,	3904,	3456,	2944,	2432,	1984,	1408,	832,	256,	-256,	-832,	-1408,	-1984,	-2432,	-2944,	-3456,	-3904,	-4288,	-4608,	-4992,	-5248,	-5440,	-5632,	-5696,	-5760,	},

{	5760,	5504,	5120,	4480,	3712,	2688,	1664,	512,	-512,	-1664,	-2688,	-3712,	-4480,	-5120,	-5504,	-5760,	-5760,	-5504,	-5120,	-4480,	-3712,	-2688,	-1664,	-512,	512,	1664,	2688,	3712,	4480,	5120,	5504,	5760,	},

{	5675,	5223,	4301,	3000,	1350,	-308,	-1935,	-3425,	-4650,	-5445,	-5806,	-5600,	-4925,	-3927,	-2464,	-875,	875,	2464,	3927,	4925,	5600,	5806,	5445,	4650,	3425,	1935,	308,	-1350,	-3000,	-4301,	-5223,	-5675,	},

{	5696,	4800,	3264,	1088,	-1088,	-3264,	-4800,	-5696,	-5696,	-4800,	-3264,	-1088,	1088,	3264,	4800,	5696,	5696,	4800,	3264,	1088,	-1088,	-3264,	-4800,	-5696,	-5696,	-4800,	-3264,	-1088,	1088,	3264,	4800,	5696,	},

{	5622,	4249,	2002,	-933,	-3378,	-5246,	-5760,	-4953,	-3039,	-270,	2478,	4710,	5727,	5456,	3893,	1332,	-1332,	-3893,	-5456,	-5727,	-4710,	-2478,	270,	3039,	4953,	5760,	5246,	3378,	933,	-2002,	-4249,	-5622,	},

{	5543,	3568,	619,	-2802,	-5096,	-5747,	-4506,	-1639,	1639,	4506,	5747,	5096,	2802,	-619,	-3568,	-5543,	-5543,	-3568,	-619,	2802,	5096,	5747,	4506,	1639,	-1639,	-4506,	-5747,	-5096,	-2802,	619,	3568,	5543,	},

{	5449,	2865,	-759,	-4329,	-5772,	-4570,	-1521,	2553,	5204,	5553,	3560,	-334,	-3842,	-5753,	-4990,	-1877,	1877,	4990,	5753,	3842,	334,	-3560,	-5553,	-5204,	-2553,	1521,	4570,	5772,	4329,	759,	-2865,	-5449,	},

{	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	},

{	5261,	1318,	-3377,	-5786,	-3986,	904,	4905,	5420,	2055,	-3015,	-5722,	-4188,	153,	4719,	5601,	2353,	-2353,	-5601,	-4719,	-153,	4188,	5722,	3015,	-2055,	-5420,	-4905,	-904,	3986,	5786,	3377,	-1318,	-5261,	},

{	5167,	474,	-4379,	-5528,	-1806,	3773,	5728,	2591,	-2591,	-5728,	-3773,	1806,	5528,	4379,	-474,	-5167,	-5167,	-474,	4379,	5528,	1806,	-3773,	-5728,	-2591,	2591,	5728,	3773,	-1806,	-5528,	-4379,	474,	5167,	},

{	4957,	-300,	-5225,	-4676,	878,	5450,	4275,	-1419,	-5642,	-3825,	1900,	5736,	3447,	-2475,	-5725,	-2981,	2981,	5725,	2475,	-3447,	-5736,	-1900,	3825,	5642,	1419,	-4275,	-5450,	-878,	4676,	5225,	300,	-4957,	},

{	4770,	-1080,	-5670,	-3240,	3240,	5670,	1080,	-4770,	-4770,	1080,	5670,	3240,	-3240,	-5670,	-1080,	4770,	4770,	-1080,	-5670,	-3240,	3240,	5670,	1080,	-4770,	-4770,	1080,	5670,	3240,	-3240,	-5670,	-1080,	4770,	},

{	4626,	-1887,	-5808,	-1417,	4966,	4362,	-2538,	-5747,	-821,	5184,	3834,	-2978,	-5621,	-330,	5451,	3452,	-3452,	-5451,	330,	5621,	2978,	-3834,	-5184,	821,	5747,	2538,	-4362,	-4966,	1417,	5808,	1887,	-4626,	},

{	4410,	-2700,	-5490,	540,	5760,	1710,	-5040,	-3690,	3690,	5040,	-1710,	-5760,	-540,	5490,	2700,	-4410,	-4410,	2700,	5490,	-540,	-5760,	-1710,	5040,	3690,	-3690,	-5040,	1710,	5760,	540,	-5490,	-2700,	4410,	},

{	4230,	-3420,	-4950,	2430,	5400,	-1440,	-5670,	270,	5760,	810,	-5580,	-1980,	5220,	2970,	-4590,	-3870,	3870,	4590,	-2970,	-5220,	1980,	5580,	-810,	-5760,	-270,	5670,	1440,	-5400,	-2430,	4950,	3420,	-4230,	},

{	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	},

{	3870,	-4590,	-2970,	5220,	1980,	-5580,	-810,	5760,	-270,	-5670,	1440,	5400,	-2430,	-4950,	3420,	4230,	-4230,	-3420,	4950,	2430,	-5400,	-1440,	5670,	270,	-5760,	810,	5580,	-1980,	-5220,	2970,	4590,	-3870,	},

{	3690,	-5040,	-1710,	5760,	-540,	-5490,	2700,	4410,	-4410,	-2700,	5490,	540,	-5760,	1710,	5040,	-3690,	-3690,	5040,	1710,	-5760,	540,	5490,	-2700,	-4410,	4410,	2700,	-5490,	-540,	5760,	-1710,	-5040,	3690,	},

{	3452,	-5451,	-330,	5621,	-2978,	-3834,	5184,	821,	-5747,	2538,	4362,	-4966,	-1417,	5808,	-1887,	-4626,	4626,	1887,	-5808,	1417,	4966,	-4362,	-2538,	5747,	-821,	-5184,	3834,	2978,	-5621,	330,	5451,	-3452,	},

{	3240,	-5670,	1080,	4770,	-4770,	-1080,	5670,	-3240,	-3240,	5670,	-1080,	-4770,	4770,	1080,	-5670,	3240,	3240,	-5670,	1080,	4770,	-4770,	-1080,	5670,	-3240,	-3240,	5670,	-1080,	-4770,	4770,	1080,	-5670,	3240,	},

{	2981,	-5725,	2475,	3447,	-5736,	1900,	3825,	-5642,	1419,	4275,	-5450,	878,	4676,	-5225,	300,	4957,	-4957,	-300,	5225,	-4676,	-878,	5450,	-4275,	-1419,	5642,	-3825,	-1900,	5736,	-3447,	-2475,	5725,	-2981,	},

{	2591,	-5728,	3773,	1806,	-5528,	4379,	474,	-5167,	5167,	-474,	-4379,	5528,	-1806,	-3773,	5728,	-2591,	-2591,	5728,	-3773,	-1806,	5528,	-4379,	-474,	5167,	-5167,	474,	4379,	-5528,	1806,	3773,	-5728,	2591,	},

{	2353,	-5601,	4719,	-153,	-4188,	5722,	-3015,	-2055,	5420,	-4905,	904,	3986,	-5786,	3377,	1318,	-5261,	5261,	-1318,	-3377,	5786,	-3986,	-904,	4905,	-5420,	2055,	3015,	-5722,	4188,	153,	-4719,	5601,	-2353,	},

{	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	},

{	1877,	-4990,	5753,	-3842,	334,	3560,	-5553,	5204,	-2553,	-1521,	4570,	-5772,	4329,	-759,	-2865,	5449,	-5449,	2865,	759,	-4329,	5772,	-4570,	1521,	2553,	-5204,	5553,	-3560,	-334,	3842,	-5753,	4990,	-1877,	},

{	1639,	-4506,	5747,	-5096,	2802,	619,	-3568,	5543,	-5543,	3568,	-619,	-2802,	5096,	-5747,	4506,	-1639,	-1639,	4506,	-5747,	5096,	-2802,	-619,	3568,	-5543,	5543,	-3568,	619,	2802,	-5096,	5747,	-4506,	1639,	},

{	1332,	-3893,	5456,	-5727,	4710,	-2478,	-270,	3039,	-4953,	5760,	-5246,	3378,	-933,	-2002,	4249,	-5622,	5622,	-4249,	2002,	933,	-3378,	5246,	-5760,	4953,	-3039,	270,	2478,	-4710,	5727,	-5456,	3893,	-1332,	},

{	1088,	-3264,	4800,	-5696,	5696,	-4800,	3264,	-1088,	-1088,	3264,	-4800,	5696,	-5696,	4800,	-3264,	1088,	1088,	-3264,	4800,	-5696,	5696,	-4800,	3264,	-1088,	-1088,	3264,	-4800,	5696,	-5696,	4800,	-3264,	1088,	},

{	875,	-2464,	3927,	-4925,	5600,	-5806,	5445,	-4650,	3425,	-1935,	308,	1350,	-3000,	4301,	-5223,	5675,	-5675,	5223,	-4301,	3000,	-1350,	-308,	1935,	-3425,	4650,	-5445,	5806,	-5600,	4925,	-3927,	2464,	-875,	},

{	512,	-1664,	2688,	-3712,	4480,	-5120,	5504,	-5760,	5760,	-5504,	5120,	-4480,	3712,	-2688,	1664,	-512,	-512,	1664,	-2688,	3712,	-4480,	5120,	-5504,	5760,	-5760,	5504,	-5120,	4480,	-3712,	2688,	-1664,	512,	},

{	256,	-832,	1408,	-1984,	2432,	-2944,	3456,	-3904,	4288,	-4608,	4992,	-5248,	5440,	-5632,	5696,	-5760,	5760,	-5696,	5632,	-5440,	5248,	-4992,	4608,	-4288,	3904,	-3456,	2944,	-2432,	1984,	-1408,	832,	-256,	},

};

APPENDIX B: c-code used in h/w test for 32 pt full-factorized transform,

void fullButterflyInverse32(short src[32],short des[32])

{

short Oin[16];

short EOin[8];

short EEOin[4];

short EEEEin[2];

short EEEOin[2];

int E[16];

int O[16];

int EE[8];

int EO[8];

int EEE[4];

int EEO[4];

int EEEE[2];

int EEEO[2];

int shift = 18;   // 13 (1st) but  18 (2nd)

int add = 131072;  // 4096 (1st) but  131072(2nd)

int F_16_odd[8];

int FF_16_odd[12];

int EF_16_odd[12];

int D_16_odd[16];

int C_16_odd[10];

int B_16_odd[10];

int A_16_odd[16];

int A_8_odd[8];

int B_8_odd[4];

int BC_8_odd[4];

int C_8_odd[4];

int CO_8_odd[4];

int A_4_odd[4];

int B_4_odd[4];

int A_2_even[2];

int A_2_odd[2];

Oin[ 0] = src[ 1];

Oin[ 1] = src[ 3];

Oin[ 2] = src[ 5];

Oin[ 3] = src[ 7];

Oin[ 4] = src[ 9];

Oin[ 5] = src[11];

Oin[ 6] = src[13];

Oin[ 7] = src[15];

Oin[ 8] = src[17];

Oin[ 9] = src[19];

Oin[10] = src[21];

Oin[11] = src[23];

Oin[12] = src[25];

Oin[13] = src[27];

Oin[14] = src[29];

Oin[15] = src[31];

EOin[ 0] = src[ 2];

EOin[ 1] = src[ 6];

EOin[ 2] = src[10];

EOin[ 3] = src[14];

EOin[ 4] = src[18];

EOin[ 5] = src[22];

EOin[ 6] = src[26];

EOin[ 7] = src[30];

EEOin[ 0] = src[ 4];

EEOin[ 1] = src[12];

EEOin[ 2] = src[20];

EEOin[ 3] = src[28];

EEEOin[0] = src[ 8] ;	EEEOin[1] = src[24] ;

EEEEin[0] = src[ 0] ;	EEEEin[1] = src[16] ;

F_16_odd[ 0] = (Oin[11]+Oin[ 3]);

F_16_odd[ 1] = (Oin[11]-Oin[ 3]);

F_16_odd[ 2] = (Oin[12]+Oin[ 4]);

F_16_odd[ 3] = (Oin[12]-Oin[ 4]);

F_16_odd[ 4] = (Oin[ 9]+Oin[ 6]);

F_16_odd[ 5] = (Oin[ 9]-Oin[ 6]);

F_16_odd[ 6] = (Oin[13]+Oin[ 2]);

F_16_odd[ 7] = (Oin[13]-Oin[ 2]);

FF_16_odd[0] = 90*Oin[ 7];

FF_16_odd[1] = 90*Oin[ 8];

FF_16_odd[2] =   18 *Oin[ 1] - 89*Oin[14];

FF_16_odd[3] =   89 *Oin[ 1] + 18*Oin[14];

FF_16_odd[4] =   75 *Oin[ 1] - 50*Oin[14];

FF_16_odd[5] =   50 *Oin[ 1] + 75*Oin[14];

FF_16_odd[6] =  -75 *Oin[ 5] - 50*Oin[10];

FF_16_odd[7] =   50 *Oin[ 5] - 75*Oin[10];

FF_16_odd[8] =   89 *Oin[ 5] - 18*Oin[10];

FF_16_odd[9] =   18 *Oin[ 5] + 89*Oin[10];

FF_16_odd[10] = (Oin[ 0]+Oin[ 15])<<6;

FF_16_odd[11] = (Oin[ 0]-Oin[ 15])<<6;

EF_16_odd[0] =   83 *F_16_odd[ 0] - 36*F_16_odd[3];

EF_16_odd[7] =   36 *F_16_odd[ 0] + 83*F_16_odd[3];

EF_16_odd[1] =  -83 *F_16_odd[ 2] + 36*F_16_odd[1];

EF_16_odd[6] =  -36 *F_16_odd[ 2] - 83*F_16_odd[1];

EF_16_odd[3] =   36 *F_16_odd[ 4] - 53*F_16_odd[5];

EF_16_odd[2] =   53 *F_16_odd[ 4] + 36*F_16_odd[5];

EF_16_odd[5] =   12 *F_16_odd[ 6] - 63*F_16_odd[7];

EF_16_odd[4] =   63 *F_16_odd[ 6] + 12*F_16_odd[7];

EF_16_odd[8] =  -63 *F_16_odd[ 4] + 12*F_16_odd[5];

EF_16_odd[9] =  -12 *F_16_odd[ 4] - 63*F_16_odd[5];

EF_16_odd[10] =  -36 *F_16_odd[ 6] - 53*F_16_odd[7];

EF_16_odd[11] =   53 *F_16_odd[ 6] - 36*F_16_odd[7];

D_16_odd[ 0] = EF_16_odd[ 2] + FF_16_odd[ 4];

D_16_odd[ 5] = EF_16_odd[ 3] + FF_16_odd[ 5];

D_16_odd[ 1] = EF_16_odd[ 4] + FF_16_odd[ 8];

D_16_odd[ 4] = EF_16_odd[ 5] + FF_16_odd[ 9];

D_16_odd[ 2] = FF_16_odd[ 0] + EF_16_odd[ 0];

D_16_odd[ 3] = FF_16_odd[ 1] + EF_16_odd[ 1];

D_16_odd[ 6] = FF_16_odd[ 1] - EF_16_odd[ 6];

D_16_odd[ 7] = FF_16_odd[ 1] + EF_16_odd[ 6];

D_16_odd[ 8] = EF_16_odd[ 7] - FF_16_odd[ 0];

D_16_odd[ 9] = EF_16_odd[ 7] + FF_16_odd[ 0];

D_16_odd[10] = FF_16_odd[ 1] - EF_16_odd[ 1];

D_16_odd[11] = FF_16_odd[ 0] - EF_16_odd[ 0];

D_16_odd[12] = FF_16_odd[ 3] + EF_16_odd[ 8];

D_16_odd[14] = FF_16_odd[ 2] + EF_16_odd[ 9];

D_16_odd[13] = EF_16_odd[10] + FF_16_odd[ 7];

D_16_odd[15] = EF_16_odd[11] + FF_16_odd[ 6];

C_16_odd[ 0] =  FF_16_odd[11] + D_16_odd[10];

C_16_odd[ 1] =  FF_16_odd[10] + D_16_odd[11];

C_16_odd[ 2] =  FF_16_odd[10] + D_16_odd[ 8];

C_16_odd[ 3] =  FF_16_odd[10] - D_16_odd[ 9];

C_16_odd[ 4] =  FF_16_odd[11] - D_16_odd[ 6];

C_16_odd[ 5] =  FF_16_odd[11] - D_16_odd[ 7];

C_16_odd[ 6] =  FF_16_odd[10] + D_16_odd[ 0];

C_16_odd[ 7] =  FF_16_odd[11] + D_16_odd[ 0];

C_16_odd[ 8] =  FF_16_odd[11] - D_16_odd[ 0];

C_16_odd[ 9] =  FF_16_odd[10] - D_16_odd[ 0];

B_16_odd[ 8] =  D_16_odd[ 4]+D_16_odd[ 5];

B_16_odd[ 9] =  D_16_odd[ 4]-D_16_odd[ 5];

B_16_odd[ 4] =  D_16_odd[ 1]+D_16_odd[ 2];

B_16_odd[ 5] =  D_16_odd[ 3]-D_16_odd[ 1];

B_16_odd[ 6] =  D_16_odd[ 3]+D_16_odd[ 1];

B_16_odd[ 7] =  D_16_odd[ 2]-D_16_odd[ 1];

B_16_odd[ 0] =  D_16_odd[14]-D_16_odd[15];

B_16_odd[ 1] =  D_16_odd[14]+D_16_odd[15];

B_16_odd[ 2] =  D_16_odd[12]+D_16_odd[13];

B_16_odd[ 3] =  D_16_odd[12]-D_16_odd[13];

A_16_odd[ 0] =  C_16_odd[ 0]+B_16_odd[ 8];

A_16_odd[14] =  C_16_odd[ 0]-B_16_odd[ 8];

A_16_odd[ 7] =  C_16_odd[ 1]+B_16_odd[ 9];

A_16_odd[ 9] =  C_16_odd[ 1]-B_16_odd[ 9];

A_16_odd[ 1] = C_16_odd[ 6]+B_16_odd[ 4];

A_16_odd[ 6] = C_16_odd[ 7]+B_16_odd[ 5];

A_16_odd[ 8] = C_16_odd[ 8]+B_16_odd[ 6];

A_16_odd[15] = C_16_odd[ 9]+B_16_odd[ 7];

A_16_odd[ 2] = C_16_odd[ 4]+B_16_odd[ 2];

A_16_odd[ 5] = C_16_odd[ 2]+B_16_odd[ 3];

A_16_odd[11] = C_16_odd[ 2]-B_16_odd[ 3];

A_16_odd[12] = C_16_odd[ 4]-B_16_odd[ 2];

A_16_odd[ 3] = C_16_odd[ 3]+B_16_odd[ 1];

A_16_odd[10] = C_16_odd[ 5]+B_16_odd[ 0];

A_16_odd[ 4] = C_16_odd[ 5]-B_16_odd[ 0];

A_16_odd[13] = C_16_odd[ 3]-B_16_odd[ 1];

O[15] =   47 *A_16_odd[ 0] - 43*A_16_odd[ 1];

O[ 0] =   43 *A_16_odd[ 0] + 47*A_16_odd[ 1];

O[14] =   51 *A_16_odd[ 3] - 38*A_16_odd[ 2];

O[ 1] =   38 *A_16_odd[ 3] + 51*A_16_odd[ 2];

O[13] =   55 *A_16_odd[ 4] - 33*A_16_odd[ 5];

O[ 2] =   33 *A_16_odd[ 4] + 55*A_16_odd[ 5];

O[12] =   58 *A_16_odd[ 7] - 27*A_16_odd[ 6];

O[ 3] =   27 *A_16_odd[ 7] + 58*A_16_odd[ 6];

O[11] =   60 *A_16_odd[ 8] - 22*A_16_odd[ 9];

O[ 4] =   22 *A_16_odd[ 8] + 60*A_16_odd[ 9];

O[10] =   62 *A_16_odd[11] - 16*A_16_odd[10];

O[ 5] =   16 *A_16_odd[11] + 62*A_16_odd[10];

O[ 9] =   63 *A_16_odd[12] -  9*A_16_odd[13];

O[ 6] =    9 *A_16_odd[12] + 63*A_16_odd[13];

O[ 8] =   64 *A_16_odd[15] -  3*A_16_odd[14];

O[ 7] =    3 *A_16_odd[15] + 64*A_16_odd[14];

C_8_odd[0] = (EOin[0] +EOin[7])  <<6;

C_8_odd[1] = (EOin[0] -EOin[7])  <<6;

CO_8_odd[0] = EOin[1]+ EOin[5];

CO_8_odd[1] = EOin[1]- EOin[5];

CO_8_odd[2] = EOin[2]+ EOin[6];

CO_8_odd[3] = EOin[2]- EOin[6];

C_8_odd[2]  = EOin[3]*90;

C_8_odd[3]  = EOin[4]*90;

B_8_odd[0] = C_8_odd[0]- C_8_odd[2];

B_8_odd[1] = C_8_odd[1]+ C_8_odd[3];

B_8_odd[2] = C_8_odd[0]+ C_8_odd[2];

B_8_odd[3] = C_8_odd[1]- C_8_odd[3];

BC_8_odd[ 1] =   83 *CO_8_odd[ 3] - 36*CO_8_odd[ 0];

BC_8_odd[ 0] =   36 *CO_8_odd[ 3] + 83*CO_8_odd[ 0];

BC_8_odd[ 2] =   83 *CO_8_odd[ 1] - 36*CO_8_odd[ 2];

BC_8_odd[ 3] =   36 *CO_8_odd[ 1] + 83*CO_8_odd[ 2];

A_8_odd[0] = (B_8_odd[2]+BC_8_odd[0]);

A_8_odd[7] = (B_8_odd[2]-BC_8_odd[0]);

A_8_odd[3] = (B_8_odd[0]+BC_8_odd[1]);

A_8_odd[4] = (B_8_odd[0]-BC_8_odd[1]);

A_8_odd[2] = (B_8_odd[3]+BC_8_odd[2]);

A_8_odd[5] = (B_8_odd[3]-BC_8_odd[2]);

A_8_odd[1] = (B_8_odd[1]+BC_8_odd[3]);

A_8_odd[6] = (B_8_odd[1]-BC_8_odd[3]);

EO[ 7] =   49 *A_8_odd[ 1] - 41*A_8_odd[ 0];

EO[ 0] =   41 *A_8_odd[ 1] + 49*A_8_odd[ 0];

EO[ 6] =   56 *A_8_odd[ 3] - 30*A_8_odd[ 2];

EO[ 1] =   30 *A_8_odd[ 3] + 56*A_8_odd[ 2];

EO[ 5] =   61 *A_8_odd[ 5] - 19*A_8_odd[ 4];

EO[ 2] =   19 *A_8_odd[ 5] + 61*A_8_odd[ 4];

EO[ 4] =   64 *A_8_odd[ 7] -  6*A_8_odd[ 6];

EO[ 3] =    6 *A_8_odd[ 7] + 64*A_8_odd[ 6];

B_4_odd[0] = (EEOin[0]+EEOin[3])<<6;

B_4_odd[1] = EEOin[1]*90;

B_4_odd[2] = EEOin[2]*90;

B_4_odd[3] = (EEOin[0]-EEOin[3])<<6;

A_4_odd[0] = B_4_odd[0]+B_4_odd[1];

A_4_odd[1] = B_4_odd[0]-B_4_odd[1];

A_4_odd[2] = B_4_odd[2]-B_4_odd[3];

A_4_odd[3] = B_4_odd[2]+B_4_odd[3];

EEO[ 3] =   53 *A_4_odd[ 3] - 36*A_4_odd[ 0];

EEO[ 0] =   36 *A_4_odd[ 3] + 53*A_4_odd[ 0];

EEO[ 1] =   12 *A_4_odd[ 1] - 63*A_4_odd[ 2];

EEO[ 2] =   63 *A_4_odd[ 1] + 12*A_4_odd[ 2];

A_2_even[0] = EEEEin[0]+EEEEin[1];

A_2_even[1] = EEEEin[0]-EEEEin[1];

EEEE[0] =  A_2_even[0]<<12 ;

EEEE[1] =  A_2_even[1]<<12 ;

A_2_odd[ 1] =   36 *EEEOin[ 0] - 83*EEEOin[ 1];

A_2_odd[ 0] =   83 *EEEOin[ 0] + 36*EEEOin[ 1];

EEEO[0] =  A_2_odd[0]<<6 ;

EEEO[1] =  A_2_odd[1]<<6 ;

EEE[0] = EEEE[0] + EEEO[0];

EEE[3] = EEEE[0] - EEEO[0];

EEE[1] = EEEE[1] + EEEO[1];

EEE[2] = EEEE[1] - EEEO[1];

EE[0] = EEE[0] + EEO[0];

EE[1] = EEE[1] + EEO[1];

EE[2] = EEE[2] + EEO[2];

EE[3] = EEE[3] + EEO[3];

EE[4] = EEE[3] - EEO[3];

EE[5] = EEE[2] - EEO[2];

EE[6] = EEE[1] - EEO[1];

EE[7] = EEE[0] - EEO[0];

E[ 0] = EE[0] + EO[0];

E[ 1] = EE[1] + EO[1];

E[ 2] = EE[2] + EO[2];

E[ 3] = EE[3] + EO[3];

E[ 4] = EE[4] + EO[4];

E[ 5] = EE[5] + EO[5];

E[ 6] = EE[6] + EO[6];

E[ 7] = EE[7] + EO[7];

E[ 8] = EE[7] - EO[7];

E[ 9] = EE[6] - EO[6];

E[10] = EE[5] - EO[5];

E[11] = EE[4] - EO[4];

E[12] = EE[3] - EO[3];

E[13] = EE[2] - EO[2];

E[14] = EE[1] - EO[1];

E[15] = EE[0] - EO[0];

des[ 0] = (E[ 0] + O[ 0] + add)>>shift;

des[ 1] = (E[ 1] + O[ 1] + add)>>shift;

des[ 2] = (E[ 2] + O[ 2] + add)>>shift;

des[ 3] = (E[ 3] + O[ 3] + add)>>shift;

des[ 4] = (E[ 4] + O[ 4] + add)>>shift;

des[ 5] = (E[ 5] + O[ 5] + add)>>shift;

des[ 6] = (E[ 6] + O[ 6] + add)>>shift;

des[ 7] = (E[ 7] + O[ 7] + add)>>shift;

des[ 8] = (E[ 8] + O[ 8] + add)>>shift;

des[ 9] = (E[ 9] + O[ 9] + add)>>shift;

des[10] = (E[10] + O[10] + add)>>shift;

des[11] = (E[11] + O[11] + add)>>shift;

des[12] = (E[12] + O[12] + add)>>shift;

des[13] = (E[13] + O[13] + add)>>shift;

des[14] = (E[14] + O[14] + add)>>shift;

des[15] = (E[15] + O[15] + add)>>shift;

des[16] = (E[15] - O[15] + add)>>shift;

des[17] = (E[14] - O[14] + add)>>shift;

des[18] = (E[13] - O[13] + add)>>shift;

des[19] = (E[12] - O[12] + add)>>shift;

des[20] = (E[11] - O[11] + add)>>shift;

des[21] = (E[10] - O[10] + add)>>shift;

des[22] = (E[ 9] - O[ 9] + add)>>shift;

des[23] = (E[ 8] - O[ 8] + add)>>shift;

des[24] = (E[ 7] - O[ 7] + add)>>shift;

des[25] = (E[ 6] - O[ 6] + add)>>shift;

des[26] = (E[ 5] - O[ 5] + add)>>shift;

des[27] = (E[ 4] - O[ 4] + add)>>shift;

des[28] = (E[ 3] - O[ 3] + add)>>shift;

des[29] = (E[ 2] - O[ 2] + add)>>shift;

des[30] = (E[ 1] - O[ 1] + add)>>shift;

des[31] = (E[ 0] - O[ 0] + add)>>shift;

}