| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G738 |
| --- | --- |

| Title: | On angular intra prediction main array extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents a simplification to subsampling process of the main array extension scheme by using reduced precision slope computation. Current design uses 12-bit precision slope tables to compute the subsampling positions given a block size and the prediction angle. Using 8-bit precision slope tables has neglible impact on coding efficiency. Simulation results show 0.00%, and 0.00% BD rate change for AI_HE and AI_LC cases, respectively.

# Introduction

In angular intra prediction, extended main array is derived for negative prediction angles by projecting samples from the side array onto it according to the prediction direction [1]. This projection consists of copying a subset of the samples in the side array into the main array. Subsampling positions are computed using slope tables [2].

# Algorithm Description

In the current HEVC WD 4 specification, the extension of the main array is derived by subsampling the side array by a subsampling factor that is proportional to the slope (invAngle). Current absolute prediction angles are defined by abs(intraPredAngle) table

= { 2,    5,   9,  13,  17,  21,  26,  32 }.

Inverse of the absolute prediction angle is the slope which is specified by the 12-bit abs(invAngle) table

256*32/= {4096, 1638, 910, 630, 482, 390, 315, 256}.

The subsampling of samples from the side array for a negative prediction angle intraPredAngle and its corresponding negative invAngle is specified for a prediction of size nS by

p[ -1, -1+( ( x*invAngle+128 )>>8 ) ],    with x=( nS*intraPredAngle ) >>5..-1

for vertical prediction, and

p[ -1+( ( x*invAngle+128 )>>8 ), -1 ], with x=( nS*intraPredAngle ) >>5..-1

for horizontal prediction.

Simpler 8-bit invAngle tables with values

16*32/= {256,   102,    56,    39,    30,    24,    19,    16}

with subsampling process defined by

p[ -1, -1+( ( x*invAngle+8 )>>4 ) ],    with x=( nS*intraPredAngle ) >>5..-1

for vertical prediction, and

p[ -1+( ( x*invAngle+8 )>>4 ), -1 ], with x=( nS*intraPredAngle ) >>5..-1

for horizontal prediction  can achieve same similar coding performance.

# Results

The proposed modification to main array extension process is implemented in HM4.0. The simulations are conducted based on the JCT-VC common test condition defined in Error: Reference source not found. Table 1 shows summary of simulation results. As can be from the results on average there is 0.0% BD rate loss for all configurations.

Table 1 Summary of simulation results (BD-rate), HM40 anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.03% | 0.02% | 0.00% | 0.04% | 0.01% |
| Class B | 0.01% | 0.02% | 0.02% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | -0.02% | 0.01% |
| Class D | 0.00% | 0.00% | -0.02% | 0.00% | 0.00% | -0.02% |
| Class E | 0.01% | -0.01% | 0.04% | 0.00% | 0.00% | 0.01% |
| Overall | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | -0.01% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 94% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.18% | 0.28% | 0.01% | 0.22% | 0.66% |
| Class B | 0.00% | -0.01% | -0.09% | 0.01% | -0.04% | 0.11% |
| Class C | 0.01% | -0.12% | 0.05% | -0.02% | -0.03% | 0.09% |
| Class D | 0.02% | -0.02% | 0.00% | -0.02% | -0.03% | 0.04% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | -0.08% | 0.05% | 0.00% | 0.03% | 0.22% |
|  | 0.01% | -0.09% | 0.05% | 0.00% | 0.03% | 0.19% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.03% | -0.03% | -0.38% | 0.01% | -0.03% | 0.04% |
| Class C | 0.02% | 0.08% | -0.09% | -0.01% | 0.26% | -0.18% |
| Class D | 0.05% | 0.45% | -0.25% | -0.02% | 0.27% | 0.05% |
| Class E | 0.03% | 0.09% | 0.04% | -0.11% | 0.12% | 0.30% |
| Overall | 0.03% | 0.14% | -0.20% | -0.03% | 0.14% | 0.04% |
|  | 0.03% | 0.18% | -0.23% | -0.03% | 0.14% | 0.05% |
| Enc Time[%] | 100.36% |  |  | 100.27% |  |  |
| Dec Time[%] | 95% |  |  | 97% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.03% | -0.10% | -0.02% | 0.02% | 0.09% | -0.10% |
| Class C | 0.00% | 0.24% | 0.07% | -0.01% | 0.16% | -0.02% |
| Class D | 0.01% | -0.16% | -0.14% | 0.03% | -0.13% | -0.16% |
| Class E | -0.02% | -0.13% | -0.58% | 0.01% | -0.51% | -0.30% |
| Overall | 0.01% | -0.03% | -0.13% | 0.01% | -0.06% | -0.13% |
|  | 0.01% | -0.03% | -0.14% | 0.01% | -0.08% | -0.16% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 94% |  |  | 93% |  |  |

# Conclusions

This contribution presented a simplification to subsampling process of the main array extension scheme by using reduced precision slope computation. Using 8-bit precision slope tables has neglible impact on coding efficiency. Simulation results show 0.00%, and 0.00% BD rate change for AI_HE and AI_LC cases, respectively.

# References

- T. K. Tan, et al., “Simplified angular intra prediction,” JCT-VC document JCTVC-B093, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd meeting, Geneva, Jul 2010.

- M. Budagavi, “Angular intra prediction and ADI simplification”, JCT-VC document JCTVC-B118, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd meeting, Geneva, Jul 2010.

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC document JCTVC-F803, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 4[4]

##### Specification of Intra_Angular prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is in the range of 3..33.

Table 8-2 specifies the mapping table between intraPredMode and the rearranged intra prediction order intraPredOrder.

Table 8-2 – Specification of intraPredOrder

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredOrder | - | - | - | - | 1 | 5 | 13 | 17 | 21 | 29 | 33 | 3 | 7 | 11 | 15 | 19 | 23 | 27 |
| intraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| intraPredOrder | 31 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 |  |

Figure 8-1 illustrates the total 34 intra angles and Table 8-3 specifies the mapping table between intraPredOrder and the angle parameter intraPredAngle.

Figure 8-1 – Intra prediction angle definition (informative)

Table 8-3 – Specification of intraPredAngle

| intraPredOrder | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | -32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 |
| intraPredOrder | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| intraPredAngle | 32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-4 further specifies the mapping table between intraPredOrder and the inverse angle parameter invAngle.

Table 8-4 – Specification of invAngle

| intraPredOrder | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | -16 | -19 | -24 | -30 | -39 | -56 | -102 | -256 |
| intraPredOrder | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| invAngle | -19 | -24 | -30 | -39 | -56 | -102 | -256 | - |

The reference pixel array refMain[ x ], with x=-nS..2*nS is specified as follows.

–	If intraPredOrder is less than 18,

refMain[ x ] = p[ -1+x, -1 ], with x=0..nS		(8-1)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1, -1+( ( x*invAngle+8 )>>4 ) ], with x=( nS*intraPredAngle ) >>5..-1	(8-2)

- Otherwise,

refMain[ x ] = p[ -1+x, -1 ], with x=nS+1..2*nS		(8-3)

Otherwise,

refMain[ x ] = p[ -1, -1+x ], with x=0..nS		(8-4)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1+( ( x*invAngle+8 )>>4 ), -1 ], with x=( nS*intraPredAngle ) >>5..-1	(8-5)