| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G745 |
| --- | --- |

| Title: | CE9: Cross-check report on MRG_ID02 A and B (JCTVC-G421) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Cross-check on CE9:MRG_ID02 A and B of JCTVC-G421

In this document, report on MRG_ID02 A and B test in CE9 is provided. In the experiments, non-zero MV from spatial neighbor and different MV from spatial neighbor are utilized respectively when the collocated motion information of a bi-predicted Merge/SKIP PU is identical. Brief description of each test is given in Table 1.

Table 1. Brief description of tests

| MRG_ID02 | Non-zero MV from spatial neighbor | JCTVC-F712 | A: Keep the original refIdxL1Col |
| --- | --- | --- | --- |
|  |  |  | B: Derive refIdxL1Col from the one related to the found MV |

Software was provided by the proponent of JCTVC-G421. It is confirmed that the source code has modified according to the description and the results under common test condition [1] are same with the results of JCTVC-G421.

Table 2. Experimental results of MRG_ID01 A

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% | -0.1% |
| Class D | 0.1% | 0.7% | 0.3% | 0.2% | -0.1% | -0.1% |
| Class E | 0.3% | 0.5% | 0.6% | 0.2% | 0.4% | 0.5% |
| Overall | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 3. Experimental results of MRG_ID01 B

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.1% | -0.3% | 0.1% | 0.0% | -0.3% |
| Class E | 0.3% | 0.4% | 1.3% | 0.2% | 0.6% | 0.4% |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, Jul. 2011.