| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G750 |
| --- | --- |

| Title: | Non-CE2: Harmonization of HE residual coding and NSQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
|  | Jungsun Kim Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea |  | +82-10-2201-9774 jungsun.kim@lge.com bm.jeon@lge.com |
| Source: | Qualcomm, LG electronics |  |  |

_____________________________

# Abstract

Harmonization of CABAC with residual data coding of non-square block transforms (NSQT) is proposed. The method avoids the current mapping between square and non-square blocks. Scans and context derivation for non-square blocks are provided. The method reports a BD-rate of -0.02%, -0.06%, -0.10% for luma and chroma components under Random access High efficiency condition and -0.06%, -0.29%, 0.00% for the Low delay High efficiency case.

- Introduction

NSQT makes use of non-square transforms [1-2]. The following sizes are currently applied: 32×8, 8×32, 16×4, and 4×16. Entropy coding is applied to the resulting coefficients. However, the HE entropy coding has not been modified to deal with non-square transform coefficients. Instead, a mapping is applied in order to re-use the existing method for square blocks. The mapping is defined from a non-square block to a square block with the same number of pixels. Then, the regular entropy coding and decoding are applied to the square block. Finally, prior to the inverse transform, the inverse mapping is applied again for the non-square blocks from the square shape to the original shape.

This method re-uses directly the existing entropy coding, but fails to exploit the underlying correlation of the coefficients in a non-square block because the mapping ‘breaks’ some of the neighboring relationship and correlation among coefficients. Also it introduces intermediate steps that add complexity to the coder.

In this proposal, we propose several modifications in the current ‘square-based’ entropy coding to deal with the non-square coefficient block. Modifications are in the following parts: scans, coding of the last significant coefficient, and significance map.

- Technical description

This contribution proposes the harmonization of the CABAC entropy coding with the non-square block transforms and it is essentially the same method as described in CE6.c [3] for SDIP. We propose to scan coefficients directly at the non-square shape. Three parts of the entropy coding are modified in a straightforward way to deal with non-square transforms: scans, last significance coefficient coding, and significance map coding.

## Scans

Similar to the square blocks in HM4.0, we propose to scan the non-square blocks directly and with a diagonal scan starting from the left side. The figure below shows an example of such scan for a block of size 16×4 and 4×16 blocks:

Fig. 1 the proposed scanning orders for 16×4/4×16 transform block

## Last Significant Coefficient Position

For non-square blocks, the context model for the signaling of the last significant coefficient depends on the length of the side of the coordinate being coded. In other words, the horizontal coordinate (X component of the last coefficient) has a context depending on the width of the block (and the bin of X being coded). Equivalently, the vertical coordinate (Y component) has a context set assigned depending on the height of the block. No additional contexts are needed.

## Significance Map

For the significance map of non-square blocks, blocks with the same number of coefficients share the same contexts. Therefore, the total number of contexts remains the same. For example, the context table for 16×4/4×16 block is same as that for 8×8 block. But mapping of context indexes is modified for its shape as below.

Blocks with 64 coefficients use contexts for the significance map depending on the position (like in HEVC the 4×4 and 8×8 blocks). The other blocks (32×8/8×32) use the neighbor based derivation equivalent to the larger blocks in HEVC. The derivation of the context based on position has to be different from HM4.0 because of the non-square nature of the blocks. The context derivation for a non-square block and its ‘transposed’ is the same. Details are found in section 6. As an example, for the 16×4 and 4×16 cases, the context depending of the position is shown in the following figure:

Fig. 2 Context index for the significance of 16×4/4×16 blocks

- Results

This method is implemented in HM4.0. Simulation results are presented for the high efficiency common test conditions [4]. Table 1 shows the results for this method. The BD-rate of luma component is -0.02% and -0.06% for RA and LB configurations, respectively. The BD-rate of chroma component is -0.06%, -0.10% and -0.29%, 0.00% for RA and LB configuration respectively.

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.03% | -0.12% | -0.31% |  |  |  |
| Class B | -0.05% | -0.06% | 0.00% | -0.05% | -0.19% | -0.20% |
| Class C | -0.04% | -0.04% | 0.05% | -0.07% | -0.19% | -0.15% |
| Class D | 0.00% | -0.02% | -0.19% | -0.08% | -0.23% | -0.17% |
| Class E |  |  |  | -0.01% | -0.66% | 0.77% |
| Overall | -0.02% | -0.06% | -0.10% | -0.06% | -0.29% | 0.00% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 1. Simulation Results

- Conclusions

It is proposed a straightforward extension of CABAC to deal with the entropy coding of non-square transform coefficients. This method removes the complexity associated with the current remap between square and non-square blocks. The proposed scanning orders are consistent to those of square blocks and keep the benefit of diagonal scanning. No new contexts are added. The performance increases by -0.02%, -0.06%, -0.10% and -0.06%, -0.29%, 0.00% for luma component and chroma components for RA and LB configurations, respectively.

- References

- Y. Yuan, X. Zheng, X. Peng, J. Xu, L. Liu, Y. Wang, X. Cao, C. Lai, J. Zheng, Y. He, H. Yu , “CE2: Non-Square Quadtree Transform for symmetric motion partition”, JCTVC-F410, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, July, 2011.

- X. Zheng, X. Peng, J. Xu, IK Kim, L. Liu, Y. Wang, X. Cao, C. Lai, J. Zheng, Y. He, H. Yu , “CE2: Non-Square Quadtree Transform for symmetric and asymmetric motion partition”, JCTVC-F412, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, July, 2011.

- J. Sole, R. Joshi, X. Wang, M. Karczewicz, “CE6.c: Harmonization of HE residual coding with non-square block transforms”, JCTVC-G320, 7th JCT-VC Meeting, Geneva, CH, November 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, July, 2011.

- Text for Working Draft

### Inverse scanning process for transform coefficients

Inputs to this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit.

–	a list of (nW)x(nH) values transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ].

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is a variable c containing a two-dimensional array of (nW)x(nH) values.

The variable nS is set equal to Sqrt(nW*nH).

The variable scanIdx, indicating wich scan is used to scan the transform coefficients (zigzag, horizontal or vertical), is derived as follows.

- If PredMode is equal to MODE_INTRA, the following applies.

  - If cIdx is equal to 0, scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredMode ].

  - Otherwise (cIdx is equal to 1 or 2), scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredModeC ]

- Otherwise (PredMode is not equal to MODE_INTRA), scanIdx is set to 0.

When entropy_coding_mode_flag is equal to 1 (CABAC) and scanIdx is equal to 0 (zigzag), scanIdx is set to 3 (diagonal).

The variable c[ i ][ j ] which is located in the ( i, j ) position in the array c is derived as follows.

- The transform coefficient level at scanning position n is mapped to the position ( i, j ) in the array c using the scan specified by scanIdx as follows.

i = ScanOrder[ scanIdx ] [ Log2( nW)-2 ][ Log2(nH )-2][ n ][ 0 ]			
j = ScanOrder[ scanIdx ] [Log2( nW)-2 ][ Log2(nH )-2 ][ n ][ 1 ]			
c[ i ][ j ] = transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ][ n ],			
with n = 0.. ( nS * nS ) – 1	(8-1)

(remove second equation)

[Ed. (BB): Despite different scans NSQT reordering always uses zigzag scan. Needs to be reviewed and may be harmonized with the different scans.]

###### Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x and last_significant_coeff_y

Inputs to this process are the binIdx, the color component index cIdx and the transform block size log2TrafoSize of the axis processed, for which Size being either Width or Height.

Output of this process is ctxIdxInc.

Table 9-53 Specification of lastCtx[i]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx | 0 | 1 | 2 | 3 | 3 | 4 | 4 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 7 |
| i | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |  |
| lastCtx | 7 | 7 | 7 | 8 | 8 | 8 | 8 | 9 | 9 | 9 | 9 | 10 | 10 | 10 | 10 |  |

The vector lastCtx[ i ] is specified in Error: Reference source not found. For the derivation of ctxIdxInc, the following applies.

–	If log2TrafoSize is less than or equal to 2, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ binIdx ]		(9-2)

–	Otherwise (log2TrafoSize is greater than 2), ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( 1 << ( log2TrafoSize – 1 ) ) + binIdx ]	(9-3)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  19		(9-4)

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

###### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size (blkWidth, blkHeight).

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3 and blkWidth is not equal to blkHeight, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-5)

- Otherwise if blkWidth*blkHeight is equal to 64

  - If blkWidth < blkHeight

- Temp = xC

- xC = yC

- yC = xC

xC = xC – ( ( xC >> 3 ) << 3 )

yC = yC + ( ( xC >> 3 ) << 2 )

sigCtx = 15 + ( ( yC >> 1 ) << 2 ) + ( xC >> 1 )

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC	(9-66)

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-77)
sigCtx  =  34  +  Min( 4, temp )

- Otherwise (xC  +  yC is greater than 4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39		(9-88)

- When xC is less than ( blkWidth – 1 ), the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-59)

- When xC is less than ( blkWidth – 1 ) and yC is less than ( blkHeight – 1 ), the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-90)

- When xC is less than ( blkWidth – 2 ), the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-101)

- When yC is less than ( blkHeight – 1 ), the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-112)

- When yC is less than ( blkHeight – 2 ) and sigCtx is less than 43, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-123)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-134)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  44  +  sigCtx		(9-145)

[Ed. (BB): The context derivation assumes maximum transform sizes less than or equal to 32x32 for luma and 16x16 for chroma and minimum transform sizes greater than or equal to 4x4.]

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).