| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G753 |
| --- | --- |

| Title: | CE1: Crosscheck of CE1 subtest C.6 (JCTVC-G633) proposed by HHI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng | Tel: Email: | +86-755-28422009 zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of proposal G633. The source code was implemented based on HM-4.0. We compiled, inspected, and ran the code of these cases with Intra HE RA_HE, LB_HE and LP_HE configurations. The cross-check has been completed successfully and the results match those provided by proponents of JCTVC-G633.

# Test description and crosscheck results

This cross-check document report the cross-check results of CE1 subtest C.6 The test case was crosschecked by setting the Macros as default provided by proponents, the encoder and decoder reconstructed YUV files match. The experimental results of the rate-distortion are illustrated in Error: Reference source not found. These results match the BD-rate results provided by proponents of JCTVC- G633. The decoding time and encoding time change of the crosscheck report shouldn't be considered.

Table 1 Results of CE1 subtest C.6

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.3% | -0.5% |
| Class D | 0.0% | -1.2% | -1.4% |
| Class E | -0.1% | -0.2% | 0.0% |
| Class F | 0.0% | -0.2% | -0.2% |
| Overall (without classF) | 0.0% | -0.3% | -0.4% |
|  | 0.0% | -0.3% | -0.4% |
| Enc Time[%] (without classF) | 98% |  |  |
| Dec Time[%] (without classF) | 100% |  |  |
| Overall (with classF) | 0.0% | -0.3% | -0.4% |
|  | 0.0% | -0.3% | -0.4% |
| Enc Time[%] (with classF) | 98% |  |  |
| Dec Time[%] (with classF) | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.1% | 0.1% | -0.2% |
| Class B | -0.2% | 0.2% | 0.0% |
| Class C | -0.3% | -0.2% | -0.2% |
| Class D | -0.4% | -0.6% | -1.0% |
| Class E |  |  |  |
| Class F | -0.2% | -0.1% | -0.3% |
| Overall (without classF) | -0.3% | -0.1% | -0.3% |
|  | -0.3% | -0.1% | -0.3% |
| Enc Time[%] (without classF) | 99% |  |  |
| Dec Time[%] (without classF) | 100% |  |  |
| Overall (with classF) | -0.3% | -0.1% | -0.3% |
|  | -0.2% | -0.1% | -0.3% |
| Enc Time[%] (with classF) | 99% |  |  |
| Dec Time[%] (with classF) | 101% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | 0.1% | -0.2% |
| Class C | -0.3% | 0.2% | -0.2% |
| Class D | -0.6% | 0.3% | 0.3% |
| Class E | -0.9% | -0.3% | -0.1% |
| Class F | -0.6% | -0.7% | -0.4% |
| Overall (without classF) | -0.5% | 0.1% | 0.0% |
|  | -0.5% | 0.1% | -0.1% |
| Enc Time[%] (without classF) | 100% |  |  |
| Dec Time[%] (without classF) | 100% |  |  |
| Overall (with classF) | -0.5% | 0.0% | -0.1% |
|  | -0.5% | 0.0% | -0.1% |
| Enc Time[%] (with classF) | 100% |  |  |
| Dec Time[%] (with classF) | 101% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | 0.3% | 0.2% |
| Class C | -0.2% | 0.5% | 0.3% |
| Class D | -0.4% | 0.8% | 1.0% |
| Class E | -0.6% | -0.2% | -0.4% |
| Class F | -0.4% | -0.1% | 0.0% |
| Overall (without classF) | -0.3% | 0.4% | 0.3% |
|  | -0.3% | 0.4% | 0.2% |
| Enc Time[%] (without classF) | 99% |  |  |
| Dec Time[%] (without classF) | 98% |  |  |
| Overall (with classF) | -0.3% | 0.3% | 0.2% |
|  | -0.3% | 0.3% | 0.2% |
| Enc Time[%] (with classF) | 99% |  |  |
| Dec Time[%] (with classF) | 99% |  |  |

After the inspection of software provided by the proponents, it is confirmed that the modification of source code is match with G633's description.