| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G754 |
| --- | --- |

| Title: | Non-CE6: Line buffer reduction for CABAC context of SDIP syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo, Xianglin Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8125 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

SDIP (short-distance-intra-prediction) introduces two extra syntax elements sdip_flag and sdip_direction. The CABAC context modeling of these two syntax elements involves the corresponding syntax values from top blocks, and thus introduces line buffer storage. This contribution describes modified CABAC contexts for sdip_flag and sdip_direction that avoid upper block data and thus eliminates the line buffer for these two syntax elements. The average B-D rate change is 0.0% and no encoding/decoding time change is observed.

# Introduction

In SDIP, two extra syntax elements sdip_flag and sdip_direction are introduced. If the CU is encoded as a regular-shape CU, sdip_flag = 0 is sent; otherwise sdip_flag = 1 followed by sdip_direction is sent to further specify the type of SDIP CU (hNx2N or 2NxhN). The CABAC contexts of sdip_flag and sdip_direction are derived based on the corresponding syntax values from the left block and the top block. Since LCUs are processed in a raster scan order, the coded data of bottom blocks of an LCU row are required to be stored in line buffers for the coding of a next LCU row. This contribution describes CU-size based contexts for sdip_flag and sdip_direction that avoids line buffer storage.

# CU-size based context for SDIP syntax

SDIP coding are allowed for 3 CU sizes: 32x32 CU, 16x16CU and 8x8 CU. It is proposed to select the CABAC context for sdip_flag and sdip_direction based on the CU size:

| CtxIdx | CU Size |
| --- | --- |
| 0 | 8x8 |
| 1 | 16x16 |
| 2 | 32x32 |

The above table can be described using a formula

CtxIdx = log2CUSize -3

The original context numbers for sdip_flag and sdip_direction are both 3, so there is no increase in the number of contexts. The saving on line buffer storage is in the following table

Table I. Contexts related memory usage comparison (assuming 1920x1080 frame)

| Syntax | Original memory needed (bit) | Proposed (bit) |
| --- | --- | --- |
| sdip_flag | (1920+64)/8 – 1 = 247 | 0 |
| sdip_direction | (1920+64)/8 – 1 = 247 | 0 |
| Total | 494 | 0 |

# Experimental results

We implemented new CABAC contexts for SDIP syntax on top of HM 4.0 based SDIP software ( HM4.0-SDIP released by Huawei to CE6). The coding efficiency is evaluated under common conditions specified in [1]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time is measured on clusters with same CPU, and the decoding time is measured by decoding all sequences on a single PC. There are 3 different configurations of SDIP (SDIP1, SDIP2 and SDIP3) in HM4.0-SDIP.

I  Performance with SDIP 1 configuration (Anchor: HM4.0-SDIP1)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% |
| Class F | #NAME? | #NAME? | #NAME? |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 96% |  |  |

II  Performance with SDIP 2 configuration (Anchor: HM4.0-SDIP2)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | 0.0% |
| Class F | #NAME? | #NAME? | #NAME? |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

V  Performance with SDIP 3 configuration (Anchor: HM4.0-SDIP3)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% |
| Class E | -0.1% | -0.2% | -0.1% |
| Class F | #NAME? | #NAME? | #NAME? |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 102% |  |  |

# Conclusions

This contribution modifies the CABAC contexts derivation for the two SDIP syntax elements sdip_flag and sdip_direction. The modified contexts eliminates line buffer storage for these contexts without introducing any loss.

# Supplementary results  (with Class F)

Performance with SDIP 1 configuration (Anchor: HM4.0-SDIP1)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% |
| Class F | -0.1% | -0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |

I Performance with SDIP 2 configuration (Anchor: HM4.0-SDIP2)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | 0.0% |
| Class F | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

II Performance with SDIP 3 configuration (Anchor: HM4.0-SDIP3)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% |
| Class E | -0.1% | -0.2% | -0.1% |
| Class F | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

# Draft Texts

| Table 9-1 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| alf_cu_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| split_coding_unit_flag | Error: Reference source not found | 0 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
| skip_flag | Error: Reference source not found | 0 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na |
| cu_qp_delta | Error: Reference source not found | 0 | 0 | na (uses Decode Bypass) | 1 | 2 | 2 |
|  |  | 3 | 0 | na (uses Decode Bypass) | 1 | 2 | 2 |
|  |  | 6 | 0 | na (uses Decode Bypass) | 1 | 2 | 2 |
| pred_type | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | 1 | 2 | 3 | na |
|  |  | 5 | 0 | 1 | 2 | 3 | 4 |
| prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | 1 | 0 | 0 | 0 | 0 | 0 |
|  |  | 2 | 0 | 0 | 0 | 0 | 0 |
| intra_chroma_pred_mode | Error: Reference source not found | 0 | 0 | 1 | 1 | 1 | na |
|  |  | 2 | 0 | 1 | 1 | 1 | na |
|  |  | 4 | 0 | 1 | 1 | 1 | na |
| merge_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| merge_idx | Error: Reference source not found | 0 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
|  |  | 4 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
| inter_pred_flag | Error: Reference source not found | 0 | cuDepth | na | na | na | na |
| ref_idx_l0 | Error: Reference source not found | 0 | 0 | 1 | 2 | 2 | 2 |
| ref_idx_l0, ref_idx_l1, ref_idx_lc | Error: Reference source not found | 3 | 0 | 1 | 2 | 2 | 2 |
| abs_mvd_greater0_flag[ ] | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| abs_mvd_greater1_flag[ ] | Error: Reference source not found | 2 | 0 | na | na | na | na |
|  |  | 3 | 0 | na | na | na | na |
| mvp_idx_l0 | Error: Reference source not found | 0 | 0 | 1 | 1 | 1 | 1 |
| mvp_idx_l0, mvp_idx_l1, mvp_idx_lc | Error: Reference source not found | 2 | 0 | 1 | 1 | 1 | 1 |
| no_residual_data_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| split_transform_flag | Error: Reference source not found | 0 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 4 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 8 | cuDepth + trafoDepth | na | na | na | na |
| cbf_luma | Error: Reference source not found | 0 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na | na |
|  |  | 2 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na | na |
|  |  | 4 | ( trafoDepth = = 0 ) ¦ ¦ ( log2TrafoSize = = Log2MaxTrafoSize ) ? 1 : 0 | na | na | na | na |
| cbf_cb | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |
| cbf_cr | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |
| last_significant_coeff_x, last_significant_coeff_y | Error: Reference source not found | 0 | 0..30 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 31 | 0..30 (subclause Error: Reference source not found) |  |  |  |  |
|  |  | 62 | 0..30 (subclause Error: Reference source not found) |  |  |  |  |
| significant_coeff_flag | Error: Reference source not found | 0 | 0..87 (subclause Error: Reference source not found) | na | na | na | na |
|  | Error: Reference source not found | 0 | 0..87 (subclause Error: Reference source not found) | na | na | na | na |
|  | Error: Reference source not found | 0 | 0..87 (subclause Error: Reference source not found) | na | na | na | na |
| coeff_abs_level_greater1_flag | Error: Reference source not found | 0 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 60 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 120 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
| coeff_abs_level_greater2_flag | Error: Reference source not found | 0 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 60 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
|  |  | 120 | 0..59 (subclause Error: Reference source not found) | na | na | na | na |
| sdip_flag | Error: Reference source not found | 0 | log2CUSize-3 | na | na | na | na |
|  |  | 3 | log2CUSize-3 | na | na | na | na |
|  |  | 6 | log2CUSize-3 | na | na | na | na |
| Sdip_direction | Table 9-47 | 0 | log2CUSize-3 | na | na | na | na |
|  |  | 3 | log2CUSize-3 | na | na | na | na |
|  |  | 6 | log2CUSize-3 | na | na | na | na |

# References

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC Meeting, Torino, Italy, Jul. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).