| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G757r1 |
| --- | --- |

| Title: | On software complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information / Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen 3240 Hillview Ave Palo Alto CA 94304 USA | Tel: Email: | +1 650 496 4742 bossen@docomoinnovations.com |
| Source: | DOCOMO Innovations, Inc. |  |  |

_____________________________

# Abstract

Complexity is a complex topic. While the commonly used run time measurements obtained with the HM software may be indicative of complexity, they often remain a poor proxy of actual complexity. A software decoder was thus written from scratch with the goal of achieving real-time decoding on x86 and ARM devices, and thereby provide better estimations of run times in real-world environments. This contribution provides some insight on which tools significantly contribute to decoding time and also suggests some modifications to the draft HEVC specification.

# Introduction

A software decoder was written from scratch with the goal of achieving real-time decoding on x86 and ARM devices, and thereby provide better estimations of run times in real-world environments. The decoder takes advantage of SIMD technology to improve decoding times (MMX/SSE on x86, and NEON on ARM). In its current state the following decoding rates are achieved:

- 1920x1080@50fps on a single core of a Core i5 processor (Arrandale) clocked at 2.53GHz

- 832x480@30fps on a single core of a Cortex A9 processor clocked at 1GHz

It is expected that higher decoding rates can be achieved, and no claim is made on the optimality of the decoder in terms of speed.

## Limitations

The software decoder was written to decode streams generated with HM 4.0. However it does not fully support the HM syntax and has the following limitations:

- Most high-level syntax elements are assumed to take values defined in the common test conditions

- The “chroma-from-luma” intra mode is not supported. This mode can be disabled through the configuration file (LMChroma parameter). It should be noted that the HM 4.0 implementation of this mode has a bug (#216).

- NSQT is not supported. This tool can be disabled using a macro (NSQT). It should be noted that this tool lacks some maturity, in particular with regards to ordering of the horizontal and vertical transforms and data reordering.

- AMP is not supported. This tool can be disabled using a macro (AMP). It should be noted that the implementation of this tool may require significant changes due to the fact that partition sizes may not be expressed as powers of 2.

- ALF is not supported. This tool can be disabled through the configuration file.

- CALVC is not supported. CABAC is used instead for improved coding efficiency.

- Bit depth is limited to 8 bits per sample. This can be configured through the configuration file. It should be noted that using more than 8 bits per sample has significant complexity implications (mostly in terms of memory and memory bandwidth).

Most of these restrictions are in place due to the limited time that was available to rewrite a decoder from scratch.

# Profiling

Profiling of the decoder was done using time sampling where an interrupt regularly polls the program counter to determine the percentage of time spent in each function. Each profiling run was 60 seconds.

Instead of reporting time percentages for individual function, functions are grouped into several modules:

- Motion compensation, which covers the interpolation process but not the motion vector derivation process

- Deblocking filter, which includes derivation of filtering parameters (such as boundary strength) and actual filtering

- Parsing. Note that this also includes generating motion vector prediction and any other process required to reconstruct data, such as intra prediction modes

- Sample-adaptive offset filter (SAO)

- Inverse quantization and transform

- Intra prediction / adding residual

RA configurations were used in these experiments. Results are presented below for x86 and ARM architectures.

## x86

RaceHorses, QP=32, ~1Mbps

| Module | Time |
| --- | --- |
| Motion compensation | 43% |
| Deblocking filter | 19% |
| Parsing | 20% |
| SAO | 6% |
| Inverse quantization and transform | 3% |
| Intra prediction / adding residual | 5% |

RaceHorses, QP=27, ~2Mbps

| Module | Time |
| --- | --- |
| Motion compensation | 38% |
| Deblocking filter | 16% |
| Parsing | 31% |
| SAO | 5% |
| Inverse quantization and transform | 4% |
| Intra prediction / adding residual | 5% |

BasketballDrive, QP=32, ~3Mbps

| Module | Time |
| --- | --- |
| Motion compensation | 55% |
| Deblocking filter | 15% |
| Parsing | 13% |
| SAO | 9% |
| Inverse quantization and transform | 3% |
| Intra prediction / adding residual | 3% |

BasketballDrive, QP=27, ~6Mbps

| Module | Time |
| --- | --- |
| Motion compensation | 48% |
| Deblocking filter | 15% |
| Parsing | 20% |
| SAO | 8% |
| Inverse quantization and transform | 4% |
| Intra prediction / adding residual | 4% |

## ARM

RaceHorses, QP=32, ~1Mbps

| Module | Time |
| --- | --- |
| Motion compensation | 47% |
| Deblocking filter | 14% |
| Parsing | 17% |
| SAO | 10% |
| Inverse quantization and transform | 4% |
| Intra prediction / adding residual | 5% |

RaceHorses, QP=27, ~2Mbps

| Module | Time |
| --- | --- |
| Motion compensation | 41% |
| Deblocking filter | 13% |
| Parsing | 25% |
| SAO | 8% |
| Inverse quantization and transform | 5% |
| Intra prediction / adding residual | 6% |

## Notes

- Although no hard data is available on this aspect, cache misses seem to be an important contributing factor in determining decoding speed. While previous standards such as AVC use a 16x16 macroblock structure, HEVC uses a 64x64 LCU structure which has a significant impact on cache requirements.

- The motion compensation module is the most time-consuming module (38-55%)

- The inverse quantization and transform module uses only a small fraction of decoding time (3-5%). On an Intel Core i5 M540 processor (Nehalem microarchitecture), the number of cycles per transform has been measured to be:

  - 158 cycles for an inverse 8x8

  - 861 cycles for an inverse 16x16

  - 4696 cycles for an inverse 32x32

These numbers assume that the processor was operating at its maximum frequency of 3.06 GHz (“Turbo boost” instead of the nominal 2.53 GHz frequency) when time measurements were made. It should also be noted that the implementation does trade off some speed for a reduced code size. The machine code for the inverse 32x32 holds in 2300 bytes. When allowing for larger code size (4000 bytes), cycle counts as low as 4490 have been measured for an inverse 32x32.

- In the deblocking module about half the time is spent deriving “decisions” (e.g., which edges to filter, boundary strength, strong/weak decision).

# Issues

Several issues became apparent when rewriting the decoder. A non-exhaustive list is provided below (in no particular order).

## NxN inter partitions

Support for the NxN inter partition mode seems to be different for P and B slices. For instance this mode is supported for CU sizes larger than 8x8 in P slices, but not in B slices where it is replaced by the intra mode. This should be made consistent.

## Subsampling of reference data

In JCTVC-F060 it was suggested to subsample motion vector data to reduce line buffer requirements. If adopted, the same technique should be applied to all reference data, including intra prediction modes.

## Merge/skip mode

It would be desirable to reduce the number of candidates used for the merge/skip mode. While the final list of candidates is limited to 5 entries, up to 17 candidates may be tested and compared against each other for equality.

For partition sizes other than 2Nx2N, a set of one or two candidates is derived to avoid emulation of other partition sizes (see TComDataCU::avoidMergeCandidate() function). This set is however inconsistently used (it is used only when testing motion data from spatially neighboring blocks). It should be determined whether the current design makes sense, or if this set should either be used all the time or not at all.

## Deblocking filter

The current deblocking filter implements 2 levels of parallelism: “parallel decisions” and “parallel filter”. While the “parallel filter” feature seems to be a worthwhile addition, “parallel decisions” seems to add a burden without providing a real benefit.

The current design could be roughly summarized as follows:

- decisions H+V

- filter H

- filter V

Removing “parallel decisions” would lead to:

- decisions H, filter H

- decisions V, filter V

This would be preferable in software considering the following. To keep the code size to a minimum (such as for example to avoid program cache misses), it is preferable to implement a single function for processing horizontal and vertical edges. This implies that a transpose operation is required for one of the directions. In the current design such a transpose operation would have to be applied twice, once for the decisions step, and once for the filter step. Furthermore additional buffers are required to store the results of the decisions step and sample data has to be accessed multiple times.

It is therefore suggested to remove the “parallel decisions” feature from the design.

## Motion vector clipping

Motion vector clipping is applied at several stages. Clipping in the motion predictor derivation should be removed, except for the case where scaling is used. In such a case clipping should be done to maintain a fixed range, such as -4096 to 4095 (this may be adapted to the maximum range supported for a given profile@level). Clipping in the motion compensation process should be made non-normative, similarly to what is done in AVC (clipping may be used non-normatively, for example to limit the amount of picture padding which is assumed to be infinite).

## Filter sizes for motion compensation

Motion compensation currently uses 8-tap filters for horizontal and vertical fractional-sample interpolation. It would be desirable to reduce the number of taps as this filtering process represents a major complexity bottleneck. For example a 6-tap filter may be used for vertical filtering. It should be noted that it is desirable to keep an even number of taps as some implementations rely on pair-wise computations, and to keep the number of taps constant across fractional sample position to avoid duplication of code.

## Intra mode coding

Currently most PU sizes support 35 intra prediction modes for luma (33 angular, DC and planar). For 64x64 PUs only a small set of modes is supported. There doesn’t seem to be a particularly good reason for this, as it would seem natural to support the same 35 modes in that case too. To minimize impact on encoding time, searching at the 64x64 level may be skipped altogether (or the search may be limited to the 2 most probable modes as defined by the mode prediction process). Harmonizing the number of modes would enable the removal of a mode mapping table (35 entries) and simplify a bit the mode derivation process.

## SAO

SAO comprises 6 modes: 4 “edge-offset” modes and 2 “band-offset” modes. Profiling data indicates that the “band-offset” modes are used infrequently and their effectiveness should be tested separately.

Furthermore the current signaling of SAO modes relies on frame-based optimization in the encoder. This makes it unsuitable for many applications. It is thus suggested to disable the SAO feature in the LC test configurations.

## Intra DC prediction

A boundary smoothing post-process is currently applied to the intra DC prediction. However this process is applied only to the luma component. For harmonization purposes, it would be better to either apply this process to all picture components (i.e., luma and chroma) or to none.

## CABAC binarization

The binarizations of the intra mode residual and the suffix of the last significant coefficient position are such that the least significant bit is coded first. It would be preferable to code the most significant bit first to be consistent with other parts of the design.

Unary codes coded using the bypass process should also be made consistent. Sometimes they are in 0..01 form, and sometimes in 1..10 form. The same applies to unary codes coded using the regular decoding process (see for example issue #219).

# Conclusion

Time profiling results of a real-time HEVC decoder running on x86 and ARM platforms were presented. They show that the motion interpolation process is currently the most time-consuming part of the HEVC design.

Several issues became apparent when implementing a decoder from scratch. These issues should either be addressed directly or further studied in a CE, as appropriate.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The actual version used includes an encoder fix related to the “chroma-from-luma” mode, as submitted in revision 1360 on the SVN server.