| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G758 WG11 Number: m22328 |
| --- | --- |

| Title: | Non-CE11: Cross-check report for TI's proposal (JCTVC-G123) on Simplified Coefficient Scans for NSQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hisao Sasai Kengo Terada Takahiro Nishi 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result for TI's proposal in JCTVC-G123. The experimental results perfectly match with the revised one provided by TI in the rate-distortion performance.

- Experimental Condition

The software to be tested was provided by TI and its performance relative to HMv4 software was checked under the common test condition described in JCTVC-F900 [1].
The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

- Results

In JCTVC-G123 [2], two methods are proposed. The first one is to change mapping scan order for NSQT (Table 1). The second one is to change context derivation method for NSQT (Table 2).

BD BR and execution time is shown in Table 1 and 2.

Table 1. BD BR for TI's proposal on Coding efficiency of proposed NSQT scan (high efficiency)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | -0.23% | -0.27% |  |  |  |
| Class B | 0.00% | -0.15% | -0.02% | 0.01% | -0.12% | -0.33% |
| Class C | 0.00% | -0.09% | -0.11% | 0.03% | -0.16% | -0.05% |
| Class D | 0.03% | 0.01% | 0.01% | 0.00% | -0.02% | 0.17% |
| Class E |  |  |  | 0.01% | 0.18% | 0.42% |
| Overall | 0.00% | -0.12% | -0.09% | 0.01% | -0.05% | 0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2. BD BR for TI's proposal on Coding efficiency of proposed NSQT scan and modified context selection (high efficiency)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.08% | -0.01% | -0.02% |  |  |  |
| Class B | 0.10% | 0.14% | 0.16% | 0.23% | 0.43% | 0.24% |
| Class C | 0.06% | 0.05% | 0.06% | 0.14% | 0.10% | 0.12% |
| Class D | 0.07% | 0.11% | 0.14% | 0.08% | 0.09% | 0.27% |
| Class E |  |  |  | 0.27% | 0.32% | 0.51% |
| Overall | 0.08% | 0.08% | 0.09% | 0.18% | 0.24% | 0.27% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

- Conclusion

The revised results provided by the proponent for TI's proposal on Simplified Coefficient Scans for NSQT (JCTVC-G123) were confirmed.

- References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

- V. Sze, “Non-CE11: Simplified Coefficient Scans for NSQT”, JCTVC-G123, Geneva, November 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).