| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G765 WG11 Number: m22335 |
| --- | --- |

| Title: | CE1: Crosscheck of Subtest C – Tests 12 and 13 (JCTVC-G210 and JCTVC-G233) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rajan Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Email: | rajanj@qualcomm.com Tel. +1-858-658-4511 |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of tests 12 and 13, subtest 3 of CE1. The BD-rate results matched those reported by Cisco in JCTVC-G210 and JCTVC-G233. The encoding and decoding times matched closely except for the encoding time for AI-LC case for both the tests.

# Introduction

Test 12 explored used encoding techniques that more accurately reflect real-world constraints. RDOQ was turned off. Also, the probability models were not updated within an LCU during the RDO mode decision process. In this approach the contexts are re-initialized at the beginning of LCU RDO search. As pointed out in JCTVC-G633, another approach is to update the probability models from one LCU to the next, after mode decision has been made.

Test 13 explored the effect of turning off probability model update altogether in LC configurations. In this mode RDOQ is off, CABAC is used with no probability model update. The reference is HM4.0 LC anchor. Please note that in JCTVC-G233, the results are reported with a different anchor.

# Results

For test 12, results for LC configurations were crosschecked for subtest 3 in JCTVC-G210. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding as well as decoding was done on a LINUX cluster (64 bit) by specifying only one type of CPU. For test 13, results for LC configurations were crosschecked for subtest 2 in JCTVC-G233. Table 2 shows a summary of the BD-rate results. Please note that the results in JCTVC-G233 are reported with respect to a different anchor.

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -4.2% | -6.7% | -7.7% |
| Class B | -5.0% | -6.5% | -6.8% |
| Class C | -3.6% | -3.7% | -3.7% |
| Class D | -3.2% | -3.3% | -3.2% |
| Class E | -5.7% | -9.1% | -8.4% |
| Overall | -4.3% | -5.7% | -5.9% |
|  | -4.3% | -5.7% | -5.9% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -6.3% | 0.8% | -1.0% |
| Class B | -5.7% | -2.2% | -3.4% |
| Class C | -4.0% | -0.1% | 0.0% |
| Class D | -3.3% | -1.0% | -0.4% |
| Class E |  |  |  |
| Overall | -4.9% | -0.7% | -1.4% |
|  | -4.5% | -0.7% | -1.3% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -4.2% | 7.0% | 6.8% |
| Class C | -3.1% | 6.5% | 6.8% |
| Class D | -2.3% | 5.8% | 7.2% |
| Class E | -4.5% | -5.0% | -4.8% |
| Overall | -3.5% | 4.3% | 4.7% |
|  | -3.5% | 4.2% | 4.5% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -4.8% | 3.2% | 3.1% |
| Class C | -3.5% | 3.6% | 3.8% |
| Class D | -2.9% | 3.2% | 4.4% |
| Class E | -5.4% | -6.5% | -7.0% |
| Overall | -4.1% | 1.5% | 1.7% |
|  | -4.1% | 1.5% | 1.5% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Table 1: BD-rate results for test 12

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -1.6% | -4.3% | -5.3% |
| Class B | -1.8% | -3.4% | -3.7% |
| Class C | -1.3% | -1.5% | -1.5% |
| Class D | -1.6% | -1.9% | -1.7% |
| Class E | -3.0% | -6.6% | -5.8% |
| Overall | -1.8% | -3.4% | -3.5% |
|  | -1.8% | -3.3% | -3.5% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -0.8% | 6.5% | 4.8% |
| Class B | 0.4% | 3.8% | 2.7% |
| Class C | 0.8% | 4.8% | 5.1% |
| Class D | 1.3% | 3.7% | 4.4% |
| Class E |  |  |  |
| Overall | 0.4% | 4.7% | 4.2% |
|  | 0.9% | 4.7% | 4.2% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 1.3% | 12.8% | 12.6% |
| Class C | 0.9% | 10.6% | 11.1% |
| Class D | 2.4% | 11.1% | 12.1% |
| Class E | 0.8% | 0.1% | 1.1% |
| Overall | 1.4% | 9.4% | 9.9% |
|  | 1.4% | 9.4% | 9.8% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 8.3% | 7.9% |
| Class C | -0.5% | 6.9% | 7.0% |
| Class D | 0.6% | 6.9% | 7.7% |
| Class E | 0.3% | -0.7% | -0.7% |
| Overall | 0.1% | 5.9% | 6.0% |
|  | 0.0% | 5.9% | 5.9% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2: BD-rate results for test 13

# Conclusions

We have verified results reported in JCTVC-G210 and JCTVC-G233 for CE1, subtest C, tests 12 and 13. The BD-rate results matched perfectly. The encoding and decoding times matched except for AI-LC case for both the tests.