| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G766 WG11 Number: m22336 |
| --- | --- |

| Title: | CE1: Crosscheck of Subtest C - Test 5 (JCTVC-G633) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rajan Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Email: | rajanj@qualcomm.com Tel. +1-858-658-4511 |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of test 5, subtest 3 of CE1. The combination of tools tested in this test was PIPE/V2V, 8-bit initialization and low delay mode. The BD-rate results matched those reported by HHI in JCTVC-G633. The encoding times matched closely. The decoding times were higher than reported but our decoding times were measured on a cluster where there is a higher variation. For AI-HE, RA-HE, LB-HE, and LP-HE configurations, the BD-rates were 0.4%, 0.3%, 0.4%, and 0.3%, respectively. When compared against BAC with the same 8-bit initialization as in the proposed method, the BD-rates were 0.4%, 0.5%, 0.8%, and 0.8%, respectively.

# Introduction

In test 5, subtest C of CE1 the following combination of tools was tested:

- PIPE/V2V

- Low delay mode

- 8-bit initialization

The low delay constraint uses multiplexing of v2v codewords with low delay constraint. The low delay buffer size used was 512 bytes.

# Results

Results for all 4 HE configurations were crosschecked. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding as well as decoding was done on a LINUX cluster (64 bit) by specifying only one type of CPU.

The proposed method uses 8-bit initialization. But the HM4.0 anchor does not use 8-bit initialization. To gain insight into performance of PIPE/V2V in conjunction with the low delay mode, we compare the proposed method against HM 4.0 with 8-bit initialization. These are reported in Table 2. We have simply used the results of test 5 and test 6 (BAC with 8-bit initialization) provided by HHI to generate this table.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.3% | -0.4% | -0.5% |
| Class B | 0.4% | -0.5% | -0.6% |
| Class C | 0.4% | -0.6% | -0.7% |
| Class D | 0.4% | -1.3% | -1.6% |
| Class E | 0.4% | -1.1% | -1.0% |
| Overall | 0.4% | -0.8% | -0.9% |
|  | 0.4% | -0.8% | -0.8% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.2% | -0.3% | -0.4% |
| Class B | 0.2% | -0.5% | -0.7% |
| Class C | 0.2% | -0.4% | -0.5% |
| Class D | 0.4% | -0.5% | -0.9% |
| Class E |  |  |  |
| Overall | 0.3% | -0.4% | -0.6% |
|  | 0.3% | -0.4% | -0.7% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.3% | -0.7% | -1.2% |
| Class C | 0.3% | -0.2% | -0.3% |
| Class D | 0.3% | 0.1% | -0.5% |
| Class E | 0.3% | -0.2% | 0.0% |
| Overall | 0.3% | -0.3% | -0.6% |
|  | 0.3% | -0.3% | -0.5% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.4% | -0.6% | -0.9% |
| Class C | 0.4% | -0.2% | -0.1% |
| Class D | 0.4% | 0.7% | -0.3% |
| Class E | 0.6% | 0.0% | 0.1% |
| Overall | 0.4% | 0.0% | -0.3% |
|  | 0.4% | 0.0% | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

Table 1: BD-rate results for the proposed configuration

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.34% | -0.40% | -0.46% |
| Class B | 0.40% | -0.43% | -0.55% |
| Class C | 0.38% | -0.27% | -0.24% |
| Class D | 0.40% | -0.17% | -0.17% |
| Class E | 0.45% | -0.96% | -0.99% |
| Class F | 0.35% | -0.05% | 0.01% |
| Overall | 0.39% | -0.42% | -0.46% |
|  | 0.39% | -0.42% | -0.44% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.34% | -0.36% | -0.14% |
| Class B | 0.38% | -0.68% | -0.73% |
| Class C | 0.47% | -0.15% | -0.35% |
| Class D | 0.88% | 0.13% | 0.01% |
| Class E |  |  |  |
| Class F | 0.25% | 0.12% | -0.15% |
| Overall | 0.51% | -0.29% | -0.33% |
|  | 0.52% | -0.27% | -0.37% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.54% | -0.82% | -1.03% |
| Class C | 0.63% | -0.42% | -0.14% |
| Class D | 0.88% | -0.22% | -0.80% |
| Class E | 1.27% | 0.09% | 0.14% |
| Class F | 0.82% | 0.35% | 0.25% |
| Overall | 0.78% | -0.40% | -0.53% |
|  | 0.79% | -0.41% | -0.42% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.52% | -0.92% | -1.10% |
| Class C | 0.66% | -0.66% | -0.32% |
| Class D | 0.81% | -0.06% | -1.21% |
| Class E | 1.15% | 0.18% | 0.46% |
| Class F | 0.65% | 0.21% | -0.18% |
| Overall | 0.75% | -0.43% | -0.64% |
|  | 0.75% | -0.36% | -0.49% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2: BD-rate results for the proposed method against BAC 8-bit initialization anchor

# Conclusions

We have verified results reported in JCTVC-G633 for test 5, subtest C of CE1. The encoding times closely matched those reported. The decoding times were higher than reported but our decoding times were measured on a cluster where there is a higher variation. . For AI-HE, RA-HE, LB-HE, and LP-HE configurations, the BD-rates were 0.4%, 0.3%, 0.4%, and 0.3%, respectively. When compared against BAC with the same 8-bit initialization as in the proposed method, the BD-rates were 0.4%, 0.5%, 0.8%, and 0.8%, respectively.