| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G769 |
| --- | --- |

| Title: | CE1 subtest A: Line buffer removal for CU split flag context model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the results of “Line buffer removal for CU split flag context model” method proposed in document JCTVC-F497 within the context of CE1. In this proposed method, context model split_coding_unit_flag is selected based on value of left block and coding unit size to avoid access the context of Context of above block. Experiments show that 0.01%, 0.03, 0.11% BD rate loss is observed for, respectively, all intra, random access and low delay configuration by removing the line buffer for split_coding_unit_flag.

# Introduction of tested Method

In current HEVC test model, context model index for split_coding_unit_flag is derived as following,

ctxIdx = isSplit(A) + isSplit(B)                           (1)

Whether the neighbouring unit is split or not can be derived by comparing the CU depth or CU size of neighbouring CU to current CU depth or size, so CU size or CU depth line buffer is needed to decode split_coding_unit_flag syntax.

In this contribution, context model index for split flag is determined as following, jointly by split information of left CU and the current CU size.

ctxIdx = isSplit(A) +(CU_Size>16)                         (2)

With this modification, the context model number does not change with line buffer is removed.

# Experimental results

Experiments are conducted with the common condition mentioned in JCTVC-F900 [2]. Table 1 shows that 0.01%, 0.03, 0.11% BD rate loss is observed for, respectively, all intra, random access and low delay configuration by removing the line buffer for split_coding_unit_flag.

Table 1. Results of modified ctxIdx selection for Split flag

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.02% | -0.06% | -0.04% | -0.01% | -0.35% | -0.07% |  |  |  |
| Class B | 0.01% | -0.02% | -0.04% | 0.02% | 0.07% | -0.01% | 0.05% | 0.03% | -0.10% |
| Class C | 0.00% | -0.04% | -0.09% | 0.08% | -0.01% | 0.09% | 0.09% | 0.00% | 0.07% |
| Class D | 0.02% | -0.04% | -0.04% | 0.02% | 0.11% | -0.01% | -0.03% | 0.01% | -0.10% |
| Class E | 0.05% | -0.03% | -0.04% |  |  |  | 0.45% | -0.27% | 0.73% |
| Overall | 0.01% | -0.04% | -0.05% | 0.03% | -0.04% | 0.00% | 0.11% | -0.04% | 0.10% |
|  | 0.01% | -0.03% | -0.05% | 0.03% | -0.06% | 0.01% | 0.11% | -0.07% | 0.07% |
| Enc Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  | 99% |  |  |

# References

[1]	J.Chen, T.Lee, “Simplified context model selection for block level syntax coding”, JCT-VC Document, JCTVC-F497, Torino, IT, July 2011.

[2]	F. Bossen, “Common test conditions and software reference configurations”, JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).