| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G0770_r3 WG11 Number: m22340 |
| --- | --- |

| Title: | Non-CE3: A restriction of motion vector for small block size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

An experimental result of restriction of motion vector for small block size is reported.  This is a non-normative technology to reduce memory bandwidth for motion compensation.  The worst cases of memory bandwidth of interpolation process are two-dimensional interpolation positions for both luminance and chrominance of bi-prediction block.  Therefore, in order to reduce the worst case of memory bandwidth, for example, two encoding methods that at least one motion vector of L0 or L1 is restricted to the integer position for both luma and chroma when the block size of bi-prediction is less than 8x8 and 4x8 bi-prediction block is prohibited in the former case are introduced at this time.  As experimental results, the gain coding efficiencies are lost an average of 0.33% and 0.38% respectively.  The worst cases of memory bandwidth are reduced greater than 25% and 31% respectively compared to the anchor.

# A restriction of motion vector for small block size

The interpolation filters on current HM are specified 8-tap for luminance and 4-tap for chrominance.  Considering block based processing, memory access of smaller block is increased.  Therefore, in order to reduce memory bandwidth, the restriction of small prediction block size is a useful non-normative method.

When color format is 4:2:0, the resolutions of interpolation of HM are quarter for luminance signal and eighth chrominance signal.  As shown Figure 1, when the luminance is the integer position or the one-dimensional interpolation position, there are cases that the chrominance pixel is a two-dimensional interpolation position.  In Figure 1, “A” indicates the integer position for luminance and “B” indicates the integer position for chrominance.  White “A” is the integer position for both luminance and chrominance and gray “A” is the integer position for luminance, however, the two-dimensional interpolation position for chrominance.  White blocks are the one-dimensional interpolation position for both luminance and chrominance and gray blocks are the two-dimensional interpolation position for both luminance and chrominance.  Black blocks are the one-dimensional interpolation position for luminance and the two-dimensional position for chrominance.  The worst cases of memory bandwidth of interpolation process are two dimensional interpolation positions for both luminance and chrominance of bi-prediction block.

Therefore, if we restrict motion vectors in order to reduce memory bandwidth, we should consider the fractional position for chrominance.

At this time, if the block size of bi-prediction is less than 8x8, we introduce a restriction that at least one motion vector of L0 or L1 shall indicate the integer position for both luminance and chrominance.  The worst case is that one vector indicates the integer position for both luminance and chrominance and the other vector indicates the two dimensional interpolation position for both luminance and chrominance.

Interpolation position for luminance			Interpolation position for chrominance

Figure 1  Relationship of interpolation position for luminance and chrominance (4:2:0)

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-F900 [1].  Table 1 indicates the average results of coding efficiency compared to each anchor.  According to JCTVC-F900, the interpolation filter of anchor is 8-tap DCT-IF.  Used platform is that the OS is Windows 7 64-bit; the CPU is Core™i7-2600, 3.4GHz and the compiler is Visual C++ 2008.

CE3 [2] has conducted computational complexity and memory bandwidth analyses for each proposal.  At this time, computational operations and memory bandwidth for each block size and for each single and bidirectional prediction are measured.  This proposal can reduce worst case and average of computational operations and also average of memory bandwidth.

Table 1	Summary of results

|  | Level 1 |  |  | Level 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Gain Ave.[%] | 0.33% | 0.30% | 0.27% | 0.38% | 0.31% | 0.34% |
| Gain Best [%] | -0.01% | -0.66% | -1.44% | -0.01% | -0.49% | -0.86% |
| Gain Worst [%] | 1.16% | 1.43% | 1.40% | 1.16% | 1.07% | 1.24% |
| Gain Ave. [%] (with LDP) | 0.22% | 0.20% | 0.18% | 0.25% | 0.21% | 0.22% |
| Gain Best [%] (with LDP) | -0.01% | -0.66% | -1.44% | -0.01% | -0.49% | -0.86% |
| Gain Worst [%] (with LDP) | 1.16% | 1.43% | 1.40% | 1.16% | 1.07% | 1.24% |
| Enc Time[%] | 100.06% |  |  | 98.04% |  |  |
| Dec Time[%] | 100.54% |  |  | 100.21% |  |  |
| Mult Ave.[%] | -0.45% |  |  | -0.39% |  |  |
| Mult Worst[%] | -22.41% |  |  | -22.41% |  |  |
| Adds Ave.[%] | -0.45% |  |  | -0.39% |  |  |
| Adds Worst[%] | -22.49% |  |  | -22.49% |  |  |
| Access[%] | -0.75% |  |  | -0.77% |  |  |
| MemBand(2D) Ave.[%] | -1.17% |  |  | -1.24% |  |  |
| MemBand(2D) Max.Frm[%] | 2.62% |  |  | 3.86% |  |  |
| MemBand(2D) Max.CU[%] | -10.19% |  |  | -10.35% |  |  |
| MemBand(2D) Worst[%] | -25.00% |  |  | -35.23% |  |  |
| MemBand(1D) Ave.[%] | -1.10% |  |  | -1.17% |  |  |
| MemBand(1D) Max.Frm[%] | 2.46% |  |  | 3.88% |  |  |
| MemBand(1D) Max.CU[%] | -10.43% |  |  | -10.39% |  |  |
| MemBand(1D) Worst[%] | -31.25% |  |  | -31.25% |  |  |

# Subjective quality

Regarding subjective quality, in order to check ringing noises of interpolation filter, CE3 (JCTVC-F903) recommends that three configurations should be checked and attached the specified still images on the report.  The decoded still images are attached.

It seems to be difficult to find the difference of subjective quality between proposals and HM40 anchor.

# Conclusions

The simulation results of restriction of motion vector for small block size have been shown.  As experimental results the degradation of coding efficiency is small and the worst case of memory bandwidth is improved.  When the worst case of memory bandwidth is reduced, motion vectors should be restricted, considering chrominance fractional positions.

# References

[1] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, July 2011.

[2] 	T. Chujoh and E. Alshina, “CE3: Motion compensation,” Joint Collaborative Team on Video Coding, JCTVC-F903, Torino, July 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).