| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G771 |
| --- | --- |

| Title: | CE1 (subset C, test 11): Multi-parameter probability up-date for PIPE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexander Alshin Elena Alshina JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Heiner Kirchhoffer Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +82 10 30261305 alexander_b.alshin@samsung.com elena_a.alshina@samsung.net jeonghoon@samsung.com Heiner.Kirchhoffer@hhi.fraunhofer.de |
| Source: | Samsung Electronics, Ltd & Fraunhofer HHI |  |  |

# Abstract

In this contribution, an multi-parameter probability up-date technique [1] is proposed for relatively new entropy coding scheme PIPE [2][3]. It should be noted that probability up-date for PIPE coincides with CABAC probability up-date. Proposed method allows more precise probability estimation for current bin which means that more accurate distribution between different bin-encoders becomes possible. In terms of coding efficiency, the presented probability up-date technique on top of PIPE shows an average BD rate gain about 0.4 % in HE configuration..

# Introduction

Among the entropy coding schemes CABAC is known as one of the most efficient. Using of elegant interval subdivision technique allows CABAC to be very close to entropy limit.

However for CABAC case there are almost no possibilities for parallelism. That is why throughput and latency may be a problem for CABAC. PIPE partially solved this problem. Instead of interval subdivision technique PIPE uses systematic v2v codes. They can be efficiently implemented by using simple counters instead of using memory consuming tables. Every code has specific probability interval where its performance is close to entropy limit. Next figure illustrated it on the example of “Three bin code” Suppose that probability of “1” is equal p then

Table 1. Name….

| Index | Primary | Probability | Secondary |
| --- | --- | --- | --- |
| 1 | 000 | (1-p)3 | 0 |
| 2 | 001 | (1-p)2p | 100 |
| 3 | 010 | (1-p)2p | 101 |
| 4 | 100 | (1-p)2p | 110 |
| 5 | 110 | (1-p)p2 | 11100 |
| 6 | 101 | (1-p)p2 | 11101 |
| 7 | 011 | (1-p)p2 | 11110 |
| 8 | 111 | P3 | 11111 |

The estimated value for number of bins in secondary code is equal

((1-p)3+9(1-p)2p+15(1-p)p2 +5p3 )/3

Fig. 1. Performance of three bin code compare to entropy limit.

Taking into account that PIPE (as CABAC) uses MPS/LPS switching so codes should cover only   [0, 0.5] probability diapason. PIPE uses CABAC probability up-date scheme and for every current bin after probability estimation PIPE determines which bin- encoder will be used.

# 2 Probability Up-date.

In CABAC probabilities are represented by one of 64 discrete values on exponential mesh pn=0.5(1-)n  covered interval from 0 to 1/2.

Fig. 2. Exponential mesh for probability.

Every probability up-date is going on using look-up tables. This process can be roughly described by following formula:

pnew = y+(1- ) pold.                (1)

Here y is equal to “zero” if current symbol matches with most probable symbol (MPS) otherwise y is equal to “one”. This formula provides estimating value for probability of least probable symbol (LPS).

The parameter N=1/ (for CABAC it is approximately 19.6) is a measure for number of previously encoded bins which have significant influence to the current up-date. This parameter determines sensitivity of model. Sensitive system quickly reacts to real changing but less sensitive model does not react to noise and random errors. Both properties are useful but contradictory.

The main idea of proposed technique is to use several probability estimations (instead of one) with different  and combine them as weighted average for next bin probability prediction.

pi new =  i y+(1- i) pi old                          (2)

p new =i pi new.             (3)

For short transition period only short distance prediction is preferable. But after stabilization near optimal value two-probability update model is more accurate for majority of context models.

We proposed to introduce a counter of updates since last initialization. After every up-date the counter increases by one. Until the counter exceeds some threshold only short “window size” model (4) will be used. When the counter reaches threshold we should switch to more accurate two-probability update model (6).

Fig. 3. Multi-parameter probability up-date diagram (two probabilities model).

# 3 Test results

Multi-parameter probability up-date technique was implemented on top of [5] which was accomplished in HM 4.0. It should be noticed that initialization and quantization of probability are not changed in our implementation. Performance was tested according to [4] in high efficiency case. The results are shown in Table 2. BD-rate reported compare to an anchor which is PIPE [5].

Table2 . Test results for multi-parameters probability up-date model in HM4.0 on top of JCTVC-F268.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | -0.5% | -0.6% | -0.7% | -0.2% | -0.1% |  |  |  |
| Class B | -0.5% | -0.9% | -0.9% | -0.4% | -0.6% | -0.5% | -0.4% | -0.5% | -0.5% |
| Class C | -0.6% | -0.7% | -0.7% | -0.3% | -0.2% | -0.1% | -0.2% | -0.3% | -0.3% |
| Class D | -0.6% | -0.4% | -0.4% | -0.3% | 0.1% | 0.0% | -0.1% | -0.2% | -0.0% |
| Class E | -0.5% | -1.1% | -1.2% |  |  |  | -0.1% | -0.9% | -0.7% |
| Overall | -0.6% | -0.7% | -0.7% | -0.4% | -0.2% | -0.2% | -0.2% | -0.5% | -0.4% |
| Enc Time[%] | 103% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  | 103% |  |  |

Multi-parameter probability up-date provides about 0.4% bit saving for PIPE but same technique provides 0.9% for CABAC.

Even if after probability up-date we have more accurate probability estimation due to quantization current bin may stay in the same bin-encoder. However in CABAC this method has influence on each bin encoding. Nevertheless this method has noticeable positive effect for PIPE coding efficiency

# Conclusions

Based on test results:

0.4% (Y) / 0.5% (U) /0.5% (V) performance improvement on top of PIPE

we would like to propose usage of multi-parameter probability up-date in PIPE-HE.

# References

- Alexander Alshin, Elena Alshina “Multi-parameter probability up-date for CABAC”, JCTVC-F254, Torino, ITA, March July 2011

- Detlev Marpe, Heiko Schwarz, and Thomas Wiegand, “Novel entropy coding concept”, JCTVC-A032, Dresden, DE, April 2010.

- Dake He, Gergely Korodi, Gaelle Martin-Cocher, en-hui Yang, Xiang Yu, and Jinwen Zan, “Video coding Technology Proposal by RIM”, JCTVC-A120, Dresden, DE, April 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- D. Marpe, H. Kirchhoffer, B. Bross, V. George, T. Nguyen, M. Preiß, M. Siekmann, J. Stegemann, T. Wiegand (Fraunhofer HHI) “Unified PIPE-Based Entropy Coding for HEVC”, JCTVC-F268, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).