| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G772 |
| --- | --- |

| Title: | Cross-check result of JCTVC-G353: Non-CE4: QP prediction using spatial QP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document reports cross-check result of JCTVC-G353: Non-CE4: QP prediction using spatial QP.

# Introduction

In current HM specification, QP coding is as follows:

dQP = QP – predQP

if (Left_available) predQP = leftQP

else            predQP = lastcodedQP

Under CE4 better QP prediction methods are now being investigated.

JCTVC-G353 [3] proposes the following weighted prediction of QP as follow:

- If the QPl is equal to the QPal:

predQP=QPl+3∗QPa+2>>2

- Otherwise, if the QPa is equal to the QPal

predQP=3∗QPl+QPa+2>>2

- Otherwise,

predQP=QPl+QPa+1>>1

Where QPa, QPl, QPal are QPs for above, left and aboveleft CUs respectively.

# Simulation Condition

HM-4.0 (including QP adaptation) is used as anchor with options “-aq 1 –aqr 12” [2]. To add dQP ratio output function patch provided by NEC via CE4 ML has been applied.

On top of this anchor software the proponent implemented the proposed method and provided to the author. It is confirmed that the proposed method is implemented as described in the document.

For test of the proposed method the following macro in “Typedef.h” should be set:

#define JVC_QP_PREDICTION                 1

IT/IT_Loco/RA/RA_Loco/LD/LD_Loco conditions have been tested with class A to E sequences [1].

# Simulation Result

Summary of the result is shown in Table 3.1. Complete result is shown in JCTVC-G772.xls.

The result provided by the proponent has been verified.

Table 3.1

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.1% | -0.1% | -0.1% | -3.4% | 0.0% | 0.0% | 0.0% | -0.9% |
| Class B | -0.2% | -0.2% | -0.2% | -5.8% | -0.2% | -0.2% | -0.2% | -4.3% |
| Class C | -0.4% | -0.4% | -0.4% | -8.2% | -0.3% | -0.3% | -0.3% | -7.3% |
| Class D | -0.4% | -0.3% | -0.3% | -8.9% | -0.4% | -0.3% | -0.4% | -8.2% |
| Class E | -0.5% | -0.5% | -0.4% | -7.9% | -0.3% | -0.3% | -0.3% | -6.1% |
| Overall | -0.3% | -0.3% | -0.3% | -6.7% | -0.2% | -0.2% | -0.2% | -5.3% |
|  | -0.3% | -0.3% | -0.3% |  | -0.2% | -0.2% | -0.2% |  |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A | -0.2% | -0.1% | -0.4% | -4.9% | -0.1% | -0.1% | -0.2% | -3.5% |
| Class B | -0.3% | -0.2% | -0.2% | -6.8% | -0.3% | -0.3% | -0.3% | -6.1% |
| Class C | -0.4% | -0.3% | -0.5% | -8.9% | -0.5% | -0.4% | -0.4% | -9.0% |
| Class D | -0.4% | -0.4% | -0.3% | -8.6% | -0.4% | -0.6% | -0.5% | -8.7% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.3% | -7.3% | -0.3% | -0.4% | -0.3% | -6.8% |
|  | -0.3% | -0.3% | -0.3% |  | -0.3% | -0.4% | -0.3% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | dQP incr. | Y | U | V | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.3% | -0.5% | -0.5% | -7.4% | -0.4% | -0.4% | -0.4% | -7.0% |
| Class C | -0.4% | -0.2% | -0.3% | -9.0% | -0.6% | -0.5% | -0.4% | -9.6% |
| Class D | -0.4% | -1.0% | -0.2% | -8.6% | -0.5% | -0.4% | -0.3% | -9.1% |
| Class E | -0.4% | 0.1% | -0.6% | -9.8% | -0.5% | -1.5% | -0.1% | -9.7% |
| Overall | -0.4% | -0.4% | -0.4% | -8.6% | -0.5% | -0.6% | -0.3% | -8.6% |
|  | -0.4% | -0.4% | -0.4% |  | -0.5% | -0.6% | -0.4% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |

# Conclusion

Result of JCTVC-G353 is verified.

With the proposed method Gain is obtained by -0.3%, -0.2%, -0.3%, -0.3%, -0.4% and -0.5% for IT/IT_Loco/RA/RA_Loco/LD/LD_Loco conditions respectively.

The author recommends considering harmonization with other proposals like subtest 1.3.c in CE4 [4].

# References

[1] “Common test conditions and software reference configurations”, JCTVC-F900, Jul 2011

[2] “Description of Core Experiment 4: Quantization,” JCTVC-F904, Jul 2011

[3] “Non-CE4: QP prediction using spatial QP”, JCTVC-G353, Nov 2011

[4] “CE4 Subtest 1: Spatial QP prediction based on intra prediction and prediction mode (test 1.3.c)”, JCTVC-G067, Nov 2011

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.