| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G773 |
| --- | --- |

| Title: | CE4 Subtest 1.2.c: Higher granularity of quantization parameter scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park Keiichi Chono | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com chono@ct.jp.nec.com |
| Source: | Samsung Electronics Co., Ltd., NEC Corporation |  |  |

_____________________________

# Abstract

This document reports the results of “Higher granularity of quantization parameter scaling and adaptive delta QP signaling” method proposed in document JCTVC-F495 within the context of CE4. In current HEVC, the quantization step size increases by approximately 12.25% with each increment of QP which lead to average 18.7% and up to 44.8% bit increase. This contribution proposes to increase the granularity of the QP scaling in HEVC test model, i.e. the quantization step size increases by approximately 5.95% with each increment of QP which lead to average 9.0% and up to 20.7% bit increase. In addition to higher granularity, this contribution also proposes to specify dQP_scale value at the slice level to support the tradeoff between dQP bits overhead and fine rate/quality control.

- Introduction of proposed method

## Fine granularity of quantization step size scaling

The current HEVC Test Model under Consideration employs a quantization parameter (QP) scaling scheme that is same to the AVC standard. The quantization step size applied to each transform coefficient is determined approximately as follows:

Qstep≈2QP−46, with QP = 0…51                            (1)

According this equation, the quantization step size increases by approximately 12.25% with each increment of QP and the quantization step size doubles when QP is incremented by 6. In this contribution, we propose to use a higher granularity of QP scaling to support more accurate bit allocation and perceptual quality control. We propose to the straightforward extension of current scheme by doubling the QP scaling accuracy.

Qstep'≈2QP'−812, with QP’ = 0…103                         (2)

With the new equation, same quantization step size derived when the QP in proposed scheme is twice of that in original one. This simple extension results in very easy adaptation of other modules in which the operation depends on QP value, such as deblocking filter, CABAC context model initialization. The quantization step size increases by approximately 5.95% with each increment of QP.

We conducted the experiments with HM4.0 software with common test condition by using QP (22, 27, 32, 37) and QP-1 (21, 26, 31, 36), and corresponding proposed solution by using QP’ (44, 54, 64, 74) and QP’-1 (43, 53, 63, 73).

Table 1 shows the average bit rate increase when QP is reduced by 1. With current Qstep scaling scheme, average 18.7% bit increase in four inter test configurations and up to 44.8% bit increase is observed when QP is reduced by 1. With the proposed Qstep scaling scheme, average around 9.0% bit increase in four inter test configurations and up to 20.7% bit increase is observed when QP is reduced by 1.

Table 1. Bit rate increase of QP-1 vs QP in HM4.0

| Test configuration | Bit rate increase |  |  |  |
| --- | --- | --- | --- | --- |
|  | HM4.0 |  | Proposed method |  |
|  | Average | Max | Average | Max |
| HE_AI | 12.6% | 29.2% | 6.3% | 14.0% |
| LC_AI | 12.4% | 25.9% | 6.2% | 12.5% |
| HE_RA | 18.3% | 36.6% | 8.8% | 17.7% |
| LC_RA | 18.0% | 38.6% | 8.7% | 18.0% |
| HE_LD | 19.6% | 44.8% | 9.4% | 20.7% |
| LC_LD | 19.1% | 43.9% | 9.1% | 20.3% |

## Adaptive delta QP signaling

The fine granularity of QP scaling improves rate control accuracy for certain applications. However, it introduces disadvantage since more overhead bits are necessary to code delta QP to achieve same level of quantization step change.

We propose to insert one high-level syntax parameter dQP_scale into slice head to support the flexibility of tradeoff between better rate/quality allocation and dQP bits overhead. In the proposed method, QP of current block is calculated with equation (3), where dQP_scale means QP value change scale when dQP is 1.

QP_curr = QP_pred + dQP * dQP_Scale                            (3)

In the proposed method, the slice level QP scaling granularity is always kept as the highest level. With change of dQP_Scale, the CU level QP update speed is adaptively selected by different dQP_Scale according to application requirement. When dQP_Scale is equal to 2, CU level Quantization step adaptation level is same to that of current HEVC quantization scheme, meanwhile, still keeping the higher quantization step size granularity at slice level. Figure 1 shows how to match the current HEVC quantization scheme with the proposed syntax. If the slice QP is even number and dQP_Scale is 2, then current HEVC quantization scheme can be emulated with the same dQP values.

dQP_Scale = 2

dQP_Scale = 1

QP01234567891011…QStep0.6250.68750.81250.87511.1251.251.3751.6251.7522.25…

QP0246810121416182022…QStep0.6250.68750.81250.87511.1251.251.3751.6251.7522.25…

Figure 1. AVC style mapping with dQP_Scale=2

dQP_Scale = 3

dQP_Scale = 2

dQP_Scale = 1

QP prediction

Figure 2. dQP=1 with different dQP_Scale

Figure 2 shows QP value change from QP prediction according to dQP_Scale when dQP is equal to 1. QP prediction can be any value and the range of QP is within 0 to 103. This solution is integrated in CE4 subtest 1 software to interpret how dQP_Scale will control the dQP overhead bits. The outputted quantization step of CE4 subtest1 software is interpreted into floating QP with higher mapping granularity which value is exactly double of the anchor. Then the floating QP is rounding to integer QP with different accuracy according to dQP_Scale level. In this experiment, same level Qstep update with different rounding accuracy is tested. When dQP_scale is set to 2, almost same results to CE4 subtest2 are attained. Table 2 and Table 3 show the result compared to CE4 subtest 1 anchor when dQP_scale is 1 and 3. To keep same level Qstep update with higher rounding accuracy, dQP_Scale = 1 has BD-rate 1.4% loss (almost same PSNR with 1.4% bit rate increasing). In comparison of dQP bits, about 32.5% dQP bits are increased comparing to anchor. To keep same level Qstep update with coarse rounding accuracy, dQP_Scale = 3 has BD-rate -0.8% gain (almost same PSNR with 0.8% bit rate decreasing). In comparison of dQP bits, about 16.8% dQP bits are reduced comparing to anchor.

Table 2. dQP_Scale=1 in comparing to CE4 subtest 1 anchor

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 1.1% | 0.6% | 0.7% | 30.5% | 1.2% | 0.7% | 0.6% | 35.2% |
| Class B | 1.3% | 0.8% | 0.9% | 32.8% | 1.4% | 0.9% | 0.9% | 36.3% |
| Class C | 1.4% | 0.8% | 0.8% | 31.4% | 1.7% | 1.0% | 1.1% | 36.3% |
| Class D | 1.2% | 0.8% | 0.8% | 29.6% | 1.6% | 0.9% | 1.0% | 34.9% |
| Class E | 1.7% | 1.6% | 1.5% | 30.9% | 2.0% | 1.6% | 1.6% | 35.7% |
| All | 1.3% | 0.9% | 0.9% | 31.1% | 1.6% | 1.0% | 1.0% | 35.7% |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 1.0% | 0.4% | 0.0% | 28.8% | 1.3% | 0.5% | 0.3% | 34.2% |
| Class B | 1.2% | 0.4% | 0.5% | 30.7% | 1.5% | 0.6% | 0.6% | 35.6% |
| Class C | 1.3% | 0.6% | 0.7% | 29.4% | 1.7% | 0.9% | 1.1% | 34.9% |
| Class D | 1.1% | 0.6% | 0.6% | 29.1% | 1.6% | 0.9% | 0.6% | 34.3% |
| Class E |  |  |  |  |  |  |  |  |
| All | 1.2% | 0.5% | 0.4% | 29.6% | 1.5% | 0.8% | 0.7% | 34.8% |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.4% | 1.2% | 1.4% | 29.7% | 1.8% | 1.5% | 1.2% | 35.3% |
| Class C | 1.4% | 1.0% | 1.3% | 28.7% | 1.9% | 1.5% | 1.6% | 34.6% |
| Class D | 1.3% | 0.8% | 1.9% | 28.8% | 1.9% | 1.3% | 1.3% | 33.8% |
| Class E | 1.6% | 1.2% | 1.0% | 29.2% | 1.9% | 1.2% | 1.2% | 34.2% |
| All | 1.4% | 1.0% | 1.4% | 29.1% | 1.9% | 1.4% | 1.3% | 34.5% |

Table 3. dQP_Scale=3 in CE4 subtest 1 anchor

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.6% | -0.8% | -0.8% | -17.0% | -0.5% | -0.9% | -0.9% | -17.4% |
| Class B | -0.6% | -1.0% | -1.0% | -17.6% | -0.6% | -1.0% | -1.0% | -17.4% |
| Class C | -0.6% | -1.2% | -1.1% | -17.2% | -0.7% | -1.3% | -1.2% | -17.7% |
| Class D | -0.5% | -0.9% | -0.9% | -16.5% | -0.6% | -1.1% | -1.0% | -17.5% |
| Class E | -0.9% | -0.9% | -0.8% | -16.5% | -0.8% | -1.2% | -1.2% | -17.0% |
| All | -0.6% | -1.0% | -0.9% | -17.0% | -0.6% | -1.1% | -1.1% | -17.4% |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.5% | -1.3% | -0.9% | -16.2% | -0.6% | -1.3% | -1.3% | -17.4% |
| Class B | -0.6% | -1.4% | -1.3% | -16.9% | -0.7% | -1.6% | -1.5% | -17.6% |
| Class C | -0.6% | -1.2% | -1.3% | -16.4% | -0.7% | -1.6% | -1.2% | -17.6% |
| Class D | -0.5% | -1.1% | -1.0% | -16.5% | -0.7% | -1.5% | -1.6% | -17.7% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.6% | -1.2% | -1.1% | -16.5% | -0.7% | -1.5% | -1.4% | -17.6% |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.8% | -0.7% | -0.8% | -16.5% | -0.8% | -0.9% | -1.0% | -17.8% |
| Class C | -0.8% | -1.2% | -0.7% | -16.2% | -0.9% | -1.3% | -1.1% | -17.5% |
| Class D | -0.6% | -1.3% | -0.5% | -16.5% | -0.8% | -1.4% | -1.3% | -17.7% |
| Class E | -0.6% | 0.1% | -1.4% | -16.3% | -0.9% | -2.1% | -0.7% | -17.4% |
| All | -0.7% | -0.8% | -0.8% | -16.4% | -0.9% | -1.4% | -1.1% | -17.6% |

- Conclusion

It was found in HM4.0 that the bit rate increasing is average 18.7% and up to 44.8% when QP is reduced by 1. QStep scaling scheme with higher granularity is proposed since the current scheme may be too coarse for rate control and quality allocation. A high-level syntax parameter is also introduced to support the tradeoff between dQP bits overhead and fine rate/quality control. All operations in quantization process are fixed without regarding to dQP_Scale adaption. We recommend adopting the proposed QStep scaling scheme into HM.

- Reference

[1]	J.Chen, T.Lee, “MVP index parsing with fixed number of candidates”, JCT-VC Document, JCTVC-F402, Torino, IT, July 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- Specification

- In 7.3.3 Slice header syntax,

| if( !lightweight_slice_flag ) { |  |
| --- | --- |
| slice_qp_delta | se(v) |
| scale_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |

In 7.4.3 Slice header semantics,

scale_qp_delta specifies how much cu_qp_delta is scaled up. scale_qp_delta shall be inferred to be equal to 2 when it is not present. The decoded value of scale_qp_delta shall be the greater than or equal to 1.

In 7.4.9 Transform coefficient semantics,

QPY=(((QPY,PREV+cu_qp_delta*scale_qp_delta +104+ 2*QpBdOffsetY)%(104+QpBdOffsetY))-QpBdOffsetY  (7-1)

In 8.5.3,

dij = ( cij * LevelScale[ qP%12 ] ) << ( qP/12 + trafoPrecisionExt ), with i, j = 0..nS-1	(8-2)

Where, LevelScale[12] is {40, 42, 45, 48, 51, 54, 57, 60, 64, 68, 72, 76}.