| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G774 |
| --- | --- |

| Title: | Non-CE8: Modified ALF DC coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Takashi Watanabe1 Takeshi Chujoh1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Marta Karczewicz3 In Suk Chong3 | Email: | tomoo.yamakage@toshiba.co.jp takashi39.watanabe@toshiba.co.jp takeshi.chujoh@toshiba.co.jp chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com martak@qualcomm.com ichong@qualcomm.com |
| Source: | TOSHIBA Corporation1, MediaTek Inc.2, and Qualcomm3 |  |  |

_____________________________

# Abstract

This evaluates a modified ALF DC coefficients. This technique uses less bit-depth for ALF DC coefficients. Coding efficiency gain for luma is 0.0 %, 0.1 %, 0.1% and 0.1 % in HE-AI, RA, LB, and LP without encoding/decoding time increase on average.

# Introduction

In the current HM4.0 ALF design, AC and DC coefficients of ALF uses the same bitdepth (i.e., 9 bits). But we found that DC coefficients requires less bitdepth than AC coeffieints. We propose to use less bit-depth for DC coefficients.

# Changes in Working Draft Text

#### 8.6.3.2 Derivation process for filter coefficients

Outputs of this process are filter coefficients cL for the luma samples and filter coefficients cC for the chroma samples.

The luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters – 1, j = 0..AlfCodedLengthLuma – 1 is derived as follows:

- If alf_pred_method is equal to 0 or the value of i is equal to 0,

cL[ i ][ j ] = alf_coeff_luma[ i ][ j ]		(8-1)

- Otherwise (alf_pred_method is equal to 1 and the value of i is greater than 1),

cL[ i ][ j ] = alf_coeff_luma[ i ][ j ] + cL[ i – 1 ][ j ]		(8-2)

Considering the symmetry of the filter, the luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters – 1 is derived as follows:

cL[ i ][ AlfLengthLuma – 1 ] = (cL[ i ][ AlfCodedLengthLuma – 1 ]	 << 7)(8-3)

cL[ i ][ AlfLengthLuma – 2 – j ] = cL[ i ][ j ] for j = 0..AlfCodedLengthLuma – 2	(8-4)

# Simulation results

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% |
| Class B | -0.1% | 0.0% | 0.0% |
| Class C | -0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.2% | 0.1% |
| Class E |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% |
| Class C | -0.1% | 0.0% | 0.1% |
| Class D | -0.1% | 0.2% | 0.0% |
| Class E | -0.1% | 0.3% | 0.8% |
| Overall | -0.1% | 0.1% | 0.1% |
|  | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | 0.0% | 0.1% |
| Class D | -0.1% | 0.0% | -0.1% |
| Class E | -0.2% | 0.4% | 0.0% |
| Overall | -0.1% | 0.1% | 0.0% |
|  | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusions

In this proposal, the coding efficiency is improved by adopting modified DC coefficients of ALF. Coding efficiency gain for luma is 0.0 %, 0.1 %, 0.1% and 0.1 % in HE-AI, RA, LB, and LP. There was no encoding/decoding time increase on average.

# Patent rights declaration(s)

TOSHIBA Corporation, MediaTek Inc., and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).