| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G776 |
| --- | --- |

| Title: | CE13: Merge candidates list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the results of “merge candidates list construction” method proposed in document JCTVC-F402 within the context of CE13. Three tests were done in this contribution: a) encoder modification of HM4.0; b) Optimal merge list size derived in encoder side and signal it in slice head; c) inserting additional merge candidates. Experiments show that the encoder side fix achieves average -0.11% BD rate saving for six inter configurations without enc/dec time change. Merge list size signaling in slice head shows average 0.08% BD rate loss with 96.9% encoding time. Inserting additional merge candidates shows average -0.17% BD rate saving for six inter configurations based on encoder fix version with 102.8% encoding time.

# Introduction of proposed method

## Encoder side fix

In HM4.0, the bits number for merge index during compression stage is accurately accounted. In this document, we corrected the bits number for merge index compression stage. This test is marked as Test0 in the contribution.

## Adaptive merge list size

In HM4.0, the merge list size is fixed as 5. In the proposed method, the maximum merge list size of each slice can be different in a sequence and a high-level parameter maxNumMergeCand is signaled in slice header. The value of maxNumMergeCand can vary from 1 to 5 and its value can be set according to user application or heuristically derived with encoder algorithm. In this test, the maxNumMergeCand of each slice is derived according to merge list candiates statistics of previous coded slices.

This test is marked as Test1 in the contribution.

## Additional merge candidates

The fixed list size of Merge/AMVP candidates is adopted with additional Merge/AMVP candidates insertion in HM4.0. However, it’s still possible that the Merge list is not fully filled with additional merge candidates. In this test, new additional candidates are inserted to the merge list if the current merge list size is less than 5. The additional candidates are constructed by adding MV offset to the first merge candidate and copying its prediction direction and reference indexes information. Giving the first merge candidate has list 0 mv (mx0, my0) and list1 mv (mx1, my1), following merge candidates are added to the merge list in turn.

- If the current picture lies between two reference pictures,

  - (mx0+1, my0) and (mx1-1, my1),

  - (mx0-1, my0) and (mx1+1, my1),

  - (mx0, my0+1) and (mx1, my1-1),

  - (mx0, my0-1) and (mx1, my1+1)

- Otherwise (if the current picture lies in the same direction from two reference pictures),

  - (mx0+1, my0) and (mx1+1, my1),

  - (mx0-1, my0) and (mx1-1, my1),

  - (mx0, my0+1) and (mx1, my1+1),

  - (mx0, my0-1) and (mx1, my1-1)

This test is marked as Test3 in the contribution.

# Experimental results

HM4.0 is used as anchor and six inter configuration, random access, low delay B, low delay P for both HE and LC configurations are checked.

## Test0: Encoder side fix

In this test, HM4.0 + MERGE_ENC_FIX is compared with HM4.0. Table 1 shows that the encoder side fix achieves average -0.11% BD rate saving for six inter configurations without enc/dec time change.

Table 1. Encoder side fix (anc: HM4.0)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | 0.1% |
| Class B | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
|  | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.4% | -0.2% | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | 0.0% | -0.2% | -0.1% | -0.1% | -0.4% |
| Class D | -0.2% | -0.2% | -0.2% | -0.1% | 0.1% | -0.3% |
| Class E | -0.5% | 0.0% | 0.5% | -0.2% | 0.0% | -0.2% |
| Overall | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% | -0.2% |
|  | -0.2% | -0.2% | -0.1% | -0.1% | 0.0% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | -0.4% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.3% | 0.2% | -0.8% | 0.0% | -0.3% | -0.7% |
| Overall | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
|  | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Test 1: Adaptive merge list size

In this test, HM4.0 + ADA_MERGE_LIST_SIZE is compared with HM4.0. Table 2 shows proposed adaptive merge list size shows average 0.08% BD rate loss for six inter configuration with 96.9% encoding time. Especially -0.1% BD rate gain is achieved for Low delay P configuration with around 96% encoding time.

Table 2. Adaptive merge list size (anc: HM4.0)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
| Class B | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class C | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% |
| Class D | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.0% | 0.0% | 0.3% | 0.2% | 0.1% |
| Class C | 0.2% | 0.2% | 0.3% | 0.2% | 0.4% | 0.0% |
| Class D | 0.2% | 0.6% | -0.2% | 0.2% | 0.0% | -0.1% |
| Class E | 0.1% | 0.3% | 0.9% | 0.3% | 0.1% | 0.3% |
| Overall | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% |
|  | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | -0.1% |
| Class C | -0.1% | 0.1% | 0.0% | -0.1% | -0.2% | -0.2% |
| Class D | -0.1% | 0.1% | -0.4% | -0.1% | -0.1% | -0.4% |
| Class E | -0.1% | -0.1% | -0.4% | -0.2% | -0.7% | -0.3% |
| Overall | -0.1% | 0.1% | -0.1% | -0.1% | -0.2% | -0.2% |
|  | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% | -0.3% |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Test3: Additional merge candidates

In this test, HM4.0 + MERGE_ENC_FIX is used as anchor. Table 3 shows Inserting additional merge candidates shows average -0.17% BD rate saving for six inter configurations based on MERGE_ENC_FIX with 102.8% encoding time.

Table 3. Additional merge candidates (anc: HM4.0+MERGE_ENC_FIX)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.4% |
| Class B | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | -0.1% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class D | -0.2% | 0.1% | 0.2% | -0.2% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% |
|  | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.0% | -0.2% | 0.2% | 0.1% |
| Class C | -0.1% | -0.2% | -0.1% | -0.1% | 0.2% | 0.2% |
| Class D | -0.2% | 0.5% | 0.3% | -0.3% | -0.2% | -0.1% |
| Class E | 0.2% | 0.8% | 0.0% | 0.1% | 1.5% | 0.3% |
| Overall | -0.1% | 0.3% | 0.0% | -0.1% | 0.4% | 0.1% |
|  | -0.1% | 0.3% | 0.0% | -0.1% | 0.3% | 0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.4% | 0.4% | -0.4% | 0.5% | 0.2% |
| Class C | -0.2% | 0.1% | 0.0% | -0.3% | 0.0% | -0.1% |
| Class D | -0.4% | 0.0% | -0.2% | -0.7% | -0.6% | -0.7% |
| Class E | 0.1% | 0.2% | 0.5% | -0.2% | 1.1% | -0.1% |
| Overall | -0.2% | 0.2% | 0.2% | -0.4% | 0.2% | -0.2% |
|  | -0.2% | 0.2% | 0.2% | -0.4% | 0.2% | -0.2% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## Test5: combined test1 and test3

In this test, test1 and test3 is jointly tested to check the synergy between two methods. This test is marked as Test5 in CE13. Table4 shows average 0.03% BD rate loss for six inter configuration with 99.2% encoding time

Table 4. Test1+Test3 (anc: HM4.0+MERGE_ENC_FIX)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -0.1% | 0.1% | 0.1% | -0.1% |
| Class B | 0.1% | 0.2% | 0.3% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.2% |
| Class D | 0.0% | 0.1% | 0.4% | 0.0% | -0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.5% | 0.2% | 0.2% | 0.3% | 0.4% |
| Class C | 0.2% | 0.1% | 0.2% | 0.2% | 0.4% | 0.4% |
| Class D | 0.1% | 0.4% | 0.4% | 0.0% | 0.1% | 0.4% |
| Class E | 0.5% | 0.4% | 0.2% | 0.6% | 1.8% | 0.5% |
| Overall | 0.2% | 0.4% | 0.3% | 0.2% | 0.5% | 0.4% |
|  | 0.2% | 0.3% | 0.3% | 0.2% | 0.5% | 0.4% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.4% | 0.4% | -0.3% | 0.3% | 0.1% |
| Class C | -0.1% | 0.0% | -0.1% | -0.2% | 0.2% | 0.1% |
| Class D | -0.3% | 0.2% | -0.2% | -0.5% | -0.2% | -0.3% |
| Class E | 0.0% | 0.7% | -0.3% | 0.0% | 1.2% | 0.1% |
| Overall | -0.1% | 0.3% | 0.0% | -0.3% | 0.3% | 0.0% |
|  | -0.1% | 0.3% | 0.0% | -0.3% | 0.3% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).