| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G776 |
| --- | --- |

| Title: | CE13: Merge candidates list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the results of “merge candidates list construction” method proposed in document JCTVC-F402 within the context of CE13. Three tests were done in this contribution: a) encoder modification of HM4.0; b) Optimal merge list size derived in encoder side and signal it in slice head; c) inserting additional merge candidates. Experiments show that the encoder side fix achieves average -0.11% BD rate saving for six inter configurations without enc/dec time change. Merge list size signaling in slice header shows average 0.08% BD rate loss with 96.9% encoding time. Inserting additional merge candidates shows average -0.17% BD rate saving for six inter configurations based on encoder fix version with 102.8% encoding time.

# Introduction of proposed method

## Encoder side fix

In HM4.0, the number of bits for merge index coding is not considered during merge estimation stage for normal merge modes. In this proposal, the number of bits for merge index coding is considered as estimated bits without entropy coding in merge estimation stage. This test is marked as Test0 in the contribution.

## Adaptive merge list size

In HM4.0, the merge list size is fixed as 5. In the proposed method, the maximum merge list size of each slice can be different in a sequence and a high-level parameter maxNumMergeCand is signaled in slice header. The value of maxNumMergeCand can vary from 1 to 5 and its value can be set according to user application or heuristically derived with encoder algorithm. In this test, the maxNumMergeCand of each slice is derived according to merge list candiates statistics of previous coded slices.

This test is marked as Test1 in the contribution.

## Additional merge candidates

The fixed list size of Merge/AMVP candidates is adopted with additional Merge/AMVP candidates insertion in HM4.0. However, it’s still possible that the Merge list is not fully filled with additional merge candidates. In this test, new additional candidates are inserted to the merge list if the current merge list size is less than 5. The additional candidates are constructed by adding MV offset to the first merge candidate and copying its prediction direction and reference indexes information. Giving the first merge candidate has list 0 mv (mx0, my0) and list1 mv (mx1, my1), following merge candidates are added to the merge list in turn.

- If the current picture lies between two reference pictures,

  - (mx0+1, my0) and (mx1-1, my1),

  - (mx0-1, my0) and (mx1+1, my1),

  - (mx0, my0+1) and (mx1, my1-1),

  - (mx0, my0-1) and (mx1, my1+1)

- Otherwise (if the current picture lies in the same direction from two reference pictures),

  - (mx0+1, my0) and (mx1+1, my1),

  - (mx0-1, my0) and (mx1-1, my1),

  - (mx0, my0+1) and (mx1, my1+1),

  - (mx0, my0-1) and (mx1, my1-1)

This test is marked as Test3 in the contribution.

# Experimental results

HM4.0 is used as anchor and six inter configuration, random access, low delay B, low delay P for both HE and LC configurations are checked.

## Test0: Encoder side fix

In this test, HM4.0 + MERGE_ENC_FIX is compared with HM4.0. Table 1 shows that the encoder side fix achieves average -0.11% BD rate saving for six inter configurations without enc/dec time change.

Table 1. Encoder side fix (anc: HM4.0)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | 0.1% |
| Class B | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
|  | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.4% | -0.2% | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | 0.0% | -0.2% | -0.1% | -0.1% | -0.4% |
| Class D | -0.2% | -0.2% | -0.2% | -0.1% | 0.1% | -0.3% |
| Class E | -0.5% | 0.0% | 0.5% | -0.2% | 0.0% | -0.2% |
| Overall | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% | -0.2% |
|  | -0.2% | -0.2% | -0.1% | -0.1% | 0.0% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | -0.4% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.3% | 0.2% | -0.8% | 0.0% | -0.3% | -0.7% |
| Overall | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
|  | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Test 1: Adaptive merge list size

In this test, HM4.0 + ADA_MERGE_LIST_SIZE is compared with HM4.0. Table 2 shows proposed adaptive merge list size shows average 0.08% BD rate loss for six inter configuration with 96.9% encoding time. Especially -0.1% BD rate gain is achieved for Low delay P configuration with around 96% encoding time.

Table 2. Adaptive merge list size (anc: HM4.0)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
| Class B | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class C | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% |
| Class D | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.0% | 0.0% | 0.3% | 0.2% | 0.1% |
| Class C | 0.2% | 0.2% | 0.3% | 0.2% | 0.4% | 0.0% |
| Class D | 0.2% | 0.6% | -0.2% | 0.2% | 0.0% | -0.1% |
| Class E | 0.1% | 0.3% | 0.9% | 0.3% | 0.1% | 0.3% |
| Overall | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% |
|  | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | -0.1% |
| Class C | -0.1% | 0.1% | 0.0% | -0.1% | -0.2% | -0.2% |
| Class D | -0.1% | 0.1% | -0.4% | -0.1% | -0.1% | -0.4% |
| Class E | -0.1% | -0.1% | -0.4% | -0.2% | -0.7% | -0.3% |
| Overall | -0.1% | 0.1% | -0.1% | -0.1% | -0.2% | -0.2% |
|  | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% | -0.3% |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Test3: Additional merge candidates

In this test, HM4.0 + MERGE_ENC_FIX is used as anchor. Table 3 shows Inserting additional merge candidates shows average -0.17% BD rate saving for six inter configurations based on MERGE_ENC_FIX with 102.8% encoding time.

Table 3. Additional merge candidates (anc: HM4.0+MERGE_ENC_FIX)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.4% |
| Class B | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | -0.1% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class D | -0.2% | 0.1% | 0.2% | -0.2% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% |
|  | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.0% | -0.2% | 0.2% | 0.1% |
| Class C | -0.1% | -0.2% | -0.1% | -0.1% | 0.2% | 0.2% |
| Class D | -0.2% | 0.5% | 0.3% | -0.3% | -0.2% | -0.1% |
| Class E | 0.2% | 0.8% | 0.0% | 0.1% | 1.5% | 0.3% |
| Overall | -0.1% | 0.3% | 0.0% | -0.1% | 0.4% | 0.1% |
|  | -0.1% | 0.3% | 0.0% | -0.1% | 0.3% | 0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.4% | 0.4% | -0.4% | 0.5% | 0.2% |
| Class C | -0.2% | 0.1% | 0.0% | -0.3% | 0.0% | -0.1% |
| Class D | -0.4% | 0.0% | -0.2% | -0.7% | -0.6% | -0.7% |
| Class E | 0.1% | 0.2% | 0.5% | -0.2% | 1.1% | -0.1% |
| Overall | -0.2% | 0.2% | 0.2% | -0.4% | 0.2% | -0.2% |
|  | -0.2% | 0.2% | 0.2% | -0.4% | 0.2% | -0.2% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

## Test5: combined test1 and test3

In this test, test1 and test3 is jointly tested to check the synergy between two methods. This test is marked as Test5 in CE13. Table4 shows average 0.03% BD rate loss for six inter configuration with 99.2% encoding time

Table 4. Test1+Test3 (anc: HM4.0+MERGE_ENC_FIX)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -0.1% | 0.1% | 0.1% | -0.1% |
| Class B | 0.1% | 0.2% | 0.3% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | 0.1% | 0.3% | 0.1% | 0.1% | 0.2% |
| Class D | 0.0% | 0.1% | 0.4% | 0.0% | -0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.5% | 0.2% | 0.2% | 0.3% | 0.4% |
| Class C | 0.2% | 0.1% | 0.2% | 0.2% | 0.4% | 0.4% |
| Class D | 0.1% | 0.4% | 0.4% | 0.0% | 0.1% | 0.4% |
| Class E | 0.5% | 0.4% | 0.2% | 0.6% | 1.8% | 0.5% |
| Overall | 0.2% | 0.4% | 0.3% | 0.2% | 0.5% | 0.4% |
|  | 0.2% | 0.3% | 0.3% | 0.2% | 0.5% | 0.4% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.4% | 0.4% | -0.3% | 0.3% | 0.1% |
| Class C | -0.1% | 0.0% | -0.1% | -0.2% | 0.2% | 0.1% |
| Class D | -0.3% | 0.2% | -0.2% | -0.5% | -0.2% | -0.3% |
| Class E | 0.0% | 0.7% | -0.3% | 0.0% | 1.2% | 0.1% |
| Overall | -0.1% | 0.3% | 0.0% | -0.3% | 0.3% | 0.0% |
|  | -0.1% | 0.3% | 0.0% | -0.3% | 0.3% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# Conclusion

Test0 is encoder-side fix with average -0.11% BD rate saving without enc/dec time change and so it is proposed to be adopted as bugfix.

Test1 gives the flexibility of merge list size to control the trade-off of complexity and performance. This conception even can be adopted with setting the default value as 5 in common test configuration without performance change. Or we can set it as 3 in LC configuration and 5 in HE configuration. Our test1 is showing the possibility of optimized encoder method to minimize performance loss with complexity reduction.

Test3 shows that performance can be improved by adding offsets to the existing candidate. Even though the offset value 1 is used in this proposal, this performance can be improved further with optimal offset setting by joint consideration with other proposal [1].

# References

- T. Lee, et al, “Non-CE13: Additional merge candidates with MV dependent offsets”, Document of Joint Collaborative Team on Video Coding, JCTVC-G787, November 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

Proposed working draft text is included below using the following text conventions:

- Removed text from JCTVC-F803_d5 is shown with strikethrough.

- Newly added text is shown in red.

7.3.3  Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( slice_type != I) { |  |
| diff_ max_num_merge_cand | u(v) |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |

7.4.3  Slice header semantics

diff_ max_num_merge_cand specifies the difference between 5 and the maximum number of merge candidates. diff_max_num_merge_cand shall be in the range of 0 to 4. The value of maxNumMergeCand is specified by 5 - diff_max_num_merge_cand

8.4.2.1.1  Derivation process for luma motion vectors for merge mode

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to 5.

- When slice_type is equal to B, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- The derivation process for offset added motion vector merging candidates specified in subclause Error: Reference source not foundx is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0offsetCandn and refIdxL1offsetCandn, the prediction list utilization flags predFlagL0offsetCandn and predFlagL1offsetCandn, the motion vectors mvL0offsetCandn and mvL1offsetCandn of every new candidate offsetCandn being added in mergeCandList. The number of candidates being added numOffsetMergeCand is set equal to ( numMergeCand – numOffsetMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numOffsetMergeCand is greater than 0, n ranges from 0 to numOffsetMergeCand – 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-1)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-2)

refIdxLX = refIdxLXN	(8-3)

predFlagLX = predFlagLXN	(8-4)

##### 8.4.2.1.6 Derivation process for zero motion vector merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0zeroCandm and refIdxL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0zeroCandm and predFlagL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- motion vectors mvL0zeroCandm and mvL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

The variable numRefIdx is derived as follows.

- If slice_type is equal to P, numRefIdx is set to num_ref_idx_l0_active_minus1 + 11

- Otherwise (slice_type is equal to B), numRefIdx is set to min(num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1)1

When numMergeCand is less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables zeroIdx and zeroCnt are set to 0, the variable zeroStop is set to false and the following steps are repeated until zeroStop is equal to true.

- For the derivation of the reference indices, the prediction list utilization flags and the motion vectors of the zero motion vector merging candidate, the following applies.

    - If all of the following conditions are true,

      - slice_type is equal to P

      - no candidate in mergeCandList has a list0 motion vector equal to (0,0) and reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0),

the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-5)

refIdxL1zeroCandm = −1	(8-6)

predFlagL0zeroCandm = 1	(8-7)

predFlagL1zeroCandm = 0	(8-8)

mvL0zeroCandm[ 0 ] = 0	(8-9)

mvL0zeroCandm[ 1 ] = 0	(8-10)

mvL1zeroCandm[ 0 ] = 0	(8-11)

mvL1zeroCandm[ 1 ] = 0	(8-12)

numMergeCand = numMergeCand + 1	(8-13)

    - Otherwise if all of the following conditions are true,

      - slice_type is equal to B

      - no candidate in mergeCandList has a list0 motion vector equal to (0,0), a list0 reference index equal to zeroIdx, a list1 motion vector equal to (0,0) and a list1 reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0)

the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-14)

refIdxL1zeroCandm = zeroIdx	(8-15)

predFlagL0zeroCandm = 1	(8-16)

predFlagL1zeroCandm = 1	(8-17)

mvL0zeroCandm[ 0 ] = 0	(8-18)

mvL0zeroCandm[ 1 ] = 0	(8-19)

mvL1zeroCandm[ 0 ] = 0	(8-20)

mvL1zeroCandm[ 1 ] = 0	(8-21)

numMergeCand = numMergeCand + 1	(8-22)

- The variables zeroCnt and zeroIdx are modifed as follows.

    - If zeroIdx is equal to numRefIdx or zeroCnt is equal to 2, zeroIdx is set to 0.

    - Otherwise (zeroIdx is less than and zeroCnt is less than 2), zeroIdx and zeroCnt are incremented by 1.

- When numMergeCand is equal to maxNumMergeCand, zeroStop is set to true.

##### 8.4.2.1.x Derivation process for offset added motion vector merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0offsetCandl and refIdxL10offsetCandl of every new candidate offsetCandl being added in mergeCandList during the invocation of this process,

- prediction list utilization flags predFlagL0offsetCandl and predFlagL10offsetCandl of every new candidate offsetCandl being added in mergeCandList during the invocation of this process,

- motion vectors mvL0offsetCandl and mvL10offsetCandl of every new candidate offsetCandl being added in mergeCandList during the invocation of this process,

When numMergeCand is less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variable offsetStop is set to false and the following steps are repeated until offsetStop is equal to true.

- The following assignment is made with origCand being the candidate at position 0 in the merging candidate list mergeCandList ( origCand = mergeCandList[ 0 ] ).

- If slice_type is equal to B, the candidate offsetCandn with n equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = offsetCandn ) and the reference indices, the prediction list utilization flags and the motion vectors of offsetCandn are dervied as follows and numMergeCand is incremented by 1 when it is added.

refIdxL0offsetCandn = refIdxL0origCand	(8-23)

refIdxL1offsetCandn = refIdxL1origCand	(8-24)

predFlagL0offsetCandn = 1	(8-25)

predFlagL1offsetCandn = 1	(8-26)

If the current picture lies between two reference pictures, the followings are added.

mvL0offsetCandn [ 0 ] = mvL0origCand [0]+1	(8-27)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]	(8-28)

mvL1offsetCandn [ 0 ] = mvL1origCand [0]-1	(8-29)

mvL1offsetCandn [ 1 ] = mvL1origCand [1]	(8-30)

and

mvL0offsetCandn [ 0 ] = mvL0origCand [0]-1	(8-31)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]	(8-32)

mvL1offsetCandn [ 0 ] = mvL1origCand [0]+1	(8-33)

mvL1offsetCandn [ 1 ] = mvL1origCand [1]	(8-34)

and

mvL0offsetCandn [ 0 ] = mvL0origCand [0]	(8-35)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]+1	(8-36)

mvL1offsetCandn [ 0 ] = mvL1origCand [0]	(8-37)

mvL1offsetCandn [ 1 ] = mvL1origCand [1]-1	(8-38)

and

mvL0offsetCandn [ 0 ] = mvL0origCand [0]	(8-39)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]-1	(8-40)

mvL1offsetCandn [ 0 ] = mvL1origCand [0]	(8-41)

mvL1offsetCandn [ 1 ] = mvL1origCand [1]+1	(8-42)

Otherwise (if the current picture lies in the same direction from two reference pictures), the followings are added.

mvL0offsetCandn [ 0 ] = mvL0origCand [0]+1	(8-43)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]	(8-44)

mvL1offsetCandn [ 0 ] = mvL1origCand [0]+1	(8-45)

mvL1offsetCandn [ 1 ] = mvL1origCand [1]	(8-46)

and

mvL0offsetCandn [ 0 ] = mvL0origCand [0]-1	(8-47)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]	(8-48)

mvL1offsetCandn [ 0 ] = mvL1origCand [0]-1	(8-49)

mvL1offsetCandn [ 1 ] = mvL1origCand [1]	(8-50)

and

mvL0offsetCandn [ 0 ] = mvL0origCand [0]	(8-51)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]+1	(8-52)

mvL1offsetCandn [ 0 ] = mvL1origCand [0]	(8-53)

mvL1offsetCandn [ 1 ] = mvL1origCand [1]+1	(8-54)

and

mvL0offsetCandn [ 0 ] = mvL0origCand [0]	(8-55)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]-1	(8-56)

mvL1offsetCandn [ 0 ] = mvL1origCand [0]	(8-57)

mvL1offsetCandn [ 1 ] = mvL1origCand [1]-1	(8-58)

- If slice_type is equal to P, the candidate offsetCandn with n equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = offsetCandn ) and the reference indices, the prediction list utilization flags and the motion vectors of offsetCandn are dervied as follows and numMergeCand is incremented by 1 when it is added.

refIdxL0offsetCandn = refIdxL0origCand	(8-59)

refIdxL1offsetCandn = refIdxL1origCand	(8-60)

predFlagL0offsetCandn = 1	(8-61)

predFlagL1offsetCandn = 0	(8-62)

mvL0offsetCandn [ 0 ] = mvL0origCand [0]+1	(8-63)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]	(8-64)

mvL1offsetCandn [ 0 ] = 0	(8-65)

mvL1offsetCandn [ 1 ] = 0	(8-66)

and

mvL0offsetCandn [ 0 ] = mvL0origCand [0]-1	(8-67)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]	(8-68)

mvL1offsetCandn [ 0 ] = 0	(8-69)

mvL1offsetCandn [ 1 ] = 0	(8-70)

and

mvL0offsetCandn [ 0 ] = mvL0origCand [0]	(8-71)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]+1	(8-72)

mvL1offsetCandn [ 0 ] = 0	(8-73)

mvL1offsetCandn [ 1 ] = 0	(8-74)

and

mvL0offsetCandn [ 0 ] = mvL0origCand [0]	(8-75)

mvL0offsetCandn [ 1 ] = mvL0origCand [1]-1	(8-76)

mvL1offsetCandn [ 0 ] = 0	(8-77)

mvL1offsetCandn [ 1 ] = 0	(8-78)

- If several merging candidates have the motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList and numMergeCand is decremented by the number of candidates which are removed.

- When numMergeCand is equal to maxNumMergeCand, offsetStop is set to true.