| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G781 |
| --- | --- |

| Title: | Reduced chroma contexts for significance map coding in CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Junghye Min Elena Alshina JeongHoon Park | Email: | yj1003.piao@samsung.com jh643.min@samsung.com elena_a.alshina@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document proposes a modified significance map coding for chroma with TU size of 4x4 and 8x8. Rarely used contexts in high frequency domain were removed to optimize memory cost for contexts. This contribution reduces the number of contexts in chroma from 31 to 14 for 4x4 and 8x8 transform unit without any performance drop in high efficiency configurations. The average BD-rate is 0.01% for AI, 0.01% for RA and 0.03% for LD respectively.

# Introduction

In current HM4.0, a total of 62 contexts are used for significance map coding of TU size 4x4 and 8x8: 15 for 4x4 luma, 15 for 4x4 chroma, 16 for 8x8 luma and 16 for 8x8 chroma. The context derivation is same in luma and chroma (Fig 1.a). However, compared with luma case, relatively less number of non-zero coefficients exist in chroma transform coefficients and they are usually highly concentrated in the low frequency area. Motivated by this observation, this contribution proposes to remove some rarely used contexts of chroma to save contexts and reduce memory cost.

- HM4.0                               (b) Proposed

Fig 1. Contexts for significance map coding of T4x4 and T8x8 in HM4.0 and proposed

Followings are the changes of this contribution:

- 1 context for region pos_x + pos_y >2 in T4x4 chroma

- 1 context for region pos_x>>1 + pos_y>>1 >2 in T8x8 chroma

With above two simple modifications, number of context for chroma significance map coding of TU size 4x4 and 8x8 is reduced by 55%.

# Results

The test was implemented on top of HM4.0 s/w under common test condition described in JCTVC-F900 [1]. Summarized results are given in Table I.

Table I. Test results in HM4.0.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | 0.3% |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  | 0.1% | -0.3% | 0.2% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |

# Conclusions

In this document, 17 (55%) contexts for chroma significance map coding of TU size 4x4 and 8x8 were removed without performance change. We recommend our proposal to be adopted as a part of the next HM.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text for Working Draft

###### 9.3.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size log2TrafoSize.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3, sigCtx is derived as follows.

shift  	=  log2TrafoSize  = =  3 ? 1 : 0
sigCtx  =  ( shift  *  15 )  +  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )	(9-1)

If (cIdx is greater than 0)

if  ((log2TrafoSize is 2)&&( xC+ yC>2))

sigCtx  =  6

Otherwise (log2TrafoSize is 3),

If  ((xC>>1)+ (yC>>1)>2)

sigCtx  =  7+6

- Otherwise if xC + yC is less than 2, sigCtx is derived as follows.

sigCtx  =  31  +  ( yC  <<  1 )  +  xC	 (9-2)

- Otherwise if xC + yC is less than 5, sigCtx is derived as follows.

temp  =  significant_coeff_flag[ xC + 1 ][ yC ]  +  significant_coeff_flag[ xC + 2 ][ yC ]  +  
		significant_coeff_flag[ xC ][ yC + 1 ]  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]  +  
		significant_coeff_flag[ xC ][ yC + 2 ]	(9-3)
sigCtx  =  34  +  Min( 4, temp )

- Otherwise (xC  +  yC is greater than 4), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  39

…………………………..