| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G783 |
| --- | --- |

| Title: | Context number reduction for level coding in CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Junghye Min JeongHoon Park | Email: | yj1003.piao@samsung.com jh643.min@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents two new context modeling methods for level coding in CABAC. The context sets were differentiated in luma and chroma to remove rarely used contexts. With the first context modeling method, the number of context for level coding was reduced by 33% with no performance drop. And with the second context modeling method, the number of context was reduced by 50% with negligible loss in BD-rate, 0.07% in AI, 0.04% in RA and 0.0 % in LD configurations respectively.

# Introduction

In current HM4.0, each non-zero coefficient is coded in a sub-set unit and represented by four symbols: abs_level_greater1_flag, abs_level_greater2_flag, abs_level_minus3 and sign_flag. Each sub-set consists of 16 coefficients in reverse scan order from the last non-zero coefficient [1]. The context derivation for abs_level_greater1_flag and abs_level_greater2_flag depends on the statistic of larger than non-zero coefficients in previous sub-set and partial statistic of current sub-set as shown in table I. A total of 120 contexts are used for level coding when both luma and chroma components are taken into account: 60 contexts for coeff_abs_level_greater1_flag and 60 contexts for coeff_abs_level_greater2_flag.

Table I. Context set and context for level coding in HM4.0 [1]

| Context Set (inverse 16 scan) |  |  |
| --- | --- | --- |
| 0 | Sub-set 0 | 0 LargerT1 in previous sub-set |
| 1 |  | 1-3 LargerT1 in previous sub-set |
| 2 |  | >3 LargerT1 in previous sub-set |
| 3 | Other sub-sets | 0 LargerT1 in previous sub-set |
| 4 |  | 1-3 LargerT1 in previous sub-set |
| 5 |  | >3 LargerT1 in previous sub-set |

|  | Model bin 1 |  |  | Model bin 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 or more larger than 1 |  | 0 | Initial - no larger than one |
|  | 1 | Initial - no trailing ones |  | 1 | 1 LargerT1 |
|  | 2 | 1 trailing one |  | 2 | 2 LargerT1 |
|  | 3 | 2 trailing ones |  | 3 | 3 LargerT1 |
|  | 4 | 3 or more trailing ones |  | 4 | 4 or more LargerT1 |

In this contribution, two different modeling methods of context set are provided. In both cases, the context sets of luma and chroma were designed differently since each color component has different coefficient distribution. Coefficients in chroma component usually show more concentrated distribution than those of luma. Therefore separation of the first sub-set from others is not necessary for chroma case.

CONTEXT_SET1:

| Context Set_Luma |  |  |  | Context Set_Chroma |  |
| --- | --- | --- | --- | --- | --- |
| 0 | Sub-set 0 | 0 LargerT1 in previous sub-set |  | 0 | no LargerT1 in previous sub-set |
| 1 |  | 1-3 LargerT1 in previous sub-set |  | 1 | at least 1 LargerT1 in previous sub-set |
| 2 |  | >3 LargerT1 in previous sub-set |  |  |  |
| 3 | Other sub-sets | 0 LargerT1 in previous sub-set |  |  |  |
| 4 |  | 1-3 LargerT1 in previous sub-set |  |  |  |
| 5 |  | >3 LargerT1 in previous sub-set |  |  |  |

CONTEXT_SET2:

| Context Set_Luma |  |  |  | Context Set_Chroma |  |
| --- | --- | --- | --- | --- | --- |
| 0 | Sub-set 0 | no LargerT1 in previous sub-set |  | 0 | no LargerT1 in previous sub-set |
| 1 |  | at least 1 LargerT1 in previous sub-set |  | 1 | at least 1 LargerT1 in previous sub-set |
| 2 | Other sub-sets | no LargerT1 in previous sub-set |  |  |  |
| 3 |  | at least 1 LargerT1 in previous sub-set |  |  |  |

In the first modeling method (CONTEXT_SET1), only chroma context set was modified and 40 contexts were removed. In the second modeling method (CONTEXT_SET2), both luma and chroma context sets were modified and 60 contexts can be saved by this modification.

# Results

The tests for two context sets were implemented on top of HM4.0 s/w under common test condition described in JCTVC-F900 [2]. Summarized results are given in Table II and Table III.

Table II. Test results CONTEXT_SET1 in HM4.0.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.1 | 0.0 | 0.2 | 0.1 |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 |  |  |  | 0.1 | 0.0 | 0.3 | 0.0 | -0.1 | -0.1 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  | 99% |  |  |

Table III. Test results CONTEXT_SET2 in HM4.0.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2 | 0.2 | 0.2 | 0.0 | 0.0 | -0.1 |  |  |  |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.2 | 0.0 | -0.1 | 0.3 |
| Class C | 0.0 | 0.1 | 0.1 | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | -0.3 |
| Class E | 0.1 | 0.1 | 0.1 |  |  |  | 0.0 | 0.1 | -0.4 | 0.0 | 0.2 | -0.9 |
| Overall | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | -0.1 |
|  | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | -0.2 |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 99% |  |  | 100% |  |  |

# Conclusions

This document presented two context modeling methods to reduce the number of contexts used in level coding. According to the provided results, the number of contexts for level coding was reduced by 33% and 50% in two test cases with negligible performance drop. We recommend one of context sets to be adopted as a part of the next HM.

# References

- JCT-VC, “CE11: Unified scans for the significance map and coefficient level coding in high efficiency”, JCTVC-F288, 6th JCT-VC Meeting, Torino, IT, 14-22 July, 2011

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text for Working Draft

###### 9.3.3.1.1.5 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

Inputs to this process are the color component index cIdx, the 16 coefficient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntax elements coeff_abs_level_greater1_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

  - If the current subset index i is equal to 0, the following applies.

ctxSet  =  0		(9-1)

  - Otherwise (i is greater than 0), the following applies.

ctxSet  =  3		(9-2)

    - If  CONTEXT_SET2

ctxSet  =  2

- If cIdx is greater than 0

ctxSet  =  0

- When the subset i is not the first one to be processed in this subclause, the following applies.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of subclause  for the syntax element coeff_abs_level_greater2_flag for the subset i + 1.

- When ( numGreater1  >>  1 ) is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-3)

- When ( numGreater1  >>  1 ) is greater than 3, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet   +  1		(9-4)

- The variable greater1Ctx is set equal to 1.

- Otherwise (coeff_abs_level_greater1_flag[ n ] is not the first to be parsed within the current subset i),.for the derivation of ctxSet and greater1Ctx the following applies.

- The variable ctxSet is set equal to the variable ctxSet that has been derived during the last invocation of this subclause.

- The variable greater1Ctx is set equal to the variable greater1Ctx that has been derived during the last invocation of this subclause.

- When greater1Ctx is greater than 0, the variable lastGreater1Flag is set equal to the syntax element coeff_abs_level_greater1_flag that has been used during the last invocation of this subclause and greater1Ctx is modifed as follows.

- If lastGreater1Flag is equal to 1, greater1Ctx is set equal to 0.

- Otherwise (lastGreater1Flag is equal to 0), greater1Ctx is incremented by 1.

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  5 )  +  Min( 4, greater1Ctx )	(9-5)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

- If  CONTEXT_SET1

ctxIdxInc =  ctxIdxInc  +  20

- Otherwise

ctxIdxInc =  ctxIdxInc  +  30		(9-6)

###### 9.3.3.1.1.6 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

Inputs to this process are the color component index cIdx, the 16 coeffcient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 15 or all previous syntex elements coeff_abs_level_greater2_flag[ pos ] with pos greater than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- The variable ctxSet is initialized as follows.

- If the current subset index i is equal to 0, the following applies.

ctxSet  =  0		(9-7)

- Otherwise (i is greater than 0), the following applies.

ctxSet  =  3		(9-8)

    - If  CONTEXT_SET2

ctxSet  =  2

- If cIdx is greater than 0

ctxSet  =  0

- The variable numGreater1 is set equal to 0.

- When the subset i is not the first one to be processed in this subclause, the following applies.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of this subclause for the subset i + 1 and is modifed as follows.

numGreater1  =  numGreater1  >>  1		(9-9)

- When numGreater1 is greater than 0, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet  +  1		(9-10)

- When numGreater1 is greater than 3, ctxSet is incremented by one as follows.

ctxSet  =  ctxSet   +  1		(9-11)

- The variable greater2Ctx is set equal to 0.

- Otherwise (coeff_abs_level_greater2_flag[ n ] is not the first to be parsed within the current subset i), for the derivation of ctxSet and greater2Ctx the following applies.

- The variable ctxSet is set equal to the variable ctxSet that has been derived during the last invocation of this subclause.

- The variable greater2Ctx is set equal to the variable greater2Ctx that has been derived during the last invocation of this subclause incremented by 1.

- The variable numGreater1 is set equal to the variable numGreater1 that has been derived during the last invocation of this subclause incremented by 1.

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater2Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  5 )  +  Min( 4, greater2Ctx )	(9-12)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

- If  CONTEXT_SET1

ctxIdxInc =  ctxIdxInc  +  20

- Otherwise

ctxIdxInc =  ctxIdxInc  +  30		                                      (9-13)