| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G785 |
| --- | --- |

| Title: | Unified Pred_type coding in CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Junghye Min JeongHoon Park | Email: | yj1003.piao@samsung.com jh643.min@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document proposes to unify pred_type coding of slice B and P in CABAC. Prediction mode and partition mode coding in slice B is modified to be same as that of slice P to simplify the pred_type coding and enhance the readability of specification. The average BD-rate by this unification is 0.01% in RA and 0.03% in LD high efficiency configuration. With bug fix reported in JCTVC-G655, the average BD-rate is 0.xx% in RA and 0.xx% in LD.

# Introduction

Current prediction mode and partition mode are coded in a different way in slice B and slice P. Unlike the independent coding of prediction mode and partition mode in slice P, they are jointly coded in slice B where higher priority is given to inter prediction mode. Therefore more condition check and contexts are needed in case of slice B, which is inconsistent with slice P. This contribution simply unified the prediction mode and partition mode coding of slice B with that of slice P to make current pred_type coding much simpler.

TableI. Unified pred_type coding in CABAC

| Slice type | Value of pred_type | PredMode | PartMode | Bin string |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cLog2CUSize > Log2MinCUSize | cLog2CUSize = = Log2MinCUSize |  |
|  |  |  |  |  | cLog2CUSize = = 3 && !inter_4x4_enabled_flag | cLog2CUSize > 3 ¦ ¦ inter_4x4_enabled_flag |
| I | 0 | MODE_INTRA | PART_2Nx2N | - | 1 | 1 |
|  | 1 | MODE_INTRA | PART_NxN | - | 0 | 0 |
| P/B | 0 | MODE_INTER | PART_2Nx2N | 0 1 | 0 1 | 0 1 |
|  | 1 | MODE_INTER | PART_2NxN | 0 011 | 0 01 | 0 01 |
|  | 2 | MODE_INTER | PART_Nx2N | 0 001 | 0 00 | 0 001 |
|  | 4 | MODE_INTER | PART_2NxnU | 0 0100 | - | - |
|  | 5 | MODE_INTER | PART_2NxnD | 0 0101 | - | - |
|  | 6 | MODE_INTER | PART_nLx2N | 0 0000 | - | - |
|  | 7 | MODE_INTER | PART_nRx2N | 0 0001 | - | - |
|  | 3 | MODE_INTER | PART_NxN | - | - | 0 000 |
|  | 4 | MODE_INTRA | PART_2Nx2N | 1 | 11 | 11 |
|  | 5 | MODE_INTRA | PART_NxN | - | 10 | 10 |

# Results

The test is done under common test condition described in JCTVC-F900 [1]. As shown in summarized result in Table II, no significant change is observed. The result with bug fix reported in JCTVC-G655 is also shown in Table III.

Table II. Test result of unified pred_type coding in HM4.0

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.2% |
| Class E |  |  |  | 0.0% | 0.3% | 0.7% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100.0% |  |  | 99.8% |  |  |
| Dec Time[%] | 100.1% |  |  | 99.9% |  |  |

Table III. Test result of unified pred_type coding with bug fix in HM4.0

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% |
| Class E |  |  |  | 0.1% | 0.2% | 0.9% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution presented the result when pred_type coding of slice B and P was unified. We recommend unification of pred_type coding into next HM.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- “Cross-check of JCTVC-G151 on prediction and partition mode binarization for Low Delay P”, JCTVC-G655, 7th JCT-VC Meeting, Geneva, Switzerland, Nov. 2011,

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text for Working Draft

#### 9.3.2.7 Binarization process for pred_type

Table 9-1 – Binarization for pred_type

| Slice type | Value of pred_type | PredMode | PartMode | Bin string |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | cLog2CUSize > Log2MinCUSize | cLog2CUSize = = Log2MinCUSize |  |
|  |  |  |  |  | cLog2CUSize = = 3 && !inter_4x4_enabled_flag | cLog2CUSize > 3 ¦ ¦ inter_4x4_enabled_flag |
| I | 0 | MODE_INTRA | PART_2Nx2N | - | 1 | 1 |
|  | 1 | MODE_INTRA | PART_NxN | - | 0 | 0 |
| P/B | 0 | MODE_INTER | PART_2Nx2N | 0 1 | 0 1 | 0 1 |
|  | 1 | MODE_INTER | PART_2NxN | 0 011 | 0 01 | 0 01 |
|  | 2 | MODE_INTER | PART_Nx2N | 0 001 | 0 00 | 0 001 |
|  | 4 | MODE_INTER | PART_2NxnU | 0 0100 | - | - |
|  | 5 | MODE_INTER | PART_2NxnD | 0 0101 | - | - |
|  | 6 | MODE_INTER | PART_nLx2N | 0 0000 | - | - |
|  | 7 | MODE_INTER | PART_nRx2N | 0 0001 | - | - |
|  | 3 | MODE_INTER | PART_NxN | - | - | 0 000 |
|  | 4 | MODE_INTRA | PART_2Nx2N | 1 | 11 | 11 |
|  | 5 | MODE_INTRA | PART_NxN | - | 10 | 10 |
| B | 0 | MODE_INTER | PART_2Nx2N | 1 | 1 | 1 |
|  | 1 | MODE_INTER | PART_2NxN | 011 | 01 | 01 |
|  | 2 | MODE_INTER | PART_Nx2N | 001 | 001 | 001 |
|  | 4 | MODE_INTER | PART_2NxnU | 0100 | - | - |
|  | 5 | MODE_INTER | PART_2NxnD | 0101 | - | - |
|  | 6 | MODE_INTER | PART_nLx2N | 0000 | - | - |
|  | 7 | MODE_INTER | PART_nRx2N | 0001 | - | - |
|  | 3 | MODE_INTER | PART_NxN | - | - | 0001 |
|  | 4 | MODE_INTRA | PART_2Nx2N | 000 | 000 0 | 0000 0 |
|  | 5 | MODE_INTRA | PART_NxN | - | 000 1 | 0000 1 |