| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G787 |
| --- | --- |

| Title: | Non-CE13: Additional merge candidates with MV dependent offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park Guillaume Laroche Patrice Onno Jian-Liang Lin Yi-Wen Chen Yu-Wen Huang Shawmin Lei | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr jl.lin@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | Samsung, Canon, MediaTek |  |  |

_____________________________

# Abstract

This contribution presents additional Merge/AMVP candidates to compensate empty positions in the fixed length candidates list. The additional Merge/AMVP candidates are produced by adding offsets to the first existing candidate where the offset value is chosen according to the first candidate’s MV value. Relative to the HM4.0+MRG_ENC_FIX which is reported in JCTVC-G776, experimental results showed -0.3% BR savings in average of results in RA-HE, RA-LC, LB-HE, LB-LC, LP-HE, LP-LC.

# Introduction

In the last meeting, the fixed list size of Merge/AMVP candidates is adopted with additional Merge/AMVP candidates to decrease coding loss by it. But there is still chance to be empty in candidates list if some motion information doesn’t exist or duplicated. Moreover, adopted additional merge candidates in HM4.0 are only applied in B Slice except zero predictors.

In this proposal, additional Merge/AMVP candidates are introduced to increase performance and which also work in P Slice.

# Description

## MV dependent offset

In CE13, there are three tests in [1], [1] and [2] that use additional candidates by adding offsets to the first existing candidate’s each mv component in turn. In those tests, the offset value is fixed by 1, 4, or 8 with sign changes in turn. This offset can be regarded as MVD in Merge mode, but used as a new merge candidate without mvd signaling as it was. Since motion prediction is the harder and MVD tends to be various when motion is the larger, the offset value needs to be adjusted in that way.

This proposal changes the constant offset value to non-constant offset value which is dependent on mv value of the first candidate. The offset value v to be added to one mv component m is determined as follows:

If the magnitude of m is larger than 16, v is set to 8

Else if it is larger than 8,            v is set to 4

Else if it is larger than 4,            v is set to 2

Otherwise,                       v is set to 1

As in [1], the maximum 4 new merge candidates are introduced and those are compared to other candidates to exclude duplicates. Since 4 new candidates are always distinct each other, the number of comparison for duplicates checking can be reduced and it requires 9 comparisons in the worst case, where 3 (except the first one) of 4 existing candidates are compared with 3 new candidates and the last new candidate is inserted without comparison if 3 new candidates are the same as 3 existing candidates.

## Rounding candidate

In CE13 section 3.1 [2]a rounding candidate is proposed by truncating the first existing candidate’s MV from list 0 or list 1 to integer precision. In this proposal, the rounding candidate is proposed by truncating the first existing candidate’s MVs from both list 0 and list 1 and is added before the 4 new offset based candidates. Since this rounding candidate is simple and does not require the interpolation process, the coding time can be reduced by adding this rounding candidate.

## Fast encoding algorithm

This section presents an encoder simplification in order to reduce the encoding time induced by the addition of new candidates in the AMVP and Merge/Skip list. As we can observe in section 2.4, the encoding time has increased in our tests. This is mainly due to the rate-distortion cost estimation of the additional candidates for the Merge mode. Indeed, for each candidate of the Merge candidate, the rate distortion cost estimation is computed by coding the residual data and by checking a quad-tree of transforms. This computation is quite costly compared to the Skip mode where only a predictor index is coded.

In the following section2.3.2, we propose an early termination rule when estimating the rate distortion cost of the 2Nx2N Merge candidates. This avoids estimating all the Merge candidates with a very little impact on the coding efficiency (see Table 4)

### HM4.0 rate-distortion estimation for the Skip/Merge candidates.

Figure 1 presents the current encoder algorithm to estimate the rate distortion cost for each Merge/Skip candidate in the HM4.0. This diagram is very simple and shows that all predictors are exhaustively tested through the Skip and the Merge mode. Please note that for the Merge mode, the rate-distortion cost computation is more important since the residual data is encoded by checking different transforms

Figure 1: HM4.0 RD cost estimation procedure for Skip/Merge candidates

### Proposed modification for the Merge mode rate distortion computation (fast encoding mode)

Figure 2 presents the proposed encoder algorithm to avoid some rate distortion cost evaluations for some Merge/Skip candidates.  Instead of performing systematically the rate-distortion cost of each candidate, an early termination rule is applied. The diagram in Figure 2 uses an internal Early Termination Flag (ETF) that enables stopping the computation of the Rate distortion cost of the Merge mode as soon as this ETF is set to TRUE.

The basic rule is the following: the Early Termination Flag value is set to TRUE when one of the two following conditions is met:

the first motion vector predictor candidate is equal to (0,0)

- forfor any candidate of the list, the cost (JSkipi) of the Skip mode is inferior to the cost (JMrgi) of the Merge mode.

Then when the second predictor is under processing and when the ETF is equal to true, we do not need to estimate the rate distortion cost of the Merge mode.

Figure 2: Proposed RD cost estimation procedure for Skip/Merge candidates to save encoding time.

## Experimental result

## MVDep results

A proposed method was implemented on HM4.0+MRG_ENC_FIX software which is proposed in [1]. Table 1 shows its RD performance and execution time of the software relative to HM4.0+MRG_ENC_FIX. In random access cases, proposed method saves 0.2% bits for high efficiency and 0.2% bits low complexity on average. In low delay B case, proposed method saves 0.1% bits for high efficiency and 0.2% bits for low complexity on average. On the other hand, in low delay P case, proposed method saves 0.3% bits for high efficiency and 0.7% bits for low complexity on average. Due to absence of additional candidates for P slice, performance in low delay P case is relatively high.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.1% | -0.1% | -0.2% | -0.4% |
| Class B | -0.2% | 0.0% | 0.0% | -0.3% | -0.1% | -0.2% |
| Class C | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.3% |
| Class D | -0.2% | 0.0% | -0.1% | -0.3% | -0.2% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% | -0.3% |
|  | -0.2% | -0.1% | 0.0% | -0.3% | -0.2% | -0.3% |
| Enc Time[%] | 103.4% |  |  | 103.7% |  |  |
| Dec Time[%] | 100.0% |  |  | 100.2% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.4% | 0.1% | -0.3% | 0.1% | -0.2% |
| Class C | -0.3% | -0.3% | -0.3% | -0.2% | 0.2% | 0.1% |
| Class D | -0.3% | 0.3% | 0.0% | -0.3% | -0.2% | 0.2% |
| Class E | 0.4% | 0.7% | 0.4% | 0.3% | 1.1% | 0.3% |
| Overall | -0.1% | 0.3% | 0.0% | -0.2% | 0.2% | 0.1% |
|  | -0.1% | 0.3% | 0.0% | -0.2% | 0.2% | 0.1% |
| Enc Time[%] | 103.2% |  |  | 102.8% |  |  |
| Dec Time[%] | 100.3% |  |  | 99.5% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.1% | -0.1% | -0.9% | -0.2% | -0.4% |
| Class C | -0.3% | -0.2% | -0.4% | -0.6% | -0.3% | -0.5% |
| Class D | -0.6% | -0.4% | -0.5% | -0.8% | -0.5% | -0.7% |
| Class E | 0.1% | 0.7% | 1.1% | -0.4% | 1.7% | 1.1% |
| Overall | -0.3% | 0.0% | -0.1% | -0.7% | 0.1% | -0.2% |
|  | -0.3% | 0.0% | -0.1% | -0.7% | 0.0% | -0.2% |
| Enc Time[%] | 105.2% |  |  | 105.4% |  |  |
| Dec Time[%] | 101.0% |  |  | 99.8% |  |  |

Table 1. Mv dependent offset candidates (anc: HM4.0+MRG_ENC_FIX)

The encoding time is 3~5% increased because additional valid candidates need additional motion compensation in estimation. This encoding time can be reduced by fast encoding algorithm in [1] in which 2Nx2N Merge mode is not tested when Merge Skip mode is chosen as the best before testing 2Nx2N merge mode.

## MVDep results + rounding candidates

In this test, the proposed MV dependent offset candidates are combined with the rounding candidate described in section 2.2. The performance is shown in Table 2 and the average gain is 0.26%.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.2% | -0.1% | -0.1% | -0.3% |
| Class B | -0.2% | 0.0% | 0.1% | -0.3% | 0.0% | -0.2% |
| Class C | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% | -0.2% |
| Class D | -0.2% | 0.0% | -0.1% | -0.2% | -0.3% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |
|  | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.2% | 0.1% | -0.3% | 0.1% | 0.0% |
| Class C | -0.3% | -0.5% | 0.0% | -0.2% | 0.2% | 0.2% |
| Class D | -0.3% | 0.5% | 0.1% | -0.3% | -0.4% | -0.1% |
| Class E | 0.3% | 0.6% | 0.2% | 0.2% | 1.2% | 0.5% |
| Overall | -0.1% | 0.2% | 0.1% | -0.2% | 0.2% | 0.1% |
|  | -0.1% | 0.2% | 0.1% | -0.2% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.4% | 0.2% | -0.8% | 0.0% | -0.4% |
| Class C | -0.3% | -0.4% | -0.2% | -0.6% | -0.4% | -0.5% |
| Class D | -0.4% | -0.3% | -0.2% | -0.6% | -0.3% | -0.5% |
| Class E | 0.2% | 0.5% | 1.0% | -0.3% | 1.4% | 1.1% |
| Overall | -0.3% | 0.0% | 0.1% | -0.6% | 0.1% | -0.2% |
|  | -0.3% | 0.1% | 0.2% | -0.6% | 0.1% | -0.1% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2. Rounded candidate + Mv dependent offset candidates (anc: HM4.0+MRG_ENC_FIX)

  - MVDep results + fast encoding

2Table 3 shows the result of the proposed method with the fast encoding algorithm described in section2.22.3.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.3% | -0.1% | -0.2% | -0.4% |
| Class B | -0.2% | -0.1% | 0.0% | -0.3% | -0.1% | -0.3% |
| Class C | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |
| Class D | -0.2% | 0.0% | 0.1% | -0.3% | -0.2% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.3% |
|  | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.3% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.3% | 0.1% | -0.2% | 0.1% | -0.1% |
| Class C | -0.2% | -0.3% | 0.0% | -0.2% | 0.1% | 0.0% |
| Class D | -0.2% | 0.3% | 0.3% | -0.3% | -0.1% | -0.1% |
| Class E | 0.5% | 0.7% | 0.5% | 0.4% | 0.8% | 0.3% |
| Overall | -0.1% | 0.2% | 0.2% | -0.1% | 0.2% | 0.0% |
|  | -0.1% | 0.2% | 0.2% | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.0% | -0.1% | -0.8% | -0.1% | -0.2% |
| Class C | -0.3% | -0.5% | -0.5% | -0.6% | -0.4% | -0.4% |
| Class D | -0.5% | 0.0% | -0.4% | -0.7% | -0.3% | -0.5% |
| Class E | 0.2% | 0.5% | 1.6% | -0.3% | 1.7% | 1.3% |
| Overall | -0.3% | 0.0% | 0.0% | -0.6% | 0.1% | 0.0% |
|  | -0.3% | 0.1% | 0.0% | -0.6% | 0.1% | 0.0% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 3. MV dependent offset candidates with fast algorithm (anc: HM4.0+MRG_ENC_FIX)

The average gain of 6 configurations is slightly reduced from 0.29% to 0.25% by fast algorithm while encoder complexity is reduced from 103.94% to 98.5%.

## .

  - Summary

In summary of results from the above, the performance is shown in 5. The last column shows the number of additional comparisons for duplication checking in the worst case.

|  | RA_HE | RA_LC |  | LD_HE |  | LD_LC |  | LDP_HE |  | LDP_LC |  | Avg | Enc | Dec | Comp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MvDepOffset | -0.2 | -0.2 |  | -0.1 |  | -0.2 |  | -0.3 |  | -0.7 |  | -0.29 | 104 | 100 | 6 |
| MvDepOffset+Rounding | -0.1 | -0.2 |  | -0.1 |  | -0.2 |  | -0.3 |  | -0.6 |  | -0.26 | 103 | 100 | 10 |
| MvDepOffset+Fast | -0.1 | -0.2 |  | -0.1 |  | -0.1 |  | -0.3 |  | -0.6 |  | -0.25 | 98 | 99 | 6 |

Table 4. Summary of Mv dependent offset candidates results with and without the fats encoding method, as well as the rounding candidates (anc: HM4.0+MRG_ENC_FIX)

# Conclusion

By adding offsets to the existing first candidate dependently on MV component value of it if additional candidates are needed, gain can be obtained up to 0.7% in LDP_LC and 0.3% in average. Even though encoding time is increased by 4%, this involves only 6 additional comparisons (current HM4.0 contains more than 50 in the worst case) for duplicates checking in the worst case and this encoder complexity can be reduced by applying fast encoder algorithm.

Therefore, it is proposed to adopt MV dependent offset based candidates as additional candidates with possible combination of the fast encoder algorithm to reduce the encoding time.

# References

- T. Lee, J. Chen and J. H. Park, “CE13: Merge candidates list construction”, Document of Joint Collaborative Team on Video Coding, JCTVC-G776, November 2011.

- G. Laroche, T. Poirier and P. Onno, “CE13: Results of section 3.5”, Document of Joint Collaborative Team on Video Coding, JCTVC-G240, November 2011.

- J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei, “CE13: Results of section 3.1 tests 1, 3d, and 3e on replacing redundant MVPs and its combination with adaptive MVP list size,” Document of Joint Collaborative Team on Video Coding, JCTVC-G231, November 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification