| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G788_r1 |
| --- | --- |

| Title: | Proposal on Decoded Picture Buffer Description Syntax Relating to AHG21 and JCTVC-F493 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary Sullivan * Yongjun Wu * Jizheng Xu * Bin Li † | Tel: Email: | +1 (425) 703-5308 garysull@microsoft.com |
| Source: | * Microsoft Corporation, Redmond, WA 98052 USA † University of Science and Technology of China |  |  |

_____________________________

# Abstract

This contribution supports the basic concepts previously proposed in JCTVC-F493 – namely, the elimination of the frame_num syntax element and the use of explicit buffer description syntax. However, number of specific refinements of the prior proposed scheme are suggested.

# Specific proposed modifications

The JCTVC-F493 proposal is appealing, in order to try to simplify the process and syntax associated with picture buffering and provide loss robustness. In the JCTVC-F493 proposal, rather than relying on the internal state of the decoder to enable determining what prior pictures are currently available for reference in the DPB, they are listed explicitly. In this contribution, we refer to such an explicit list as the “buffer description list” (BDL, although alternative terms have been used by others).

This proposal presumes some awareness of JCTVC-F493 and the subsequent two files circulated in AHG discussion, specifically:

- ftp://ftp.hhi.de/ahg21/JCTVC-F803_d1_Buffer_Descriptions_r0.doc

- ftp://ftp.hhi.de/ahg21/JCTVC-F803_d1_Buffer_Descriptions_display_process_suggestion_r1.doc

We propose the following specific changes relative to these proposed schemes:

- A flag called the mark_unused_after_decoding_flag is proposed to be added for each picture in the BDL. An unadvertised consequence of the JCTVC-F493 proposal is that, by moving the location of the reference picture marking operation earlier in the decoding process, it reduces the number of reference pictures that can be stored in the buffer. If the DPB capacity is N pictures and the current picture is a reference picture, then there will be no room to store the current picture in the buffer when using the JCTVC-F493 proposal unless the total size of the BDL does not exceed N – 1 pictures. That is one picture less than what is possible using the AVC syntax, because in the AVC syntax, first the pictures that are in the buffer are referenced for the decoding process, then any pictures that are no longer needed are discarded (i.e. “marked as unused for reference”, which makes room in the buffer for the new picture), and then the current picture is stored in the buffer for potential use in the decoding process of subsequent pictures. By moving the marking process to before the use of the pictures as references, the number of pictures that can be referenced for a given total buffer capacity is reduced by 1. This is highly undesirable. Our proposal is instead to send a one-bit flag called the mark_unused_after_decoding_flag in the BDL for each listed picture. The marking of pictures with mark_unused_after_decoding_flag equal to 1 as “unused for reference” should be performed after the decoding process of the current picture and before the storage of the current picture in the DPB.

- Possibly, some encoders may not want to make use of the mark_unused_after_decoding_flag. To avoid sending unnecessary bits by such encoders, we propose to add a flag at the SPS level called the no_final_referencing_flag. When no_final_referencing_flag is equal to 1, the mark_unused_after_decoding_flag syntax element are proposed to be omitted from the APS level.

- At the Turin meeting, the JCT-VC decided (in response to JCTVC-F462, thinking that the decision was just a clarification of the existing intent) to prohibit allowing pictures in higher temporal layers from being able to mark pictures in lower temporal layers as “unused for reference”. This may or may not have been a wise decision for the use of the AVC-like syntax of the current HEVC design. However, we see no justification to prohibit this type of marking when using the BDL-type of syntax as proposed in JCTVC-F493 and herein. Once a picture has been omitted from a BDL, it does not appear in any subsequent BDL either, so there is no danger of the decoder missing the notification to mark the picture as “unused for reference”.

- A flag called the higher_layers_not_listed_flag is proposed to be added at the SPS level. When the flag is equal to 1, pictures of higher temporal layers are not included in the BDLs of the CVS. In the prior proposals, pictures of higher temporal layers are included in the BDLs. Such pictures are never actually referenced in the decoding process of a current picture, and this is known by the decoder. Since these pictures are irrelevant to the decoding of the current picture and any subsequent pictures in the current temporal layer and any lower temporal layer, there is questionable value to having them included in the BDL in many use cases, and therefore it is desirable to avoid burdening the syntax with this extra data that is irrelevant to the decoding process of the layers being decoded. (However, in some uses it seems to be desirable to include these extra pictures in the BDLs, in order to be able to indicate that some such pictures are to be marked as “unused for reference”.) Even if these pictures are listed, there is no reason to send their actual layer number as proposed in JCTVC-F493; instead, a simple higher_layer_picture_flag can be sent for each picture in the BDL to indicate whether or not the picture is in a higher temporal layer. When higher_layers_not_listed_flag is equal to 1, any pictures in higher temporal layers are proposed to remain unaffected by the decoding process of the current picture – simply retaining their existing marking status.

- The BDL should be in the APS, not the PPS, and there should be only one of them there. The idea of having multiple buffer lists in the parameter set is not elegant, and not necessary. The multiple list concept with an index to identify the selected BDL is something of a new hybrid level of syntax that seems unnecessarily complicated and anticipates an encoder design that pre-plans its BDL structures in advance in a way that seems unnecessary.

- With the BDL residing in the APS, there may be no need for the ability to override the APS-level BDL in the slice header level. Absent appropriate justification, we propose to support sending the BDL only at the APS level or higher.

- We suggest that long-term reference pictures (LTRPs) are a useful design element (that is actually used in some products – especially for robust real-time communication). We note that the concept of long-term reference pictures as found in AVC includes not only the ability to keep a picture in the DPB for a long time, but also the use of a different treatment for motion vector scaling and weighted prediction computations. We propose to retain this functionality, and we suggest that this is the more conservative approach, since LTRPs are supported in AVC and in the current HEVC draft design. With respect to the design for support of LTRPs as proposed in ftp://ftp.hhi.de/ahg21/JCTVC-F803_d1_Buffer_Descriptions_display_process_suggestion_r1.doc, we suggest that its syntax which uses entirely separate listing of LTRPs may be overly elaborate and that its signalling of total POC differences is clearly not appropriate. In that scheme, an LTRP is identified by its total POC difference. For an LTRP that has remained in the DPB for a very long time, the length of the codeword needed for sending its POC difference could become extremely large – wasting bits and causing decoder parsing problems and ultimately limiting the duration for which an LTRP can be retained when using the syntax. Instead we propose a simplified syntax, wherein only the difference between the LSBs of the POCs, modulo the MSB wrapping point, needs to be sent to identify the LTRP, rather than the complete POC difference. There is also no real need to track the MSB values of POCs – only the relative order is really necessary to be established, and avoiding the specification of MSB tracking may ease interpretation of such intended functionality as random-access decoding. By avoiding the need to represent the MSBs of the difference, the POC difference that needs to be sent to identify an LTRP would not need to grow larger and larger over time. With an appropriate design, there is also no real need to avoid sending a new picture that reuses the same POC LSBs that are used by an existing LTRP (unlike what is advocated in JCTVC-F493). Below are two possible approaches within this spirit. Both of them involve sending, for each picture in the BDL, a long_term_reference_flag syntax element. We have a slight preference for Scheme A (since it is more flexible, preserves the full functionality of AVC, and doesn’t seem to add any real complexity); however, Scheme B is also appealing (since it is conceptually simpler and possibly more robust).

  - Scheme A: When the value of this syntax element changes from 0 to 1 for a given value of POC LSBs, this marks the picture as “used for long-term reference” as in AVC. The same POC LSB value can again be used for a short-term reference picture later with no confusion, because the picture with the long_term_reference_flag equal to 1 remains distinctly identified from the picture with the long_term_reference_flag equal to 0 for the same POC LSBs. Only after marking the LTRP as “unused for reference” would the encoder be able to replace it with another LTRP with the same POC LSB value. Some further detail about this approach is provided below in Section 2.

  - Scheme B: Alternatively, a picture could be marked as “used for long-term reference” only by setting a flag that is sent when it is decoded (e.g., a flag called used_for_long_term_flag). All subsequent inclusions of that picture in a BDL would be required to have long_term_reference_flag equal to 1, and the value of long_term_reference_flag would be used to distinguish between the LTRP and any hypothetical short-term reference picture that has the same POC LSB value.

- Since some encoders may not wish to make use of LTRPs, a flag should be sent at the SPS level called the no_long_term_pictures_flag. When that flag is zero, the long_term_reference_flag values (and, in the case of Scheme B above, the used_for_long_term_flag) would not be present in the syntax.

- Note that in either Scheme A or B above, the syntax becomes even more efficient in cases where the number of POC LSBs that is sent is small. We suggest setting the minimum number of POC LSBs to be sent to be 4 (the same as with the use of log2_max_pic_order_cnt_lsb_minus4 in AVC POC type 0) rather than some larger number such as 8. We believe that using a larger number of bits than this would be an unnecessary burden in some applications. In scenarios where a larger number of bits is appropriate, we would expect encoders to simply use a larger value of log2_max_pic_order_cnt_lsb_minus4 (or its equivalent).

- Since the BDL itself is a list of pictures, it can potentially be sent in the order needed for direct construction of list 0 from the first listed pictures of the transmitted list. If the pictures are listed in the list 0 order, there is no real need for reference picture list modification syntax for list 0.

- A flag called poc_gaps_allowed_flag should be sent at the SPS level. When this flag is equal to 0, all consecutive pictures in output order will have consecutive POC LSB values, with no gaps. This would enable decoders to recognize picture losses by recognizing missing POC values. Its use by an encoder would help decoders to recognize strings of pictures that are ready for output from an output-order conforming decoder, because if a picture with some POC value K has already been output, and a picture with POC value K + 1 is present in the DPB, the decoder will be able to detect that this picture with POC LSB value K + 1 will be the next picture to be output (and if no such picture with POC LSB value K + 1 is present in the DPB, the decoder will know that some later picture will arrive with that POC LSB value unless something has gone wrong).

- SPS-level properties like max_reorder_frames, max_num_ref_frames (and no_final_referencing_flag, higher_layers_not_listed_flag, poc_gaps_allowed_flag above, and max_latency_frames of JCTVC-F541), etc. should be allowed to be different for different temporal layers.

- In order to minimize the need for an encoder to set higher_layers_not_listed_flag to 0, it may be desirable to specify automatic picture marking by the decoder for the case where the storage of a new reference picture at one temporal layer could cause the buffering capacity (i.e., max_num_ref_frames) to be exceeded for some higher temporal layer. We propose to specify that when higher_layers_not_listed_flag is equal to 1 and the storage of a current reference picture would cause a violation of the buffer capacity of some higher temporal layer, the decoder shall set the non-LTRP picture with the earliest POC position in a higher temporal layer to “unused for reference” (i.e., the encoder is not allowed to include that picture in any subsequent BDLs).

- Finally, we note that various syntax elements for BDLs have been proposed to be sent as ue(v) encoded values. Since the number of choices available for these syntax elements may often be small, we suggest to consider specifying such syntax elements as te(v) encodings instead. For cases with exactly three possibilities, we suggest for te(v) to be defined such that '1', '01' and '00' indicate the values 0, 1, and 2 respectively (at least in header-level syntax).

- As a further comment relating to item #7 above, we note that the AVC design for POC type 0 has a robustness problem in the way that it tracks the MSBs of POC values. We solve this by introducing a new constraint that all pictures that are used as references or are waiting for output must have relative POC positions that can be computed unambiguously relative to the POC of the current picture. For example, if the POC wrap value is 16 (and the DPB size is large enough for picture output to be delayed and all pictures are reference pictures) and the POC LSB values for consecutive pictures in decoding order are 5, 11, 2, then when the picture with POC LSB value 2 is decoded, the position of the picture with POC LSB value 5 is ambiguous relative to the current picture with POC LSB value 2. If the picture with POC LSB value 11 is lost, the other two pictures will be output in the wrong relative order, the temporal scaling of motion vectors will be incorrect, the reference picture list construction will be wrong, and temporal weighted prediction will become incorrect. By imposing the new constraint and referencing all relative POC values to the POC value for the current picture, we eliminate the need to track actual POC MSBs for the pictures in the buffer. All relative POC positions must remain correct for as long as the pictures stay in the buffer. This should alleviate some concerns that have been expressed in reflector discussions about problems that can occur when using a small number of LSBs for POC, but it solves the problem in a more complete manner than by mandating a larger minimum number of POC LSBs.

# Further details for “Scheme A” Above:

Pathological and transition cases involving long-term reference pictures (LTRPs) and short-term reference pictures (STRPs) for Scheme A (topic 7) are handled as follows:

- If, prior to decoding a new picture, the buffer contains only an STRP for some value of POC LSBs and then a BDL is received that contains only an LTRP for that value, the STRP is marked as "used for long-term reference" prior to decoding the current picture.

- If, prior to decoding a new picture, the buffer contains only an LTRP for some value of POC LSBs and then a BDL is received that contains only an STRP for that value, the bitstream is illegal.

- If, prior to decoding a new picture, the buffer contains both an LTRP and STRP for the same POC LSB values and then a BDL is received that contains only an STRP for that value, the LTRP is marked as "unused for reference".

- If, prior to decoding a new picture, the buffer contains both an LTRP and STRP for the same POC LSB values and then a BDL is received that contains only an LTRP for that value, the STRP is marked as "unused for reference".

# Text modifications

The text modifications are based on ftp://ftp.hhi.de/ahg21/JCTVC-F803_d5_RPS_r1.doc.

buffer description list (BDL): A list of reference pictures associated with the current picture, consisting of reference pictures that precede the current picture in decoding order that may be used for inter prediction of the current picture or subsequent pictures in decoding order.

7.3.2.1	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| max_num_ref_frames | ue(v) |
| no_final_referencing_flag | u(1) |
| higher_layers_not_listed_flag | u(1) |
| no_long_term_pictures_flag | u(1) |
| poc_gaps_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| … |  |
| } |  |

Associated semantics

no_final_referencing_flag specifies whether mark_unused_after_decoding_flag is sent for each picture in the BDL. When no_final_referencing_flag is equal to 1, the mark_unused_after_decoding_flag syntax element is not sent for each picture in BDLs.

higher_layers_not_listed_flag specifies whether pictures of higher temporal layers are included in the BDLs or not. When higher_layers_not_listed_flag is equal to 1, pictures of higher temporal layers are not included in the BDLs.

no_long_term_pictures_flag specifies whether or not long-term reference pictures may be used in the coded video sequence. When no_long_term_pictures_flag is equal to 1, long-term reference pictures are not used in the coded video sequence.

poc_gaps_allowed_flag specifies whether all POC values in the coded video sequence must be strictly consecutive in value. When poc_gaps_allowed_flag is equal to 0, it is a requirement of bitstream conformance that the POC values for the pictures in the coded video sequence shall have consecutive values.

7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| … |  |
| } |  |

7.3.3	Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| … |  |
| } |  |

7.3.3.1 Buffer description list syntax

| buffer_description_list( ) { | Descriptor |
| --- | --- |
| PocWrapVal = (1 << (log2_max_pic_order_cnt_minus4 + 4)) |  |
| iLastEntryFlag = 0 |  |
| iPoc = curr_poc |  |
| lengthL0 = 0 |  |
| for(i = 0; iLastEntryFlag = = 0; i++) { |  |
| poc_diff_sign_flag | u(1) |
| poc_diff_magnitude_minus1 | ue(v) |
| preModuloPoc = iPoc + (1 − 2 * poc_diff_sign_flag) * (poc_diff_magnitude_minus1 + 1) + PocWrapVal |  |
| iPoc = preModuloPoc % PocWrapVal |  |
| PocList[i] = iPoc |  |
| if ( !no_long_term_pictures_flag ) |  |
| long_term_reference_flag | u(1) |
| IsLTRList[i] = no_long_term_pictures_flag ? FALSE : long_term_reference_flag |  |
| if ( !no_final_referencing_flag ) |  |
| mark_unused_after_decoding_flag | u(1) |
| MarkUnusedList[i] = no_final_referencing_flag ? FALSE : mark_unused_after_decoding_flag |  |
| if(List0EndedFlag && !higher_layers_not_listed_flag) { |  |
| higher_layer_pic_flag | u(1) |
| IsHigherTidList[i] = higher_layer_pic_flag |  |
| } else |  |
| IsHigherTidList[i] = FALSE |  |
| last_entry_flag | u(1) |
| iLastEntryFlag = last_entry_flag |  |
| if(!(iLastEntryFlag ¦ ¦ List0EndedFlag)) { |  |
| lengthL0++ |  |
| list0_ended_flag | u(1) |
| List0EndedFlag = list0_ended_flag |  |
| } |  |
| } |  |
| LengthOfBdl = i |  |
| NumPicsList0 = lengthL0 |  |
| } |  |

Associated semantics

poc_diff_sign_flag indicates the sign of the difference used in calculating the POC value for the reference picture in the DPB that corresponds to an entry in the BDL.

poc_diff_magnitude_minus1 + 1 is the magnitude of the difference used in calculating the POC value for the reference picture in the DPB that corresponds to an entry in the BDL.

The values of poc_diff_sign_flag and poc_diff_magnitude_minus1 are used to compute the value of preModuloPoc. It is a requirement of bitstream conformance that the value of preModuloPoc shall be in the range of 0 to 3 * PocWrapVal − 1.

long_term_reference_flag, when present, is used to specify which reference picture in the DPB corresponds to an entry in the BDL and whether or not it is a long-term reference picture by determining the value of IsLTRList[i].

mark_unused_after_decoding_flag, when present, is used to specify whether the reference picture in the DPB that corresponds to the entry in the BDL will be marked as “unused for reference” after decoding the current picture. The marking of pictures with mark_unused_after_decoding_flag equal to 1 as “unused for reference” is performed after the decoding process of the current picture and before the storage of the current picture in the DPB (when applicable).

higher_layer_pic_flag, when present, is used to indicate whether the reference picture in the DPB that corresponds to the entry in the BDL has a higher value of temporal_id than that of the current picture. If higher_layer_pic_flag is equal to 1, the entry in the BDL has a higher value of temporal id than that of the current picture.

last_entry_flag is used to specify whether more entries are present in the BDL.

list0_ended_flag, when present, is used to specify whether more entries are present in the BDL that are included in list 0.

7.3.3.2	Reference picture list modification syntax

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( slice_type != 2 ) { |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |
| do { |  |
| list_modification_idc | ue(v) |
| if( list_modification_idc != 2 ) |  |
| ref_pic_set_idx | ue(v) |
| } while( list_modification_idc != 2 ) |  |
| } |  |
| if( slice_type = = 1 ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |
| do { |  |
| list_modification_idc | ue(v)te(v) |
| if( list_modification_idc != 2 ) |  |
| ref_pic_set_idx | ue(v)te(v) |
| } while( list_modification_idc != 2 ) |  |
| } |  |
| } |  |

The first entries in the BDL correspond directly to the list of pictures in list 0, so reference picture list modification is no longer necessary for list 0.

For cases with exactly three possibilities, we suggest for te(v) to be defined such that '1', '01' and '00' indicate the values 0, 1, and 2 respectively.

8.2.2	Decoding process for buffer description list

This process is invoked when a slice header of the current picture with slice_type equal to P or B is decoded, prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction for the slice as specified in subclause 8.2.3. This process may result in marking one or more reference pictures as “unused for reference” or as “used for long-term reference”. Different slices of the same picture may have different buffer description lists. However, when there are multiple P or B slices for the current picture, for each value of i in the range 0 to LengthOfBdl − 1 that is included in the buffer description list for any slice of the picture, the buffer description list for all other P or B slices of the same picture shall also contain at least one entry with the same value of PocList[i], IsLTRList[i], MarkUnusedList[i], and IsHigherTidList[i].

The input of this process is the results of subclause 7.3.3.1.

When the DPB contains a picture with some POC value iPoc that is marked as “used for short-term reference” and does not also contain a picture with the same POC value iPoc that is marked as “used for long-term reference”, the BDL shall not contain both an entry for some value of i in the range of 0 to LengthOfBdl − 1 and an entry for some value of j in the range of 0 to LengthOfBdl − 1 for which all of the following conditions apply:

- i is not equal to j

- PocList[i] and PocList[j] are both equal to iPoc

- IsLTRList[i] is equal to TRUE and IsLTRList[j] equal to FALSE.

The following is applied for i ranging from 0 to LengthOfBdl − 1:

- If IsLTRList[i] is equal to FALSE, it is a requirement of bitstream conformance that there shall be a reference picture in the DPB with POC equal to PocList[i] that is marked as “used for short-term reference”. This reference picture corresponds to the current entry in the BDL.

- Otherwise (IsLTRList[i] is equal to TRUE), the following applies:

  - If there is a reference picture in the DPB with POC equal to PocList[i] that is marked as “used for long-term reference”, this reference picture corresponds to the entry in the BDL.

  - Otherwise, it is a requirement of bitstream conformance that there shall be a reference picture in the DPB with POC equal to PocList[i] that is marked as “used for short-term reference”. In this case, the marking of this picture is changed from “used for short-term reference” to “used for long-term reference”, and this reference picture corresponds to the entry in the BDL.

The BDL shall not contain an entry for any value of i that has IsLTRList[j] equal to FALSE and PocList[i] equal to curr_poc

The function AdjPicOrderCnt( iPoc, cPoc ) is defined as the value of iResult that is obtained by the following:

if( iPoc = = cPoc )
	iResult = iPoc – PocWrapVal;
else if ( iPoc > cPoc ){
	dN = iPoc – cPoc
	dW = cPoc – (iPoc – PocWrapVal)
	iResult = (dN < dW) ? iPoc : (iPoc – PocWrapVal)
} else {
	dN = cPoc – iPoc
	dW = (iPoc + PocWrapVal) – cPoc
	iResult = (dN < dW) ? iPoc : (iPoc + PocWrapVal)
}

It is a requirement of bitstream conformance that when any two pictures are present in the DPB that are marked as “used for short-term reference” or “needed for output”, the difference between their associated values of AdjPicOrderCnt( PicOrderCnt, curr_poc ) using the value of curr_poc for the current picture shall not be different from the difference between the values of AdjPicOrderCnt( PicOrderCnt, curr_poc ) using the value of curr_poc for the preceding picture in decoding order.

The value of PicOrderCnt for the current picture is set to curr_poc.

[Ed. Note: The HRD bumping process will output pictures in increasing order of AdjPicOrderCnt.]

8.2.3	Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture list RefPicList0, and for B slices RefPicList1, are derived as specified.

RefPicList0 consists of the first NumPicsList0 entries in the BDL.

For B slices an initial reference picture list RefPicList1 is derived as specified in subclause 8.2.3.1.

When decoding a B slice with ref_pic_list_modification_flag_l1 equal to 1, the initial reference picture list RefPicList1 is modified as specified in subclause 8.2.3.2.

The number of entries in the RefPicList0 is NumPicsList0, and for B slices the number of entries in the modified reference picture list RefPicList1 is num_ref_idx_l1_active_minus1 + 1. A reference picture may appear at more than one index in RefPicList0 or RefPicList1.

8.2.3.1 Initialization process for reference picture list 1

This initialization process is invoked when decoding a B slice header.

The list ShortTermBeforeList is constructed as the list of entries in the BDL that have IsHigherTidList[i] equal to FALSE and IsLTRList[i] equal to FALSE and AdjPicOrderCnt( PocList[i], curr_poc ) less than cur_poc. Duplicate entries are not included in the construction of the list ShortTermBeforeList. NumPicsBefore is set equal to the number of pictures in the list ShortTermBeforeList. The list ShortTermBeforeList is ordered in decreasing order of AdjPicOrderCnt( PocList[i], curr_poc ). The list STBAdjPoc is constructed of the associated values of AdjPicOrderCnt( PocList[i], curr_poc ).

The list ShortTermAfterList is constructed as the list of entries in the BDL that have IsHigherTidList[i] equal to FALSE and IsLTRList[i] equal to FALSE and AdjPicOrderCnt( PocList[i], curr_poc ) greater than cur_poc. Duplicate entries are not included in the construction of the list ShortTermAfterList. NumPicsAfter is set equal to the number of pictures in the list ShortTermAfterList. The list ShortTermAfterList is ordered in increasing order of AdjPicOrderCnt( PocList[i], curr_poc ). The list STAAdjPoc is constructed of the associated values of AdjPicOrderCnt( PocList[i], curr_poc ).

The list LongTermList is constructed as the list of entries in the BDL that have IsHigherTidList[i] equal to FALSE and IsLTRList[i] equal to TRUE. Duplicate entries are not included in the construction of the list LongTermList. NumPicsLT is set equal to the number of pictures in the list LongTermList. The list LongTermList is ordered in increasing order of AdjPicOrderCnt( PocList[i], curr_poc ). The list LTAdjPoc is constructed of the associated values of AdjPicOrderCnt( PocList[i], curr_poc ).

The initial reference picture list RefPicList1 is constructed by concatenating the entries of ShortTermAfterList, then the entries of ShortTermBeforeList, and then the entries of LongTermList. The initial list List1AdjPocs is constructed by concatenating the entries of STBAdjPoc, then the entries of STAAdjPoc, and then the Entries of LTAdjPoc. The initial list List1IsLTR is constructed by concatenating NumPicsBefore + NumPicsAfter values equal to 0, followed by NumPicsLT values equal to 1.

8.2.3.2	Modification process for reference picture lists

When ref_pic_list_modification_flag_l0 is equal to 1, the following applies:

Let refIdxL0 be an index into the reference picture list RefPicList0. It is initially set equal to 0.

The corresponding syntax elements list_modification_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies.

–	If list_modification_idc is equal to 0 or equal to 1, the process specified in subclause Error: Reference source not found is invoked with refIdxL0 as input, and the output is assigned to refIdxL0.

–	Otherwise (list_modification_idc is equal to 2), the modification process for reference picture list RefPicList0 is finished.

When the current slice is a B slice and ref_pic_list_modification_flag_l1 is equal to 1, the following applies:

Let refIdxL1 be an index into the reference picture list RefPicList1. It is initially set equal to 0.

The corresponding syntax elements list_modification_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies.

- If list_modification_idc is equal to 0 or equal to 1, the process specified in subclause 8.2.2.3.1 is invoked with refIdxL1 as input, and the output is assigned to refIdxL1.

- Otherwise (list_modification_idc is equal to 2), the modification process for reference picture list RefPicList1 is finished.

8.2.4	Decoded reference picture marking process.

This process is invoked after decoding a picture.

When no_final_referencing_flag is equal to 0, any picture with mark_unused_after_decoding_flag equal to 1 is marked as “unused for reference”.

After operation of the bumping process for output of the pictures in the DPB that are marked as “needed for output” and the removal of any such pictures from the DPB that become marked as “not needed for output” and “not used for reference”, when higher_layers_not_listed_flag is equal to 1 and the storage of the current picture (nal_ref_idc is not equal to 0) would cause a violation of the maximum reference picture buffering capacity in any temporal layer, the decoder shall set the non-LTRP picture with the smallest value of AdjPicOrderCnt( PicOrderCnt, curr_pic ) in a higher temporal layer to “unused for reference”.

# Patent rights declaration(s)

Microsoft Corporation and the University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).