| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G791 |
| --- | --- |

| Title: | Non-CE6: Simplified reference samples padding for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution removes two sides checking of unavailable pixels range in reference samples padding in HM4.0 to reduce the complexity. The unavailable pixels range is padded by the nearest available pixel in one fixed direction instead of adapting direction or averaging by the nearest available pixels. It is reported that no BD-rate loss is observed for all configurations (AI, RA, LB in HE, LC) with 1500-byte slice mode setting or/and constrained intra prediction setting.

# Introduction

In the 5th Geneva meeting, padding of unavailable reference samples is adopted for intra prediction [1]. It is beneficial to use partially available reference samples in intra prediction, which occurs when constrained intra prediction is enabled or when slice coding is used in HM4.0.

In the current HM design, when a part of reference samples needs to be padded, the nearest available two pixels of the part are searched and then the following steps are applied:

If either one end of the part is available, pad the part by the available one.
Else if both ends of the part are available, pad the part by the averaged value of those.
Otherwise, pad the part by the value ( 1 << ( BitDepthY - 1 ) )

But to keep availability checking and positions of two ends of unavailable range requires additional condition checking and additional loop in implementation. In this proposal, this procedure is simplified and the detailed description is as follows in the next section.

# Description

In this proposal, unavailable reference pixels are padded by one-directional copying. For the above boundary pixels, it is defined to copy from the left to the right; and for the left boundary pixels, it is defined to copy from the top to the bottom for padding.

If the left-top pixel is unavailable, the following two methods were tested;

Method1: pad the left-top by the current HM 4.0 scheme (as described in Introduction section)
Method2: pad the first available pixel close to the left-top

In details of Method2, the left-top is filled as follows if it is not available;

If the nearest above pixel is available, then choose the value to fill the left-top
else if the nearest left pixel is available, then choose the value to fill the left-top
else choose the value ( 1 << ( BitDepthY - 1 ) ) to fill the left-top

For example of Figure 1,

= available pixels

= unavailable pixels

= one-directional copy

PLT

PT

PL

Figure 1. Example of padding by one-directional copying

PLT is filled by PT in Method2 while it is filled by ((PT + PL + 1) >> 1) in Method1.

# Experimental result

Since there is no difference in the default configuration and the case of partially unavailable pixels in reference samples for intra predictions is occurred if multiple slices coding in a picture or constrained intra prediction is enabled, the following simulations were conducted.

With constrained intra prediction,                 RA-HE, RA-LC, LB-HE, LB-LC
With 1500-byte slices,             AI-HE, AI-LC, RA-HE, RA-LC, LB-HE, LB-LC
With constrained intra prediction and 1500-byte slices, RA-HE, RA-LC, LB-HE, LB-LC

AI tests were conducted only when 1500-byte slices enabled because there is no impact with constrained intra prediction in AI tests.

Table 1, Table 2 and Table 3 show its RD performance and execution time of Method1 respectively with CIP, 1500-byte slices and CIP+1500-byte slices.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | 0.1% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

Table 1. Method 1 with CIP

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |

Table 2. Method 1 with 1500-bytes slices

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class E | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 3. Method 1 with CIP + 1500-bytes slices

Table 4, Table 5 and Table 6 show its RD performance and execution time of Method2 respectively with CIP, SliceBytes and CIP+SliceBytes.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.3% | 0.0% | 0.0% | -0.2% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.1% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

Table 4. Method 2 with CIP

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.4% |
| Class C | 0.0% | -0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.2% | -0.3% | 0.0% | 0.1% | 0.0% |
| Class E | 0.1% | 0.2% | 0.2% | -0.1% | -0.6% | -0.5% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |

Table 5. Method 2 with 1500-bytes slices

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.3% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.4% | 0.0% | 0.1% | 0.0% |
| Class E | 0.1% | 0.0% | 0.5% | 0.0% | 0.5% | 0.2% |
| Overall | 0.0% | 0.0% | 0.3% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.2% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |

Table 6. Method 2 with CIP + 1500-bytes slices

In all the above tests, performance is almost no loss and slightly reduced execution time in average is observed.

# Conclusion

By only one-directional copying to pad unavailable pixels on the boundaries, performance has no loss and complexity to check bi-directional possibility and average pixels is reduced.

For the left-top pixel, either averaging or taking one value has no loss. So it is proposed to take one value for left-top pixel padding since the latter one requires the smaller complexity.

# References

- R. Sjöberg, C. Lai, K. Chono, V. Wahadaniah, “BoG report on padding of unavailable reference samples for intra prediction”, JCTVC-E488, 5th Meeting: Geneva, CH, 16-23 March, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

Proposed working draft text is included below using the following text conventions:

- Removed text from JCTVC-F803_d5 is shown with strikethrough.

- Newly added text is shown in red.

- 8.3.3.1.1		Reference sample substitution process for intra sample prediction

Inputs to this process are the reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

Outputs of this process are the modified reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

The values of the samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are modified as follows:

- If all samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “not available for intra prediction,” the value ( 1 << ( BitDepthY - 1 ) ) is substituted for the values of all samples p[ x, y ].

- Otherwise (at least one but not all samples p[ x, y ] are marked as “not available for intra prediction”), the following steps are performed for each sample p[ x0, y0 ] with x0 = -1, y0 = -1..nS*2-1 and x0 = 0..nS*2-1, y0 = -1 marked as "not available for intra prediction":

- If p[ -1, -1 ] is marked as “not available for intra prediction”,

- If p[x0, -1] exists such that x0 is the smallest value where x0 = 0..nS*2-1 and p[x0, -1] is available for intra prediction, the value of p[x0, -1] is substituted for the value of p[ -1, -1 ]

- Else if p[-1, y0] exists such that y0 is the smallest value where y0 = 0..nS*2-1 and p[-1, y0] is available for intra prediction, the value of p[-1, y0] is substituted for the value of p[ -1, -1 ]

- Otherwise, the value ( 1 << ( BitDepthY - 1 ) ) is substituted for the values of p[ -1, -1 ]

- If p[x0, -1] is marked as “not available for intra prediction” where x0 = 0..nS*2-1, the value of p[x0-1, -1] is substituted for the value of p[ x0, -1 ]

- If p[-1, y0] is marked as “not available for intra prediction” where y0 = 0..nS*2-1, the value of p[-1, y0-1] is substituted for the value of p[ -1, y0 ]

- Let the variables q and r be initially set to -1.

- The variable q is modified as follows:

  - If x0 is equal to -1, a reference sample p[ x, y ] marked as “available for intra prediction” is identified by searching sequentially among p[ x, y ] starting from x = -1, y = y0+1 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q. [Ed.: (WJ) is there any elegant way rather than searching?]

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0-1, y = -1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = x0-1..-1, y = -1, the search is continued among p[ x, y ] sequentially starting from x = -1, y = 0 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q. [Ed.: (WJ) is there any elegant way rather than searching?]

- The variable r is modified as follows:

  - If x0 is equal to -1, a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = -1, y = y0-1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = -1, y = y0-1..-1, the search is continued among p[ x, y ] sequentially starting from x = 0, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r. [Ed.: (WJ) is there any elegant way rather than searching?]

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0+1, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r. [Ed.: (WJ) is there any elegant way rather than searching?]

- The value of the sample p[ x0, y0 ] is modified as follows:

  - If q is not equal to -1 and r is not equal to -1, the value ( ( q + r + 1 ) >> 1 ) is substituted for the value of p[ x0, y0 ].

  - Otherwise, if q is not equal to -1, the value of q is substituted for the value of p[ x0, y0 ].

  - Otherwise (q is equal to -1 and r is not equal to -1), the value of r is substituted for the value of p[ x0, y0 ].

All samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “available for intra prediction.”