| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G792 |
| --- | --- |

| Title: | Cross-check for Huawei’s proposal on PU dependent coefficient scan for inter block (JCTVC-G269) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yinji Piao Jianle Chen Junghye Min JeongHoon Park | Email: | yj1003.piao@samsung.com jianle.chen@samsung.com jh643.min@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification result of PU dependent coefficient scan for inter blocks as described in JCTVC-G269 (F501). The scan type is determined by the partition size and TU shape. The software implementation is consistent with the algorithm description, and crosscheck results matches the results from proponent.

# Code-level check

The proposal was implemented on top of HM4.0 in low complexity configurations. HW_ACS macro was used for the proposed modification on coefficient scan. Vertical scan for 2Nx1/2N transform unit and horizontal scan for 1/2Nx2N TU was implemented not directly inside the coefficient coding but prior to coefficients coding by mapping vertical scan of 2Nx1/2N into raster scan order.

# Test Results

The tests were carried out under the test conditions specified in the JCTVC-F900. S/W provided by Huawei was compiled on 64-bit Linux and executed on Linux cluster. Test is conducted only for low complexity RA and LD configurations. The summary result is represented in following table, and more details can be found in accompanying excel sheet.

|  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% |  |  |  |
| Class B | -0.1% | -0.5% | -0.3% | -0.1% | -0.7% | -0.5% |
| Class C | 0.0% | -0.2% | -0.1% | 0.0% | -0.3% | -0.5% |
| Class D | 0.0% | -0.2% | -0.1% | -0.1% | -0.2% | -0.3% |
| Class E |  |  |  | -0.3% | -0.1% | -0.2% |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | -0.3% | -0.1% |
| Overall | 0.0% | -0.2% | -0.1% | -0.1% | -0.3% | -0.3% |
|  | 0.0% | -0.2% | -0.1% | -0.1% | -0.4% | -0.3% |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# Conclusions

Test results from JCTVC-G269 (F501) are verified in this report. The BD rates matched the data provided by the proponents, but the encoding time was slightly higher in our test. This may be caused by restoring coefficients in vertical scan into raster scan prior to coefficients coding. In hardware implementation this operation may not be needed.