| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G795 |
| --- | --- |

| Title: | Crosscheck of TI’s JCTVC- G669 on delay dependent intra frame for video conferencing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Junghye Min Kwang Pyo Choi Youngo Park Yinji Piao JeongHoon Park | Email: | jh643.min@samsung.com kp5.choi@samsung.com youngo.park@samsung.com yj1003.piao@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of Delay Dependent Intra frame in JCTVC-G669. The cross-verifications were completed and the results confirm the test results from the proponents. The proponents’ code has also been analyzed and found to match to the context in the proposal.

# Introduction

In JCTVC-G669 [1], the TI has proposed “Delay Dependent Intra frame (DDI)” which was designed to smooth out bit-rate fluctuation from I frame encoding in low delay scenarios. In their proposed method, instead of an I frame, two frames, a P frame and an I frame (DDI frame), are encoded independently and transmitted. The I frame (DDI) is transmitted in pieces with later P frames with relaxed real time constraint. Picture reference structure is also changed where DDI frames are referenced after the pre-defined delay.

# Simulation Results

The tests were carried out under the test conditions specified in the JCTVC-F900 except several configuration changes. Base configuration changes include,

- RateGOPSize = 1 (default: 4)

- NumOfReference = 1 (default: 4)

- UsingNewRefSetting = 0 (default :1)

- IntraPeriod = 100 (default : -1)

Additional parameters introduced for Delay Dependent Intra frame (DDI) are set as follows.

- DDI_PIC_Enabled  = 1

- DDI_PIC_Delay  = 10

The S/W provided by TI was compiled on 64-bit Linux and executed on Linux cluster. Tests were conducted only for LD and LDP configurations. The summary result is presented in the following table, and more details can be found in accompanying excel sheet.

|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | Y | U | Y | U | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 3.0 | 3.7 | 3.4 | 3.0 | 3.4 | 3.3 | 3.0 | 3.5 | 3.4 | 2.9 | 3.3 | 3.2 |
| Class C | 2.6 | 2.8 | 2.9 | 2.5 | 2.5 | 2.6 | 2.5 | 2.6 | 2.7 | 2.5 | 2.3 | 2.4 |
| Class D | 2.6 | 2.7 | 3.2 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.5 | 2.4 | 2.4 | 2.3 |
| Class E | 3.5 | 3.0 | 3.5 | 3.4 | 3.0 | 3.1 | 3.4 | 3.1 | 3.5 | 3.4 | 2.4 | 2.9 |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 2.9 | 3.1 | 3.3 | 2.8 | 2.9 | 2.9 | 2.8 | 3.0 | 3.0 | 2.8 | 2.7 | 2.7 |
| Enc Time[%] | 102% |  |  | 102% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 91% |  |  | 94% |  |  | 90% |  |  | 93% |  |  |

2-1 Test results of Delay Dependent Intra frame  ( DDI_PIC_Delay = 10)

# Conclusions

The results (BD-rates) provided by TI were confirmed for the case of DDI_PIC_Delay  = 10. Comparison with other schemes such as “Cyclic Intra Refresh” and “Adaptive Intra Refresh”  from conventional codec should be discussed for this proposal.

# References

[1] Ranga Ramanujam Srinivasa, Chaitanya Ghone, Mihir Mody and Minhua Zhou, “Delayed Duplicate I-Frame for Video Conferencing”, JCT-VC document JCTVC-G699, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.