| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G796 |
| --- | --- |

| Title: | Cross-check for RIM’s proposal on small TU significance map coding (JCTVC-G657) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yinji Piao Elena Alshina Junghye Min JeongHoon Park | Email: | yj1003.piao@samsung.com elena_a.alshina@samsung.com jh643.min@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of context number reduction for significance map coding of TU size of 4x4 and 8x8 as described in JCTVC-G657. The number of context is reduced by half with four partition sets to map position to corresponding context index. The software implementation is consistent with the algorithm description, and crosscheck results matches the results from proponent.

# Code-level check

The proposal was implemented on top of HM4.0 in high efficiency configurations.

SIG_CTX_PARTITION_4x4 and SIG_CTX_PARTITION_8x8 macros are used for the proposed modification and are enabled to turn on and off independently to test separate results. Look-up tables for four partition sets are added to derive context index of a certain position. Furthermore the contexts for luma and chroma are designed in a different fashion. So previous 15 contexts for 4x4 TU luma, 15 contexs for 4x4 TU chroma are replaced by 8 contexts for luma and 7 contexts for chroma. Previous 16 contexts for 8x8 TU luma, 16 contexts for 8x8 TU chroma are replaced by 12 contexts for luma and 4 contexts for chroma in RIM’s proposal.

# Test Results

The tests were carried out under the test conditions specified in the JCTVC-F900. S/w provide by RIM was compiled on 64-bit Linux and executed on Linux cluster. Since there are two macros and work independently, tests under three conditions were all carried out in HE configurations,

- SIG_CTX_PARTITION_4x4  only

- SIG_CTX_PARTITION_8x8  only

- SIG_CTX_PARTITION_4x4 & SIG_CTX_PARTITION_8x8

The summary results are represented in following tables, and more details can be found in accompanying excel sheet.

Table I. SIG_CTX_PARTITION_4x4_on , SIG_CTX_PARTITION_8x8_off

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.1% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.4% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% | -0.1% | 0.1% | -0.1% | -0.3% | 0.1% | -0.6% | -0.4% |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  | 0.2% | 0.1% | -0.5% | 0.1% | 0.1% | -0.8% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.1% | 0.1% | 0.0% | -0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% | 0.1% | -0.1% | -0.2% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% | 0.1% | 0.0% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  | 101% |  |  |

Table II. SIG_CTX_PARTITION_4x4_off , SIG_CTX_PARTITION_8x8_on

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.3% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | -0.3% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.3% | 0.0% | 0.0% | 0.1% | 0.0% | 0.3% | 0.1% |
| Class D | 0.0% | 0.3% | 0.3% | 0.1% | 0.5% | 0.2% | 0.1% | 0.5% | 0.7% | 0.0% | 0.4% | 0.4% |
| Class E | 0.0% | -0.1% | 0.0% |  |  |  | 0.1% | 0.0% | -0.3% | 0.0% | -0.7% | -1.2% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% | -0.2% | -0.3% | 0.1% | 0.0% | 0.2% | 0.2% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  | 101% |  |  |

Table III. SIG_CTX_PARTITION_4x4_on , SIG_CTX_PARTITION_8x8_on

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | 0.0% | -0.5% | -0.5% | 0.0% | -0.4% | -0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.6% | -0.2% | 0.0% | -0.3% | -0.2% |
| Class D | 0.0% | -0.1% | 0.0% | 0.1% | -0.3% | -0.1% | 0.1% | -0.6% | -0.6% | 0.1% | -1.2% | -0.9% |
| Class E | 0.0% | 0.0% | -0.1% |  |  |  | 0.0% | 0.0% | 0.2% | 0.0% | -0.3% | -0.5% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | -0.4% | -0.1% | -0.1% | -0.3% | 0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.4% | -0.3% | 0.0% | -0.5% | -0.3% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.4% | -0.3% | 0.0% | -0.5% | -0.3% |
| Enc Time[%] | 100.2% |  |  | 100.2% |  |  | 100.1% |  |  | 100.3% |  |  |
| Dec Time[%] | 100.1% |  |  | 100.2% |  |  | 100.0% |  |  | 100.6% |  |  |

# Conclusions

Test results from JCTVC-G657 are verified in this report. The simulation results matched the data provided by the proponents.