| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G798 WG11 Number: m22370 |
| --- | --- |

| Title: | Cross verification of MediaTek’s proposed residual quad-tree depth for chroma in intra coding (JCTVC-G283) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of MediaTek’s proposed residual quad-tree depth for Chroma components. In HM4.0, luma and chroma components share the same RQT depth. The contribution JCTVC-G283 proposes to apply separate RQT depths to luma and chroma for intra mode. A user-defined parameter max_transform_depth_intra_chroma is inserted into SPS syntax table, and coding gains can be achieved by setting the parameter to 0.

The software package obtained from the proponent was successfully compiled and tested under the common test conditions for AI_HE and AI_LC cases. The BD rate results match exactly to the data provided by the proponent, and the differences of enc/dec time are within normal variations. Table 1 shows the performance data of the proposed technique.

Table 1 Crosscheck results of independent RQT depth for Chroma

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -1.5% | -1.4% | -0.1% | -2.0% | -2.3% |
| Class B | -0.1% | -1.8% | -1.8% | -0.3% | -2.4% | -3.0% |
| Class C | 0.0% | -1.3% | -1.5% | -0.2% | -1.5% | -1.9% |
| Class D | 0.0% | -1.5% | -1.7% | -0.1% | -1.1% | -1.2% |
| Class E | -0.1% | -2.3% | -2.2% | -0.2% | -5.9% | -4.7% |
| Class F | 0.0% | -1.1% | -1.4% | -0.1% | -1.0% | -1.0% |
| Overall A-E | -0.04% | -1.67% | -1.70% | -0.20% | -2.43% | -2.53% |
|  | -0.04% | -1.68% | -1.74% | -0.20% | -2.44% | -2.55% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |