| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G806 |
| --- | --- |

| Title: | Non CE3: On the phase offset selection for motion compensation interpolation filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon and Jian Lou 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 kminoo@motorola.com david.baylon@motorola.com lou@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In this contribution 4 sets of filters are introduced and used to conduct motion compensation with quarter-pixel motion resolution (i.e. four level sub-pixel signaling and storing). The choice of filter set is decided based on the sub-pixel information of the motion vector predictor (stored at quarter-pel resolution).  Overall 0.82% gains was observed for Luma. (0.2% gain was also observed for each of the Chroma components)

# Introduction

In JCTVC-F574 [1], Motorola Mobility has proposed a set of sub-pixel interpolation filters which improves the average coding efficiency amongst the 6 relevant anchor conditions. The main design idea for those filters was based on the observation that the distribution of sub-pixel phase offsets, statistically, has higher concentration around the integer-pel position. So in JCTVC-F574 instead of uniformly representing the sub-pixel displacements, the motion compensation used filters with 3/16 and 13/16 phase offsets (compared to 1/4 and 3/4 phase offsets in HM4.0[2]) to better represent the sub-pixel displacement between current PU and the reference PU(s).

In JCTVC-F574 it was argued that one design factor for improving the optimality of interpolation filters (used in motion compensation) should be the phase offset. Furthermore by relating the selection of phase offset, with limited precision, to selection of reconstruction values in quantization of unit interval, we justified the selection of interpolation filters with 3/16 and 13/16 phase offsets.

# Proposal

In this proposal we extend the principle of quantization of unit interval based on the distribution of phase offset values (in that interval), to consider conditional distribution of phase offsets. Looking at statistics collected from anchor sequences coded with HM4.0 [2], it becomes evident that the distribution of motion vector (used for motion compensation) is highly concentrated at around MVP. This observation opens the possibility of designing interpolation filters with phase offsets that match the sub-pixel offset of the MVP. In other words, the selection of phase offset for each sub-pixel position would be based on the conditional distribution of MV for a given MVP.

In this proposal 4 sets of interpolation filters are used to perform motion compensation for a given motion vector. The selection of filter set at both encoder and decoder takes place according to the value of MVP.

# Simulation results

The software used to run the simulations is HM4.0 [2]. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics. The values of encode and decode time will be updated once the reliable readings for the full sequence is done.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | -0.1% | -0.2% | 0.3% | 0.3% |
| Class B | -0.3% | 0.0% | 0.0% | -0.3% | -1.0% | 0.1% |
| Class C | -0.3% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class D | -0.4% | 0.0% | 0.1% | 0.5% | 0.2% | 0.3% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.3% | 0.0% | 0.0% | 0.0% | -0.2% | 0.2% |
|  | -0.3% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 124% |  |  | 117% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.2% | -0.2% | -0.5% | 0.3% | 0.3% |
| Class C | -0.3% | 0.1% | 0.3% | -0.1% | 0.3% | 0.2% |
| Class D | -0.1% | 0.4% | 0.6% | 0.2% | 0.4% | 0.6% |
| Class E | -0.5% | 0.4% | 0.1% | -1.2% | 0.0% | -0.9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.3% | 0.3% | 0.2% | -0.4% | 0.3% | 0.1% |
|  | -0.3% | 0.2% | 0.1% | -0.4% | 0.2% | 0.1% |
| Enc Time[%] | 116% |  |  | 120% |  |  |
| Dec Time[%] | 97% |  |  | 103% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.2% | -0.5% | -0.5% | -1.8% | 0.1% | -0.1% |
| Class C | -1.8% | -0.9% | -1.1% | -1.7% | -0.4% | -0.3% |
| Class D | -2.3% | -1.9% | -1.4% | -2.1% | -1.3% | -0.9% |
| Class E | -2.1% | -0.8% | -0.8% | -3.7% | -1.7% | -1.4% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1.8% | -1.0% | -0.9% | -2.2% | -0.7% | -0.6% |
|  | -1.8% | -1.0% | -1.0% | -2.2% | -0.7% | -0.6% |
| Enc Time[%] | 105% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 103% |  |  |

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] J.Lou, K.Minoo, D.Baylon, K.Panusopone, L.Wang, A.Luthra “CE3: Fixed interpolation filter tests by Motorola Mobility,” JCT-VC document, JCTVC-F574, July 2011.

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900